3GPP TSG-RAN WG3 Meeting #114-e	R3-214860

E-meeting, 1-11 NOV 2021

Title: 	(TP for MDT BLCR for TS 38.413): Discussion on Reply LS on Report Amount for M4, M5, M6, M7 measurements

Source: 	Huawei

Agenda item:	8.3.4

Document Type:	other

1. Introduction

This paper aims to discuss the reply LS on report amount of M4, M5, M6, M7 measurements in [1].

2. Discussion

- In RAN2 #115-e meeting, RAN2 has confirmed that the reportAmount Configuration is necessary to configure the UE with D1 measurement reports from the UE [1]. Besides, if RAN has not received reportAmount as part of M6 configuration from the AMF, the RAN can choose any allowed value for the reportAmount and configure it to the UE, which may cause the following limitations:

- •	RAN might configure the UE with infinity as the reportAmount which could lead to unnecessary usage of the radio resources over Uu interface and also increase the UE battery consumption.

- •	If the RAN node arbitrarily chooses a different value than infinity as the reportAmount, then the OAM might fail to correlate the M6 measurements and the corresponding M1 measurements as M6 measurement would cease to be reported after certain time despite receiving M1 measurement for the same UE.

- In our opinion, apart from M6 measurement, the M4, M5, M7 measurements should also be configured with reportAmount. In this case, we propose the TP in the Annex to make corresponding changes in the related specifications.

RAN3 to agree on introducing the reportAmout for M4, M5, M6 and M7.

3. Conclusion

Based on the discussion in this paper, we propose the following:

- RAN3 to agree on introducing the reportAmout for M4, M5, M6 and M7.

The TPs are provided in Annex, [2] ~ [4].

4. Reference

[1] R2-2108966, Reply LS on Report Amount for M4, M5, M6, M7 measurements.

[2] R3-214861	(TP for MDT BLCR for TS 38.423): Introducing Report Amount for M4, M5, M6, M7 measurements

[3] R3-214862	(TP for MDT BLCR for TS 38.473): Introducing Report Amount for M4, M5, M6, M7 measurements

[4] R3-214863	(TP for MDT BLCR for TS 38.463): Introducing Report Amount for M4, M5, M6, M7 measurements

Annex –TP for MDT BLCR for TS 38.413

9.3.1.172	M4 Configuration

This IE defines the parameters for M4 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M4 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M4 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M4 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity…) | Number of reports. |

9.3.1.173	M5 Configuration

This IE defines the parameters for M5 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M5 Collection Period | M |  | ENUMERATED (ms1024, ms2048, ms5120, ms10240, min1, …) |  |
| M5 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M5 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity…) | Number of reports. |

9.3.1.174	M6 Configuration

This IE defines the parameters for M6 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M6 Report Interval | M |  | ENUMERATED (ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …) |  |
| M6 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity…) | Number of reports. |

9.3.1.175	M7 Configuration

This IE defines the parameters for M7 measurement collection.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| M7 Collection Period | M |  | INTEGER (1..60, …) | Unit: minutes |
| M7 Links to Log | M |  | ENUMERATED (uplink, downlink, both-uplink-and-downlink, …) |  |
| M6 Report Amount | O |  | ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity…) | Number of reports. |