3GPP TSG-RAN WG3 #114-e	R3-214986

1– 11 Nov 2021

Online

Title: 	(TP for SON BL CR for TS 38.423) MRO SN change failure

Source: 	ZTE

Agenda item:	10.2.1.6

Document for:	Approval

1	Introduction and discussion

This contribution provided the TP for based on the following proposals.

Proposal 1: MN should store the Source PSCell CGI and Failed PSCell CGI into its context.

Proposal 2: Introduce an optional Suitable PSCell CGI IE in XnAp PSCell Change Report message.

Proposal 3: Include mobility information, S-NG-RAN node UE X2AP ID, M-NG-RAN node UE XnAP ID in the new message.

Proposal 4: Introduce an Choice UE context identify IE(either X-NG-RAN node UE XnAP ID or Mobility information) in XnAP PSCell Change Report message.

Proposal 5: To introduce a Class 2 message in XnAP for PScell change failure.

Proposal 6: Only support NR scenario for SN change failure in R17.

Proposal 7: There would be no ambiguity issue in SCGF failure cases, which can be avoided by network implementation.

- TP to TS 38.423

- -------------------------------------------------------Start of Change ------------------------------------------------------

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| PSCell Change Report | PSCELL CHANGE REPORT |

    - -------------------------------------------------------Next Change -----------------------------------------------------

    - 8.3.1X	PSCell Change Report

      - 8.3.1X.1	General

The purpose of the PSCell Change Report procedure is to transfer mobility related information regarding PScell change failure, from the M-NG-RAN node to the S-NG-RAN node.

The procedure uses non UE-associated signalling.

      - 8.3.1X.2	Successful Operation

Figure 8.3.1X.2-1: PScell Change Report, successful operation

The M-NG-RAN node initiates the procedure by sending the PSCELL CHANGE REPORT message to the S-NG-RAN, following an RRC SCGFailureInformation reception from a UE at NG-RAN node2, when M-NG-RAN considers that the UE may have previously suffered a mobility failure at a PScell controlled by S-NG-RAN.

If the UE SCGFailureInformation IE is included in the PSCELL CHANGE REPORT message, the S-NG-RAN  shall use it to derive failure case information.

      - 8.3.15.3	Abnormal Conditions

Void.

- -------------------------------------------------------Next Change ------------------------------------------------------------

      - 9.1.3.2X	PSCELL CHANGE REPORT

This message is sent by M-NG-RAN node to S-NG-RAN node to report a critical mobility problem.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Choice UE context identify | M |  |  |  |  |  |
| > UE AP ID |  |  |  |  |  |  |
| >>M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| >>S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| > Mobility information |  |  |  |  |  |  |
| >> SN Mobility Information |  |  | BIT STRING (SIZE (32)) | Information related to the PSCell change. It’s provided by S-NG-RAN node in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| PScell Change Information Container | M |  | OCTET STRING | SCGFailureInformation as defined in TS 38.331 [10] | YES | ignore |
| Source PSCell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of source PSCell for PSCell change procedure | YES | ignore |
| Failed cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN CGI of PSCell where SCG failure occurs for PSCell change procedure. | YES | ignore |
| Suitable PScell CGI | O |  | Global NG-RAN Cell Identity 9.2.2.27 | Suitable CGI of cell based on the measurements reported from the UE after the failure. | YES | ignore |

- -------------------------------------------------------End of Change ---------------------------------------------------------