3GPP TSG-RAN WG3 Meeting #114-e 	R3-215082

# E-Meeting, 1st Nov - 12th Nov, 2021

Agenda Item:	21.2

Source:	Samsung

Title:	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.413) Discussion on

supporting the time synchronization error budget

Document for:	Discussion & Approval

Introduction

In the previous RAN3#113-e meeting, RAN3 discussed the time synchronization error budget and decided to discuss it based on RAN2/SA2 progress:

For the time synchronization error budget, it is FFS for which level (e.g., UE level, PDU session level, or QoS flow level) and for its details (e.g., parameter name, parameter meaning, and value range)

In this document, we discuss the open issues and RAN3 impact to support the time synchronization error budget.

Discussion

In the previous RAN3 meeting, RAN3 has a consensus on delivering the time synchronization error budget over some RAN3 interfaces. But RAN2/SA2 had been still discussing the time synchronization assistance information including the time synchronization error budget, so RAN3 couldn’t make further progress. There are some open issues:

- Whether to deliver the time synchronization error budget over NG, Xn and F1 interfaces?

- Which level is used to deliver the time synchronization error budget, UE level, PDU session level, QoS flow level or others?

- Details about the time synchronization error budget information, e.g. parameter name, parameter meaning, and value range

Table 4.15.9.4-1: Description of 5G access stratum time distribution parameters

| Parameter | Description |
| --- | --- |
| 5G access stratum time distribution (enable, disable) | Indicates that the access stratum time distribution via Uu reference point should be activated or deactivated for the associated UE identities. |
| Time synchronization error budget | Indicates the time synchronization error budget for the time synchronization service (as described in clause 5.27.1.8.1 in TS 23.501 [2]). [optional] |
| Temporal Validity Condition | Indicates start-time and stop-time attributes that describe the time period when the time synchronization service is active. [optional] |

Figure 4.15.9.4: Management of 5G access stratum time information

[…]

10.	The PCF takes a policy decision and then it may initiate an AM Policy Association Modification procedure initiated by the PCF for the UE may be performed as described in clause 4.16.2.2. As part of this, the time distribution indication and the Uu time synchronization error budget are provided to NG-RAN. Based on this, NG-RAN provides the 5GS access stratum time to the UE according to the Uu time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [30].

In SA2#146-e meeting, SA2 discussed the details about control of the 5G reference time information distribution including the time synchronization error budget information and agreed the related CR [1]. The SA2 CR adds the procedures for the management of 5G access stratum time distribution procedure.

Following the SA2 agreement, the time synchronization error budget should be optionally delivered from AMF to NG-RAN. In addition, the time distribution indication should be also optionally delivered to NG-RAN.

Observation: SA2 agrees on delivering the time distribution indication and the time synchronization error budget from AMF to NG-RAN.

When the UE mobility is considered, the target NG-RAN node should know the time distribution indication information and the time synchronization error budget information. Also the different time synchronization error budget may be configured at the target NG-RAN after the handover.

And in split NG-RAN architecture, the time synchronization information could be used at the gNB-DU to decide the reference time distribution to the UEs, e.g. determining the periodicity of SIB9 including the time reference information. So the time distribution indication information and the time synchronization error budget information need to be delivered over F1 interface.

Proposal 1: The time distribution indication and the time synchronization error budget are delivered over NG, Xn and F1 interfaces. And the AMF should be able to update the parameters to the target NG-RAN after the handover.

The time synchronization distribution parameters from AMF to NG-RAN is different from the TSCAI. The TSCAI should be configured per-TSC service, so it is delivered with QoS flow level. However, SA2 has not linked the time synchronization distribution parameters to TSC service even it is certainly useful for TSC services. It is used to control the time distribution for a UE.

- 4.15.9.4	Procedures for management of 5G access stratum time distribution

The AF can use the procedure to activate, update or delete the 5G access stratum time distribution for one UE, group of UEs or any UE using the DNN and S-NSSAI.

[…]

- 4.16.2.2	AM Policy Association Modification initiated by the PCF

[…]

3.	The (V-)PCF in the roaming case and the PCF in a non-roaming case sends Npcf_AMPolicyControl_UpdateNotify including AM Policy Association ID associated with the SUPI defined in TS 29.507 [32]. The policy update may include Service Area Restrictions, UE-AMBR, RFSP index value, access stratum time distribution indication, and Uu time synchronization error budget. If an AF has previously subscribed to event request for allocation of service area coverage outcome in step 1, the (V-)PCF checks if the allocated service area coverage was changed and sends a respective notification to the AF using Npcf_AMPolicyAuthorization_Notify as defined in clause 6.3.1.18 in TS 23.503 [20].

4.	The AMF deploys and stores the updated Access and mobility related policy information, which includes storing the Service Area Restrictions and Policy Control Request Trigger of AM Policy Association, provisioning of the Service Area Restrictions to the UE, provisioning the RFSP index, UE-AMBR and Service Area Restrictions to the NG-RAN, and optionally the access stratum time distribution indication, and Uu time synchronization error budget to the NG-RAN.

Referring the SA2’s agreed CR [1], the time distribution indication and the time synchronization error budget are the parameters for per-UE policy.

Proposal 2: The time distribution indication and the time synchronization error budget is delivered per-UE level.

Regarding the details about the parameter name and parameter meaning, SA2 already agrees on the parameter names and their descriptions. So RAN3 could follow the names and the description in SA2’ spec if they don’t make confusion in RAN point of view.

Related with the value for the time synchronization error budget, SA2 hasn’t made a clear description so it would depend on RAN3 discussion. In our understanding of SA2’s agreements, the time synchronization error budget could be expressed with time value, as aligned with the Reference Time Information (RTI).

Also the granularity for the time synchronization error budget would rather be aligned with or be larger than the granularity for the RTI, i.e. 10ns as defined in TS 38.330 and TS 38.331. Even if the smaller granularity is indicated, NG-RAN couldn’t control the time synchronization with the UE with the smaller value.

To support strict synchronization accuracy requirements of TSC applications, the gNB may signal 5G system time reference information to the UE using unicast or broadcast RRC signalling with a granularity of 10 ns.

In Sect. 16.8 in TS 38.300,

Regarding the maximum value for the time synchronization error budget, there is no clear evidence either. So RAN3 needs to discuss which value is reasonable for the range of the time synchronization error budget.

The synchronicity budget for the 5G system within the global time domain shall not exceed 900 ns.

[…]

The synchronicity budget for the 5G system within a working clock domain shall not exceed 900 ns.

The time synchronization service is tightly related with the cyber-physical control applications. So, the value range for the time synchronization budget may need to be defined on the service requirements for the cyber-physical control applications as defined in TS 22.104. In TS 22.104, the requirements for the clock synchronisation are defined and among the requirements, the synchronicity budget for the 5G system is defined like this:

So we think that the maximum value for the time synchronization error budget could be equal or lower than 900 ns in Rel.17.

Proposal 3: The time synchronization error budget is expressed with a granularity of 10ns and the range would be between 10ns and 900ns.

Conclusion

Observation: SA2 agrees on delivering the time distribution indication and the time synchronization error budget from AMF to NG-RAN.

Proposal 1: The time distribution indication and the time synchronization error budget are delivered over NG, Xn and F1 interfaces. And the AMF should be able to update the parameters to the target NG-RAN after the handover.

Proposal 2: The time distribution indication and the time synchronization error budget is delivered per-UE level.

Proposal 3: The time synchronization error budget is expressed with a granularity of 10ns and the range would be between 10ns and 900ns.

Base on the Proposals 1, 2 and 3, the following is proposed for TPs:

Proposal 4: RAN3 agrees TP for the BL CRs for NGAP as in Annex. And the AS Time Distribution Parameters IE should be similarly included in XnAP and in F1AP.

Reference

- S2-2106772, Control of the 5G reference time distribution, Nokia, Nokia Shangai Bell, Samsung, Huawei, HiSilicon, ETRI, Ericsson, ZTE, NTT DOCOMO

- Annex TP for TS 38.413

********************************

Skip to the next change

********************************

- 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

[…]

For each PDU session, if the PDU Session Expected UE Activity Behaviour IE is included in the INTIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, handle this information as specified in TS 23.501 [9].

If the AS Time Distribution Parameters IE is included in the INITIAL CONTEXT SETUP REQUEST message the NG-RAN node shall, if supported, handle this information to provide the 5G system time reference information to the UE as specified in TS 23.501 [9].

Interactions with Initial UE Message procedure:

The NG-RAN node shall use the AMF UE NGAP ID IE and RAN UE NGAP ID IE received in the INITIAL CONTEXT SETUP REQUEST message as identification of the logical connection even if the RAN UE NGAP ID IE had been allocated in an INITIAL UE MESSAGE message sent over a different NG interface instance.

********************************

Skip to the next change

********************************

- 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

[…]

For each PDU session, if the PDU Session Expected UE Activity Behaviour IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, handle this information as specified in TS 23.501 [9].

If the AS Time Distribution Parameters IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message the NG-RAN node shall, if supported, handle this information to provide the 5G system time reference information to the UE as specified in TS 23.501 [9].

Interactions with RRC Inactive Transition Report procedure:

If the RRC Inactive Transition Report Request IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message and set to "single RRC connected state report" and the UE is in RRC_CONNECTED state, the NG-RAN node shall, if supported, send one RRC INACTIVE TRANSITION REPORT message to the AMF to report the RRC state of the UE.

********************************

Skip to the next change

********************************

- 9.2.2.1	INITIAL CONTEXT SETUP REQUEST

This message is sent by the AMF to request the setup of a UE context.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| Old AMF | O |  | AMF Name 9.3.3.21 |  | YES | reject |
| UE Aggregate Maximum Bit Rate | C-ifPDUsessionResourceSetup |  | 9.3.1.58 |  | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| GUAMI | M |  | 9.3.3.3 |  | YES | reject |
| PDU Session Resource Setup Request List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Setup Request Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>PDU Session NAS-PDU | O |  | NAS-PDU 9.3.3.4 |  | - |  |
| >>S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >>PDU Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the PDU Session Resource Setup Request Transfer IE specified in subclause 9.3.4.1. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network | YES | reject |
| UE Security Capabilities | M |  | 9.3.1.86 |  | YES | reject |
| Security Key | M |  | 9.3.1.87 |  | YES | reject |
| Trace Activation | O |  | 9.3.1.14 |  | YES | ignore |
| Mobility Restriction List | O |  | 9.3.1.85 |  | YES | ignore |
| UE Radio Capability | O |  | 9.3.1.74 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.3.1.61 |  | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.54 |  | YES | ignore |
| NAS-PDU | O |  | 9.3.3.4 |  | YES | ignore |
| Emergency Fallback Indicator | O |  | 9.3.1.26 |  | YES | reject |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| Location Reporting Request Type | O |  | 9.3.1.65 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| IAB Authorized | O |  | 9.3.1.129 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.3.1.146 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.147 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.148 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.149 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.3.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.3.1.160 |  | YES | ignore |
| RG Level Wireline Access Characteristics | O |  | OCTET STRING | Specified in TS 23.316 [34]. Indicates the wireline access technology specific QoS information corresponding to a specific wireline access subscription. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| AS Time Distribution Parameters | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifPDUsessionResourceSetup | This IE shall be present if the PDU Session Resource Setup List IE is present. |

********************************

Skip to the next change

********************************

- 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GC.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| UE Security Capabilities | O |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| New Security Context Indicator | O |  | 9.3.1.55 |  | YES | reject |
| PDU Session Resource Switched List |  | 1 |  |  | YES | ignore |
| >PDU Session Resource Switched Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Acknowledge Transfer | M |  | OCTET STRING | Containing the Path Switch Request Acknowledge Transfer IE specified in subclause 9.3.4.9. | - |  |
| >>PDU Session Expected UE Activity Behaviour | O |  | Expected UE Activity Behaviour 9.3.1.94 | Expected UE Activity Behaviour for the PDU Session. | YES | ignore |
| PDU Session Resource Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resource Released Item |  | 1..<maxnoofPDUSessions> |  |  | - |  |
| >>PDU Session ID | M |  | 9.3.1.50 |  | - |  |
| >>Path Switch Request Unsuccessful Transfer | M |  | OCTET STRING | Containing the Path Switch Request Unsuccessful Transfer IE specified in subclause 9.3.4.20. | - |  |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| RRC Inactive Transition Report Request | O |  | 9.3.1.91 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Redirection for Voice EPS Fallback | O |  | 9.3.1.116 |  | YES | ignore |
| CN Assisted RAN Parameters Tuning | O |  | 9.3.1.119 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.3.1.128 |  | YES | ignore |
| Enhanced Coverage Restriction | O |  | 9.3.1.140 |  | YES | ignore |
| Extended Connected Time | O |  | 9.3.3.31 |  | YES | ignore |
| UE Differentiation Information | O |  | 9.3.1.144 |  | YES | ignore |
| NR V2X Services Authorized | O |  | 9.3.1.146 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.147 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.148 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.149 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 QoS Parameters | O |  | 9.3.1.150 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| CE-mode-B Restricted | O |  | 9.3.1.155 |  | YES | ignore |
| UE User Plane CIoT Support Indicator | O |  | 9.3.1.160 |  | YES | ignore |
| UE Radio Capability ID | O |  | 9.3.1.142 |  | YES | reject |
| AS Time Distribution Parameters | O |  | 9.3.1.x |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is 256. |

********************************

Skip to the next change

********************************

- 9.3.1.x	AS Time Distribution Parameters

This IE provides the 5G access stratum time distribution information to the NG-RAN.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| AS Time Distribution | O |  | ENUMERATED (enabled, …) | Indicates that the AS time distribution via Uu reference point should be activated or deactivated. |
| Time Synchronization Error Budget | O |  | INTEGER (1..90, …) | The unit is 10 ns. Indicates the time synchronization error budget for the time synchronization service as described in TS 23.501 [9]. |