3GPP TSG-RAN WG3 Meeting #114-e	R3-215167

# E-meeting, 1-11 Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0658 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Paging DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Orange, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Paging DRX IE is defined, and referred to the TS 38.304 and TS 36.304. But the time unit is missing in the semantics descriptions. Even in TS 38.304 and TS 36.304, the time unit is not described wherein. When further checking the TS 38.331/TS 36.331, the time unit is in terms of radio frames, e.g., the default paging cycle is defined as follows. - Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on Also in section 9.3.1.40 of TS 38.473, it is already specified that the paging DRX is in unit of radio frame. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the time unit (in terms of radio frames) in the semantics descriptions for the Paging DRX IE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the paging DRX. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear about the time unit of the Paging DRX IE, thus there is a risk of the misalignment for the paging cycle. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.90 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0668 TS 36.413 CR 1833 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-214018 Rev1: R3-215167 Resubmit to RAN3-114-e meeting against latest specification. |  |  |  |  |  |  |  |  |

Change Begins

9.3.1.90	Paging DRX

This IE indicates the Paging DRX as defined in TS 38.304 [12] and TS 36.304 [29].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging DRX | M |  | ENUMERATED (32, 64, 128, 256, …) | Unit in radio frame. |

Change Ends