3GPP TSG-RAN WG3 Meeting #114-e	R3-215191

# Online, 1st - 11th November 2021

Title:	[DRAFT] Reply LS on latest progress and outstanding issues in SA WG2

Response to:	LS (R3-214710/S2-2106833) on latest progress and outstanding issues in SA WG2 from SA2

Release:	Release 17

Work Item:	5MBS, NR_MBS-Core

Source:	Ericsson (will be RAN3)

To:	SA2

Cc:	RAN2

Contact Person:

#### Name:	Alexander Vesely

Tel. Number:

E-mail Address:	alexander dot vesely at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 thanks SA2 for their LS on latest progress and outstanding issues in SA WG2.

RAN3 has reviewed all the Editor’s Notes in the latest version of TS 23.247 explicitly expecting RAN WG feedback for resolution and would like to provide the following feedback (quotes from TS 23.247 in dark blue or red):

7.2.3.2	Xn based handover from MBS supporting NG-RAN node

#### 7.2.3.3	N2 based handover from MBS supporting NG-RAN node

Editor's note:	Details on data forwarding, if applicable, needs to wait for RAN WGs.

RAN3 feedback

Handover from MBS supporting gNB to MBS supporting gNB: There is no need to perform data forwarding in case of mobility between supporting gNBs due to the nature of multicast with distribution of identical copies of multicast user data to nodes within an MBS service area.

Handover from MBS supporting gNB to MBS not-supporting gNB: In this case the MBS supporting gNB prepares the handover in a way that the non supporting target gNB can behave as during a normal handover, including the possibility to perform data forwarding.

#### 7.2.3.5	Minimization of data loss

Editor's note:	Details for how to minimize data loss between the source NG-RAN node and the target NG-RAN node should be aligned with 3GPP RAN WGs.

RAN3 feedback

RAN3 suggests that SA2 rather includes a reference to TS 38.300 than replicating specification in TS 23.247.

#### 7.2.3.6	Xn/N2 based handover for inactive MBS session

If the MBS session in "Inactive" state, comparing the handover procedure for the MBS session in "Active" state, the following additional procedures apply:

-	For Xn based handover, the information that MBS session is inactive is provided from the source RAN node towards the target RAN node.

-	For N2 based handover, the information that MBS session is inactive is provided from SMF towards the target RAN node.

-	For the MBS supporting NG-RAN node, the target NG-RAN establishes the shared tunnel with the MB-UPF as usual. However, as the MBS session is inactive state, the NG-RAN node will not allocate related radio resource.

-	After a handover to a not supporting MBS target RAN node, the SMF removes the associated QoS flow(s) information.

Editor's Notes: RAN confirmation is required.

RAN3 feedback

-	RAN3 does not foresee to provide an explicit indication of inactive MBS Sessions to the target gNB.

-	RAN3 further foresees that the NG-RAN node may release NG-U during deactivated multicast MBS sessions.

-	On removing associated QoS flow information upon handover to a non supporting MBS target RAN node, we would not expect associated QoS flow information to be understood by the target RAN node, therefore the SMF action seems to be superfluous, if it involves SM signalling towards the targe RAN node.

#### 7.2.1.3 Multicast session join and session establishment procedure

Editor's Note: The implication of not triggering PDU Session UP activation in NG-RAN when SMF informs the NG-RAN of UE join requires RAN collaboration.

RAN3 feedback

The implication of not triggering UP activation for the associated PDU Session at joining is that the associated PDU Session Resource Context is not established in the gNB and therefore at least the joining information needs to be provided to the UE Context in the gNB by means different than PDU Session Resource signalling on NG.

2. Actions:

To SA2 group.

ACTION: 	RAN3 asks SA2 to take the feedback provided into account

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting#114bis-e	17th - 26th January 2022

TSG-RAN WG3 Meeting#115-e	21st February - 3rd March 2022