3GPP TSG-RAN WG3 Meeting #114-e	R3-215206

# Online, 1st - 11th November 2021

Agenda Item:	22.2.4

Source:	Ericsson

Title:	[TP for BL CR for 38.473 and 38.463] On F1 and E1 protocol aspects for NR MBS

Document for:	Discussions & Approval

# 1	Introduction

This document follows discussions in R3-21xxxx and provides corresponding Text Proposals for E1AP and F1AP.

# 2	Text Proposal for 38.473

This chapter contains a Text Proposal for the BL CR against TS 38.473.

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.y	MBS Session Resource Context Management procedures

### 8.y.1	MBS Session Context Setup

#### 8.y.1.1	General

The purpose of the MBS Session Resource Context Setup procedure is to establish the MBS Session Resource Context. The procedure uses MBS Session Resource associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.y.1.2	Successful Operation

Figure 8.y.1.2-1: MBS Session Resource Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending MBS SESSION RESOURCE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the MBS Session Resource context, it replies to the gNB-CU with MBS SESSION RESOURCE CONTEXT SETUP RESPONSE.

Editor’s Note: Further specification text is FFS.

#### 8.y.1.3	Unsuccessful Operation

Figure 8.y.1.3-1: UE Context Setup Request procedure: unsuccessful Operation

If the gNB-DU is not able to establish an F1 MBS Session Resource context it shall consider the procedure as failed and reply with the MBS SESSION RESOURCE CONTEXT SETUP FAILURE message.

Editor’s Note: Further specification text is FFS.

#### 8.y.1.4	Abnormal Conditions

Editor’s Note: specification text is FFS.

### 8.y.2	UE Context Modification (gNB-CU initiated)

#### 8.y.2.1	General

The purpose of the gNB-CU initiated MBS Session Resource Context Modification procedure is to enable the gNB-CU to request the modification an established MBS Session Resource context. The procedure uses MBS Session Resource associated signalling.

Editor’s Note: specification text is FFS.

#### 8.y.2.2	Successful Operation

Figure 8.y.2.2-1: MBS Session Resource Context Modification procedure. Successful operation

The gNB-CU initiates the procedure by sending MBS SESSION RESOURCE CONTEXT MODIFICATION REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with MBS SESSION RESOURCE CONTEXT SETUP RESPONSE.

Editor’s Note: Further specification text is FFS.

#### 8.y.2.3	Unsuccessful Operation

Figure 8.y.2.3-1: UE Context Modification procedure. Unsuccessful operation

If the gNB-DU is not able to even partially modify the F1 MBS Session Resource context it shall consider the procedure as failed and reply with the MBS SESSION RESOURCE CONTEXT SETUP FAILURE message.

Editor’s Note: Further specification text is FFS.

#### 8.y.2.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

### 8.y.3	MBS Session Resource Context Release Request (gNB-DU initiated)

#### 8.y.3.1	General

The purpose of the MBS Session Resource Context Release Request procedure is to enable the gNB-DU to request the gNB-CU to release the MBS Session Resource Context. The procedure uses MBS Session Resource associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.y.3.2	Successful Operation

Figure 8.y.3.2-1: MBS Session Resource Context Release Request procedure. Successful operation

The gNB-DU hosting the MBS Session Resource Context initiates the procedure by sending the MBS SESSION RESOURCE CONTEXT RELEASE REQUEST message to the gNB-CU.

Editor’s Note: Further specification text is FFS.

Interactions with MBS Session Resource Context Release procedure:

Upon receiving the MBS SESSION RESOURCE CONTEXT RELESE REQUEST message the gNB-CU should initiate the MBS Session Resource Context Release procedure.

Editor’s Note: Further specification text is FFS.

#### 8.y.3.3	Abnormal Conditions

n/a

### 8.y.4	MBS Session Resource Context Release (gNB-CU initiated)

#### 8.y.4.1	General

The purpose of the MBS Session Resource Context Release procedure is to enable the gNB-CU to order the release of the MBS Session Resource Context. The procedure uses MBS Session Resource associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.y.4.2	Successful Operation

Figure 8.y.4.2-1: MBS Session Resource Context Release procedure. Successful operation

The gNB-CU initiates the procedure by sending the MBS SESSION RESOURCE CONTEXT RELEASE COMMAND message to the gNB-DU.

Upon reception of the UE CONTEXT RELEASE COMMAND message, the gNB-DU shall release all related resource and reply with the MBS SESSION RESOURCE CONTEXT RELEASE COMPLETE message.

#### 8.y.4.3	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

### 8.y.5	MBS F1-U Path Request

#### 8.y.5.1	General

The purpose of the MBS F1-U Path Request procedure is to enable the gNB-DU to request the setup of F1-U bearers. The procedure uses MBS Session Resource associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.y.5.2	Successful Operation

Figure 8.y.5.2-1: MBS F1-U Path Reqeust procedure. Successful operation

The gNB-DU initiates the procedure by sending the MBS F1-U PATH REQUEST message to the gNB-CU.

Upon reception of the MBS F1-U PATH REQUEST message, the gNB CU shall allocate F1-U resources and reply with the MBS F1-U PATH RESPONSE message.

#### 8.y.5.3	Unsuccessful Operation

Editor’s Note: Further specification text is FFS.

#### 8.y.5.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

### 8.y.6	MBS F1-U Path Release Request

#### 8.y.6.1	General

The purpose of the MBS F1-U Path Release Request procedure is to enable the gNB-DU to request the release of F1-U bearers. The procedure uses MBS Session Resource associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.y.6.2	Successful Operation

Figure 8.y.6.2-1: MBS F1-U Path Release Request procedure. Successful operation

The gNB-DU initiates the procedure by sending the MBS F1-U PATH RELEASE REQUEST message to the gNB-CU.

Upon reception of the MBS F1-U PATH RELEASE REQUEST message, the gNB CU shall release indicated F1-U resources and reply with the MBS F1-U PATH RELEASE RESPONSE message.

#### 8.y.6.3	Unsuccessful Operation

Editor’s Note: Further specification text is FFS.

#### 8.y.6.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| Candidate SpCell List |  | 0..1 |  |  | YES | ignore |
| >Candidate SpCell Item IEs |  | 1 .. <maxnoofCandidateSpCells> |  |  | EACH | ignore |
| >>Candidate SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | - |  |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | If included, it should be set to true. This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuraiton 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC basedUL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | ignore |
| Masked IMEISV | O |  | 9.3.1.55 |  | YES | ignore |
| Serving PLMN | O |  | PLMN ID 9.3.1.14 | Indicates the PLMN serving the UE. | YES | ignore |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | C-ifDRBSetup |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| New gNB-CU UE F1AP ID | O |  | gNB-CU UE F1AP ID 9.3.1.4 |  | YES | reject |
| RAN UE ID | O |  | OCTET STRING (SIZE (8)) |  | YES | ignore |
| Trace Activation | O |  | 9.3.1.88 |  | YES | ignore |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS Information | M |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| Configured BAP Address | O |  | 9.3.1.111 | The BAP address configured for the corresponding child IAB-node. | YES | reject |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier |  |  | 9.3.1.121 |  | - |  |
| >>RLC mode | M |  | 9.3.1.27 |  | - |  |
| Conditional Inter-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, …) |  | - | - |
| >Target gNB-DU UE F1AP ID | C-ifCHOmod |  | 9.3.1.5 | Allocated at the target gNB-DU | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.151 |  | YES | ignore |
| Serving NID | O |  | 9.3.1.155 |  | YES | reject |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| MBS Session ID Information for UE Context | O |  | 9.3.1.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULUPTNLInformation | Maximum no. of ULUP TNL Information allowed towards one DRB, the maximum value is 2. |
| maxnoofCandidateSpCells | Maximum no. of SpCells allowed towards one UE, the maximum value is 64. |
| maxnoofQoSFlows | Maximum no. of flows allowed to be mapped to one DRB, the maximum value is 64. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |
| maxnoofSLDRBs | Maximum no. of SL DRB allowed for NR sidelink communication per UE, the maximum value is 512. |
| maxnoofPC5QoSFlows | Maximum no. of PC5 QoS flow allowed towards one UE for NR sidelink communication, the maximum value is 2048. |
| maxnoofAdditionalPDCPDuplicationTNL | Maximum no. of additional UP TNL Information allowed towards one DRB, the maximum value is 2. |

| Condition | Explanation |
| --- | --- |
| ifDRBSetup | This IE shall be present only if the DRB to Be Setup List IE is present. |
| ifCHOmod | This IE shall be present if the CHO Trigger IE is present and set to "CHO-replace". |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| ServCellIndex | O |  | INTEGER (0..31, ...) |  | YES | reject |
| SpCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | YES | ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Action Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9] for EN-DC case or MR-DC Resource Coordination Information IE as defined in TS 38.423 [28] for NGEN-DC and NE-DC cases. | YES | ignore |
| RRC Reconfiguration Complete Indicator | O |  | 9.3.1.30 |  | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 | Includes the DL-DCCH-Message IE as defined in subclause 6.2 of TS 38.331 [8], encapsulated in a PDCP PDU. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 1.. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| >>SCellIndex | M |  | INTEGER (1..31) |  | - |  |
| >>SCell UL Configured | O |  | Cell UL Configured 9.3.1.33 |  | - |  |
| >>servingCellMO | O |  | INTEGER (1..64) |  | YES | ignore |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | 1 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NR CGI 9.3.1.12 | SCell Identifier in gNB | - |  |
| SRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SRB to Be Setup Item IEs |  | 1..<maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| >>Duplication Indication | O |  | ENUMERATED (true, ..., false) | This IE is ignored if the Additional Duplication Indication IE is present. | - |  |
| >>Additional Duplication Indication | O |  | ENUMERATED (three, four, …) |  | YES | ignore |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | M |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >>>DRB Information |  | 1 |  | Shall be used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | - |  |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, ..., false) | Indication on whether DC based PDCP duplication is configured or not. If included, it should be set to true. | YES | reject |
| >>DC Based Duplication Activation | O |  | Duplication Activation 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. < maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| >>Transmission Stop Indicator | O |  | 9.3.1.209 |  | YES | ignore |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>CHOICE QoS Information | O |  |  |  | - |  |
| >>>E-UTRAN QoS | M |  | 9.3.1.19 | Used for EN-DC case to convey E-RAB Level QoS Parameters | - |  |
| >>>DRB Information |  | 1 |  | Used for NG-RAN cases | YES | ignore |
| >>>>DRB QoS | M |  | 9.3.1.45 |  | - |  |
| >>>>S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >>>>Notification Control | O |  | 9.3.1.56 |  | - |  |
| >>>>Flows Mapped to DRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>>>>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>>>>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >>>>>QoS Flow Mapping Indication | O |  | 9.3.1.72 |  | YES | ignore |
| >>>>>TSC Traffic Characteristics | O |  | 9.3.1.141 | Traffic pattern information associated with the QFI. Details in TS 23.501 [21]. | YES | ignore |
| >>UL UP TNL Information to be setup List |  | 1 |  |  | - |  |
| >>>UL UP TNL Information to Be Setup Item IEs |  | 1 .. <maxnoofULUPTNLInformation> |  |  | - |  |
| >>>>UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>UL Configuration | O |  | UL Configuration 9.3.1.31 | Information about UL usage in gNB-DU. | - |  |
| >>DL PDCP SN length | O |  | ENUMERATED(12bits,18bits , ...) |  | YES | ignore |
| >>UL PDCP SN length | O |  | ENUMERATED (12bits, 18bits, ...) |  | YES | ignore |
| >>Bearer Type Change | O |  | ENUMERATED (true, …) |  | YES | ignore |
| >>RLC Mode | O |  | 9.3.1.27 |  | YES | ignore |
| >>Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of CA based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>DC Based Duplication Configured | O |  | ENUMERATED (true, …, false) | Indication on whether DC based PDCP duplication is configured or not. | YES | reject |
| >>DC Based Duplication Activation | O |  | 9.3.1.36 | Information on the initial state of DC based UL PDCP duplication. This IE is ignored if the RLC Duplication Information IE is present. | YES | reject |
| >>Additional PDCP Duplication TNL List |  | 0..1 |  |  | YES | ignore |
| >>>Additional PDCP Duplication TNL Items |  | 1 .. <maxnoofAdditionalPDCPDuplicationTNL> |  |  | EACH | ignore |
| >>>>Additional PDCP Duplication UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - |  |
| >>>>BH Information | O |  | 9.3.1.114 |  | YES | ignore |
| >>RLC Duplication Information | O |  | 9.3.1.146 |  | YES | ignore |
| SRB To Be Released List |  | 0..1 |  |  | YES | reject |
| >SRB To Be Released Item IEs |  | 1.. <maxnoofSRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Inactivity Monitoring Request | O |  | ENUMERATED (true, ...) |  | YES | reject |
| RAT-Frequency Priority Information | O |  | 9.3.1.34 |  | YES | reject |
| DRX configuration indicator | O |  | ENUMERATED(release,...) |  | YES | ignore |
| RLC Failure Indication | O |  | 9.3.1.66 |  | YES | ignore |
| Uplink TxDirectCurrentList Information | O |  | 9.3.1.67 |  | YES | ignore |
| GNB-DU Configuration Query | O |  | ENUMERATED (true, ...) | Used to request the gNB-DU to provide its configuration. | YES | reject |
| gNB-DU UE Aggregate Maximum Bit Rate Uplink | O |  | Bit Rate 9.3.1.22 | The gNB-DU UE Aggregate Maximum Bit Rate Uplink is to be enforced by the gNB-DU. | YES | ignore |
| Execute Duplication | O |  | ENUMERATED (true, ...) | This IE may be sent only if duplication has been configured for the UE. | YES | ignore |
| RRC Delivery Status Request | O |  | ENUMERATED (true, …) | Indicates whether RRC DELIVERY REPORT procedure is requested for the RRC message. | YES | ignore |
| Resource Coordination Transfer Information | O |  | 9.3.1.73 |  | YES | ignore |
| servingCellMO | O |  | INTEGER (1..64, ...) |  | YES | ignore |
| Need for Gap | O |  | ENUMERATED (true, …) | Indicate gap for SeNB configured measurement is requested.It only applied to NE DC scenario. | Yes | ignore |
| Full Configuration | O |  | ENUMERATED (full, ...) |  | YES | reject |
| Additional RRM Policy Index | O |  | 9.3.1.90 |  | YES | ignore |
| Lower Layer Presence Status Change | O |  | 9.3.1.94 |  | Yes | ignore |
| BH RLC Channel to be Setup List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Setup Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | M |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | M |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Modified List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Modified Item IEs |  | 1 .. <maxnoofBHRLCChannels> |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| >>CHOICE BH QoS information | O |  |  |  |  |  |
| >>>BH RLC CH QoS | M |  | QoS Flow Level QoS Parameters 9.3.1.45 | Shall be used for SA case. |  |  |
| >>>E-UTRAN BH RLC CH QoS | M |  | E-UTRAN QoS 9.3.1.19 | Shall be used for EN-DC case. |  |  |
| >>>Control Plane Traffic Type | M |  | 9.3.1.115 |  |  |  |
| >>RLC Mode | O |  | 9.3.1.27 |  | - |  |
| >>BAP Control PDU Channel | O |  | ENUMERATED (true, …) |  | - |  |
| >>Traffic Mapping Information | O |  | 9.3.1.95 |  | - |  |
| BH RLC Channel to be Released List |  | 0..1 |  |  | YES | reject |
| >BH RLC Channel to be Released Item IEs |  | 1 .. <maxnoofBHRLCChannels > |  |  | EACH | reject |
| >>BH RLC CH ID | M |  | 9.3.1.113 |  | - |  |
| NR V2X Services Authorized | O |  | 9.3.1.116 |  | YES | ignore |
| LTE V2X Services Authorized | O |  | 9.3.1.117 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.119 | This IE applies only if the UE is authorized for NR V2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.3.1.118 | This IE applies only if the UE is authorized for LTE V2X services. | YES | ignore |
| PC5 Link Aggregate Bit Rate | O |  | Bit Rate 9.3.1.22 | Only applies for non-GBR and unicast QoS Flows. | YES | ignore |
| SL DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Setup Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Modified Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| >>SL DRB Information |  | 1 |  |  | YES | ignore |
| >>>SL DRB QoS | M |  | PC5 QoS Parameters 9.3.1.122 |  | - |  |
| >>>Flows Mapped to SL DRB Item |  | 1 .. <maxnoofPC5QoSFlows> |  |  | - |  |
| >>>>PC5 QoS Flow Identifier | M |  | 9.3.1.121 |  | - |  |
| >>RLC mode | O |  | 9.3.1.27 |  | - |  |
| SL DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >SL DRB to Be Released Item IEs |  | 1 .. <maxnoofSLDRBs> |  |  | EACH | reject |
| >>SL DRB ID | M |  | 9.3.1.120 |  | - |  |
| Conditional Intra-DU Mobility Information | O |  |  |  | YES | reject |
| >CHO Trigger | M |  | ENUMERATED (CHO-initiation, CHO-replace, CHO-cancel, …) |  | - | - |
| >Candidate Cells To Be Cancelled List | C-ifCHOcancel | 0 .. <maxnoofCellsinCHO> |  |  | - | - |
| >>Target Cell ID | M |  | NR CGI 9.3.1.12 |  | - | - |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | YES | ignore |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| MBS Session ID Information for UE Context | O |  | 9.3.1.x1 |  | YES | ignore |

### 9.2.x	MBS Session Context Management messages

#### 9.2.x.1	MBS SESSION RESOURCE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of an MBS Session Resource context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU MBS F1AP ID | M |  | FFS |  | YES | reject |
| Global MBS Session ID | M |  | FFS |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | The cell for which the MBS Session Resource is setup. | YES | reject |
| CU To DU MBS RRC Information | M |  | FFS |  | YES | reject |
| MRB to Be Setup List |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | FFS |  | - |  |
| >MRB QoS | M |  | DRB QoS 9.3.1.45 |  | - |  |
| >S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >Flows Mapped to MRB Item |  | 1 .. <maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| >UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of F1-UP UL information. | - |  |
| >DL PDCP SN length | M |  | ENUMERATED (12bits, 18bits, ...) |  | - |  |
| Further IEs FFS |  |  |  |  |  |  |

#### 9.2.x.2	MBS SESSION RESOURCE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of an MBS Session Resource context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| DU To CU MBS RRC Information | M |  | FFS |  | YES | reject |
| G-RNTI | O |  | FFS | G-RNTI allocated at the gNB-DU | YES | ignore |
| MRB Setup List |  | 1 .. <maxnoofMRBs> |  | The List of MRBs which are successfully established. | YES | ignore |
| >MRB ID | M |  | FFS |  | - |  |
| >DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| MRB Failed to Setup List |  | 0 .. <maxnoofMRBs> |  |  | YES | ignore |
| >MRB ID | M |  | FFS |  | - |  |
| >Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.3	MBS SESSION RESOURCE CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the MBS Session Resource context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | FFS |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	MBS SESSION RESOURCE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide MBS Session Resource context  changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| CU To DU MBS RRC Information | M |  | FFS |  | YES | reject |
| MRB to Be Setup or Modified List |  | 0 .. <maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | FFS |  | - |  |
| >MRB QoS | O |  | DRB QoS 9.3.1.45 |  | - |  |
| >S-NSSAI | M |  | 9.3.1.38 |  | - |  |
| >Flows Mapped to MRB Item |  | 0 .. <maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.63 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.45 |  | - |  |
| MRB to Be Released List |  | 0 .. <maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | FFS |  | - |  |

#### 9.2.x.5	MBS SESSION RESOURCE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a MBS Session Resource context .

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| DU To CU MBS RRC Information | M |  | FFS |  | YES | reject |
| G-RNTI | O |  | FFS | G-RNTI allocated at the gNB-DU | YES | ignore |
| MRB Setup List |  | 1 .. <maxnoofMRBs> |  | The List of MRBs which are successfully established. | YES | ignore |
| >MRB ID | M |  | FFS |  | - |  |
| >DL UP TNL Information | O |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - |  |
| MRB Failed to Setup or Modify List |  | 0 .. <maxnoofMRBs> |  |  | YES | ignore |
| >MRB ID | M |  | FFS |  | - |  |
| >Cause | O |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.6	MBS SESSION RESOURCE CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-DU to indicate an MBS Session Resource context modification failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| Further IEs FFS |  |  |  |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.7	MBS SESSION RESOURCE CONTEXT RELEASE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to release the MBS Context Resource context

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| Further IEs FFS |  |  |  |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.x.8	MBS SESSION RESOURCE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the MBS Session Resource context..

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| Further IEs FFS |  |  |  |  |  |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.x.9	MBS SESSION RESOURCE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the MBS Session Resource context.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| Further IEs FFS |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.10	MBS F1-U PATH REQUEST

This message is sent by the gNB-DU to request the setup of F1-U resources

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| F1-U Bearers Be Setup List |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | FFS |  | - |  |
| >DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of F1-UP DL information. | - |  |
| >Session Area ID | O |  | FFS | in case the F1-U bearer is associated to a Session Area ID for location dependent services | - |  |
| Further IEs FFS |  |  |  |  |  |  |

#### 9.2.x.11	MBS F1-U PATH RESPONSE

This message is sent by the gNB-CU to reply to the request for setup of F1-U resources.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| F1-U Bearers Be Setup List |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | FFS |  | - |  |
| >UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of F1-UP UL information. | - |  |
| >Session Area ID | O |  | FFS | in case the F1-U bearer is associated to a Session Area ID for location dependent services | - |  |
| Further IEs FFS |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.12	MBS F1-U PATH RELEASE REQUEST

This message is sent by the gNB-DU to request the release of F1-U resources

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| F1-U Bearers Be Released List |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | FFS |  | - |  |
| >DL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of F1-UP DL information. | - |  |
| Further IEs FFS |  |  |  |  |  |  |

#### 9.2.x.13	MBS F1-U PATH RELEASE RESPONSE

This message is sent by the gNB-CU to reply to the request for release of F1-U resources.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | FFS |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | FFS |  | YES | reject |
| F1-U Bearers Be Released List |  | 1 .. <maxnoofMRBs> |  |  | YES | reject |
| >MRB ID | M |  | FFS |  | - |  |
| >UL UP TNL Information | M |  | UP Transport Layer Information 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of F1-UP UL information. | - |  |
| Further IEs FFS |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x1	MBS Session ID Information for UE Context

This IE contains a list of MBS Session identifiers to add or remove from the UE Context.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MBS Session Setup List |  | 0..1 |  |  |
| >MBS Session Setup Item |  | 1..<maxnoofMBSSessions> |  |  |
| >>Global MBS Session ID | M |  | FFS |  |
| MBS Session Remove List |  | 0..1 |  |  |
| >MBS Session Remove Item |  | 1..<maxnoofMBSSessions> |  |  |
| >>Global MBS Session ID | M |  | FFS |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSSessionIDs | Maximum no. of MBS Session IDs within a UE Context. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# 3	Text Proposal for E1AP

This chapter contains a Text Proposal for the BL CR against TS 38.463.

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.z	MBS Session Resource Context Management procedures

### 8.z.1	MBS Session Resource Context Setup

#### 8.z.1.1	General

The purpose of the MBS Session Resource Context Setup procedure is to allow the gNB-CU-CP to establish an MBS Session Resource context in the gNB-CU-UP. The procedure uses MBS Session Resource context associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.z.1.2	Successful Operation

Figure 8.z.1.2-1: MBS Session Resource Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the MBS SESSION RESOURCE CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the MBS SESSION RESOURCE CONTEXT SETUP RESPONSE message.

Editor’s Note: Further specification text is FFS.

#### 8.z.1.3	Unsuccessful Operation

Figure 8.z.1.3-1: MBS Session Resource Context Setup procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot establish the requested MBS Session Resource context it shall consider the procedure as failed and respond with the MBS SESSION RESOURCE CONTEXT SETUP FAILURE message and an appropriate cause value.

Editor’s Note: Further specification text is FFS.

#### 8.z.1.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

### 8.z.2	MBS Session Resource Context Modification (gNB-CU-CP initiated)

#### 8.z.2.1	General

The purpose of the gNB-CU-CP initiated MBS Session Resource Context Modification procedure is to allow the gNB-CU-CP to modify an established MBS Session Resource Context in the gNB-CU-UP. The procedure uses MBS Session Resource context associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.z.2.2	Successful Operation

Figure 8.z.2.2-1: MBS Session Resource Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the MBS SESSION RESOURCE CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to perform at least partially the requested modifications it replies to the gNB-CU-CP with the MBS SESSION RESOURCE CONTEXT MODIFICATION RESPONSE message.

Editor’s Note: Further specification text is FFS.

#### 8.z.2.3	Unsuccessful Operation

Figure 8.z.2.3-1: MBS Session Resource Context Modification procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot successfully perform any of the requested modifications, it shall respond with a MBS SESSION RESOURCE CONTEXT MODIFICATION FAILURE message and an appropriate cause value.

Editor’s Note: Further specification text is FFS.

#### 8.z.2.4	Abnormal Conditions

Editor’s Note: Further specification text is FFS.

### 8.z.3	MBS Session Resource Context Release (gNB-CU-CP initiated)

#### 8.z.3.1	General

The purpose of the gNB-CU-CP initiated MBS Session Resource Context Release procedure is to allow the gNB-CU-CP to command the release of an MBS Session Resource Context. The procedure uses MBS Session Resource context associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.z.3.2	Successful Operation

Figure 8.z.3.2-1: Bearer Context Release procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the MBS SESSION RESOURCE CONTEXT RELEASE COMMAND message to the gNB-CU-UP. Upon reception of the MBS SESSION RESOURCE CONTEXT RELEASE COMMAND message, the gNB-CU-UP shall release all related signalling and user data transport resources and reply with the MBS SESSION RESOURCE CONTEXT RELEASE COMPLETE message.

Editor’s Note: Further specification text is FFS.

#### 8.z.3.3	Abnormal Conditions

Not applicable.

### 8.z.4.5	Bearer Context Release Request (gNB-CU-UP initiated)

#### 8.z.4.1	General

The purpose of the Bearer Context Release Request procedure is to allow the gNB-CU-UP to request the gNB-CU-CP to release an MBS Session Resource context. The procedure uses MBS Session Resource context associated signalling.

Editor’s Note: Further specification text is FFS.

#### 8.z.4.2	Successful Operation

Figure 8.z.4.2-1: MBS Session Resource context Release Request procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the MBS SESSION RESOURCE CONTEXT RELEASE REQUEST message to the gNB-CU-CP.

Editor’s Note: Further specification text is FFS.

Interactions with MBS Session Resource Context Release procedure:

Upon reception of the MBS SESSION RESOURCE CONTEXT RELEASE REQUET message the gNB-CU-CP should initiate the MBS Session Resource Context Release procedure.

Editor’s Note: Further specification text is FFS.

#### 8.3.5.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.x1	MBS SESSION RESOURCE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup an MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| Global MBS Session ID | M |  | FFS |  | YES | reject |
| MBS Session Resource To Setup | M |  | 9.3.3.y1 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y1	MBS Session Resource To Setup

This IE contains MBS session resource related information used at MBS Session Resource Context Setup Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| S-NSSAI | M |  | 9.3.1.9 |  |
| MRB To Setup List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >SDAP Configuration | M |  | 9.3.1.39 |  |
| >PDCP Configuration | M |  | 9.3.1.38 |  |
| >QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | DRB QoS 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.x2	MBS SESSION RESOURCE CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested MBS Session Resource context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | FFS |  | YES | reject |
| MBS Session Resource Setup | M |  | 9.3.3.y2 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y2	MBS Session Resource Setup

This IE contains setup PDU session resource related information used at MBS Session Resource Context Setup Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 | FFS on presence |
| MRB Setup List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.x3	MBS SESSION RESOURCE CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | FFS |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.x4	MBS SESSION RESOURCE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to modify an MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | FFS |  | YES | ignore |
| MBS Session Resource To Modify | O |  | 9.3.3.y3 |  | YES | reject |
| gNB-DU ID | O |  | 9.3.1.65 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y3	MBS Session Resource To Modify

This IE contains PDU session resource to modify related information used at MBS Session Resource Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MRB To Setup List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | FFS |  |
| >SDAP Configuration | M |  | 9.3.1.39 |  |
| >PDCP Configuration | M |  | 9.3.1.38 |  |
| >QoS Flow Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | DRB QoS 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB |
| MRB To Modify List |  | 0..<maxnoofmRBs> |  |  |
| >MRB ID | M |  | FFS |  |
| >SDAP Configuration | O |  | 9.3.1.39 |  |
| >PDCP Configuration | O |  | 9.3.1.38 |  |
| >DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  |
| >Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. |
| >MRB QoS | O |  | DRB QoS 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB |
| >Data Forwarding Information MRB | O |  | FFS | Editor’s Note: Contains one or several DL addresses for individual data forwarding at HO and the request for the CU-UP to start data forwarding. The same IE could also carry a request to stop data forwarding (FFS) |
| MRB To Remove List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | FFS |  |
| S-NSSAI | O |  | 9.3.1.9 |  |
| FFS on additonal IEs fo cover location dependent services with additional NG-U DL terminations |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.x5	MBS SESSION RESOURCE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested MBS Session Resource context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | FFS |  | YES | ignore |
| MBS Session Resource Modified | O |  | 9.3.3.y4 |  | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.3.y4	MBS Session Resource Modified

This IE contains modified PDU session resource related information used at MBS Session Context Modification Response

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| FFS on additonal IEs fo cover location dependent services with additional NG-U DL terminations |  |  |  |  |
| MRB Setup List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | FFS |  |
| >Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16 |  |
| >Cause | M |  | 9.3.1.2 |  |
| MRB Modified List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | FFS |  |
| >UL UP Transport Layer Information | O |  | UP Transpor Layer Information 9.3.2.1 |  |
| >Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  |
| >Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >PDCP SN Status Information | O |  | 9.3.1.58 | PDCP SN Status Information for individual data forwarding at HO. |
| MRB Failed To Modify List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | FFS |  |
| >Cause | M |  | 9.3.1.2 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is FFS. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.x6	MBS SESSION RESOURCE CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | FFS |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.x7	MBS SESSION RESOURCE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU-CP to command the gNB-CU-UP to release an E1 MBS Session Resource context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | FFS |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.2.x8	MBS SESSION RESOURCE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of the E1 MBS Session Resource Context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | FFS |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.x9	MBS SESSION RESOURCE CONTEXT RELEASE REQUEST

This message is sent by the gNB-CU-UP to request the release of the E1 MBS Session Resource Context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | FFS |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | FFS |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>