3GPP TSG-RAN3 #114-e	R3-215214

1st November – 11th November 2021

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0814 | rev | - | Current version: | 15.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Cell to be Broadcast List presence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS38.413 the description for the Write-Replace Warning procedure states that “If the Warning Area List IE is not included in the WRITE-REPLACE WARNING REQUEST message, the NG-RAN node shall broadcast the indicated message in all of the cells within the NG-RAN node.”. Over the F1AP, the Warning Area List IE is equivalent to the Cell to be Broadcast List IE. However, an equivalent description describing what the gNB-DU behaviour should be when the Cell to be Broadcast List IE is absent is not present. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact on previous versions of the specifications |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a sentence stating the gNB-DU behaviour in absence of the Cell to be Broadcast List IE, which mirrors the gNB behaviour in case the Warning Area List IE is not received by the gNB over NGAP |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The behaviour described by TS38.413 in case the Warning Area List IE is not received by the gNB and the behaviour of the gNB-DU described in TS38.473 in case the Cell to be Broadcast List IE is not received by the gNB are not consistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<< NEXT CHANGE >>>>>>

## 8.5	Warning Message Transmission Procedures

### 8.5.1	Write-Replace Warning

#### 8.5.1.1	General

The purpose of Write-Replace Warning procedure is to start or overwrite the broadcasting of warning messages. The procedure uses non UE-associated signalling.

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2-1: Write-Replace Warning procedure: successful operation

The gNB-CU initiates the procedure by sending a WRITE-REPLACE WARNING REQUEST message to the gNB-DU.

Upon receipt of the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall prioritise its resources to process the warning message.

The gNB-DU acknowledges the WRITE-REPLACE WARNING REQUEST message by sending a WRITE-REPLACE WARNING RESPONSE message to the gNB-CU.

Upon receipt of the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the WRITE-REPLACE WARNING RESPONSE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the WRITE-REPLACE WARNING RESPONSE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the Notification Information IE is included in the PWS System Information IE in the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall use this information to avoid that duplications trigger new broadcast or replace existing broadcast.

If the gNB-DU receives a WRITE-REPLACE WARNING REQUEST message with the Notification Information IE in the PWS System Information IE which are different from those of ongoing broadcast warning messages, and if the SIB Type IE is set to "8", the gNB-DU shall broadcast the received warning message concurrently with other ongoing messages.

If the gNB-DU receives a WRITE-REPLACE WARNING REQUEST message with the Notification Information IE in the PWS System Information IE which are different from those of ongoing broadcast warning messages, and if the SIB Type IE is set to the value other than '8', the gNB-DU shall use the newly received one to replace the ongoing broadcast warning message with the same value of SIB Type IE.

If the SIB Type IE in the PWS System Information IE in the WRITE-REPLACE WARNING REQUEST message is set to "8" and if a value "0" is received in the Number of Broadcast Requested IE and if the Repetition Period IE is different from "0", the gNB-DU shall broadcast the received warning message indefinitely.

If Additional SIB Message List IE is included in PWS System Information IE, the gNB-DU shall store all SIB message(s) in PWS System Information IE, and consider that the first segment of public warning message is included in SIB message IE, and the remaining segments are listed in Additional SIB Message List IE in segmentation sequence order.

If the Cell to be Broadcast List IE is not included in the WRITE-REPLACE WARNING REQUEST message, the gNB-DU shall broadcast the indicated message in all of the In-Service Cells.

#### 8.5.1.3	Unsuccessful Operation

Not applicable.

#### 8.5.1.4	Abnormal Conditions

Not applicable.

<<<<<< END OF CHANGES >>>>>>