3GPP TSG-RAN WG3 Meeting #114-e	R3-215238

E-meeting, November 1 - 11, 2021

Agenda item:	9.3.5

Source:	Qualcomm Incorporated, Huawei, China Telecom, T-Mobile USA

Title:	Inter MN Resume without SN Change

WID/SID:	TEI-16

Document for:	Agreement

# Introduction

MN and SN have different coverage. When MN changes, it is possible to keep SN unchanged. MN change without SN change has been supported for handover but not supported for RRC resume.

This paper proposes procedure for inter MN resume without SN change.

This topic was discussed in last meeting, refer to chairman notes below. The RAN2 impacts should have been supported in R16 DC/CA enhancement work item. LS R3-214360 was sent to RAN2 to double confirm.

Summary of offline disc R3-214467 noted LS (R3-214360) to RAN2 to confirm RAN2 spec supporting status agreed It is proposed to technically endorse below items, which can be converted into agreement once RAN2 confirmed the scenario. Introduce “UE Context Reference at the S-NG-RAN node” (with SN node ID and SN XnAP UE ID, as defined in Handover Request) into Retrieve UE Context Response message Target MN needs to indicate the source MN that the UE context in the SN is kept or not? To be continued...

# Background

MN and SN have different coverage. So, when MN changes, SN may not need to be changed. Keeping SN in MN change avoids the signalling and delay of reconfiguring SN.

Inter-MN mobility without SN change has been supported in handover (defined in section 10.7.2 of TS 37.340, as shown below in figure 1), but not supported in RRC resume.

RRC resume with SCG (as shown in figure 2) has been defined in R16 as part of DCCA enhancement WID.

Figure 1: Inter-MN handover with/without MN initiated SN change procedure

Observation 1: MN change without SN change is supported in handover, but not supported in RRC resume.

Figure 2: SCG Restore in RRC resume

Observation 2: SCG restore during (intra-MN) RRC resume has been support in R16, as part of DCCA enhancement work item.

Observation 3: Inter or intra-MN RRC resume is transparent to UE. UE does not know whether new cell belongs to same or different gNB.

Observation 4: Supporting inter-MN RRC resume without SN change does not require new RAN2/Uu standard changes.

# Inter MN resume without SN change

Inter MN resume and inter MN handover are very similar. From RAN3 perspective, the key difference is in the UE context transfer. Handover uses Handover Request to transfer UE context. Resume procedure uses Retrieve UE Context procedure to transfer the UE context.

We can take the handover procedure as reference to define the procedure of inter-MN resume without SN change.

Proposal 1: Working assumption: Support inter MN resume without SN Change, with inter-MN handover without SN change procedure as reference.

Figure 2 below shows the inter MN resume without SN change procedure.

Figure 3: Inter MN resume without SN change

In RRC Resume, gNB can reconfigure SCG or indicate UE to restore SCG.

RRCResume-v1610-IEs ::=             SEQUENCE {

idleModeMeasurementReq-r16          ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreMCG-SCells-r16               ENUMERATED {true}                                               OPTIONAL, -- Need N

restoreSCG-r16                      ENUMERATED {true}                                               OPTIONAL, -- Need N

mrdc-SecondaryCellGroup-r16         CHOICE {

nr-SCG-r16                          OCTET STRING (CONTAINING RRCReconfiguration),

eutra-SCG-r16                       OCTET STRING

}                                                                                                   OPTIONAL, -- Cond RestoreSCG

needForGapsConfigNR-r16             SetupRelease {NeedForGapsConfigNR-r16}                          OPTIONAL, -- Need M

nonCriticalExtension                SEQUENCE{}                                                      OPTIONAL

}

In RRC Resume Complete message, UE sends scg-Response to the network to confirm the SCG configuration.

RRCResumeComplete-v1610-IEs ::=         SEQUENCE {

idleMeasAvailable-r16                   ENUMERATED {true}                                                       OPTIONAL,

measResultIdleEUTRA-r16                 MeasResultIdleEUTRA-r16                                                 OPTIONAL,

measResultIdleNR-r16                    MeasResultIdleNR-r16                                                    OPTIONAL,

scg-Response-r16                        CHOICE {

nr-SCG-Response                         OCTET STRING (CONTAINING RRCReconfigurationComplete),

eutra-SCG-Response                      OCTET STRING

}                                                                                                               OPTIONAL,

ue-MeasurementsAvailable-r16            UE-MeasurementsAvailable-r16                                            OPTIONAL,

mobilityHistoryAvail-r16                ENUMERATED {true}                                                       OPTIONAL,

mobilityState-r16                       ENUMERATED {normal, medium, high, spare}                                OPTIONAL,

needForGapsInfoNR-r16                   NeedForGapsInfoNR-r16                                                   OPTIONAL,

nonCriticalExtension                    RRCResumeComplete-v1640-IEs                                             OPTIONAL

}

Up to implementation, MN can decide SCG restore in Msg4 without UE measurement report, as shown in figure 3. In case MN decision in Msg4 (RRCResume) is not appropriate, the UE would know in the following SCG RACH procedure and gNB would know from Msg5 (RRCResumeComplete).

Handover Request message includes “UE Context Reference at the S-NG-RAN node” (with SN node ID and SN XnAP UE ID) for target MN to send SN Addition Request to the same SN. So, we can add the same IE into Retrieve UE Context Response message. Last meeting has technically endorsed this change:

Introduce “UE Context Reference at the S-NG-RAN node” (with SN node ID and SN XnAP UE ID, as defined in Handover Request) into Retrieve UE Context Response message.

Observation 5: Last meeting has technically endorsed: Introduce “UE Context Reference at the S-NG-RAN node” (with SN node ID and SN XnAP UE ID, as defined in Handover Request) into Retrieve UE Context Response message.

In Handover Request Acknowledge, target MN tells source MN to keep SN UE context and the accepted DRBs, PDU sessions. We can define a new message Retrieve UE Context Confirm to carry this information.

Proposal 2: Technically endorse: Define new XnAP message “Retrieve UE Context Confirm” to carry following IEs equivalent to Handover Request Acknowledge

- SN UE Context Kept Indicator

- DRBs transferred to MN

- PDU Session Resources Admitted List.

If this new message is not received, the old MN releases the SN for the UE. The old MN may start a timer after UE Context Retrieval Response to guard this new message.

This UE Context Retrieval enhancement is also useful for RRC Reestablishment at different MN. With the SN information, the new MN may configure the same SCG to the UE after RRC Reestablishment.

Observation 6: Above enhancements for UE Context Retrieval procedure is also useful for RRC Reestablishment with SN unchanged.

# Conclusions

Based on the above discussion, we have following observations and proposals:

Observation 1: MN change without SN change is supported in handover, but not supported in RRC resume.

Observation 2: SCG restore during (intra-MN) RRC resume has been support in R16, as part of DCCA enhancement work item.

Observation 3: Inter or intra-MN RRC resume is transparent to UE. UE does not know whether new cell belongs to same or different gNB.

Observation 4: Supporting inter-MN RRC resume without SN change does not require new RAN2/Uu standard changes.

Proposal 1: Working assumption: Support inter MN resume without SN Change, with inter-MN handover without SN change procedure as reference.

To support proposal 1, following standard enhancements are needed:

Observation 5: Last meeting has technical endorsed: Introduce “UE Context Reference at the S-NG-RAN node” (with SN node ID and SN XnAP UE ID, as defined in Handover Request) into Retrieve UE Context Response message.

Proposal 2: Technically endorse: Define new XnAP message “Retrieve UE Context Confirm” to carry following IEs equivalent to Handover Request Acknowledge

- SN UE Context Kept Indicator

- DRBs transferred to MN

- PDU Session Resources Admitted List.

Proposal 3: When RAN2 confirmation is received, convert the working assumption and technical endorsements into agreements and agree the CR based on the agreements.

Observation 6: Above enhancements for UE Context Retrieval procedure is also useful for RRC Reestablishment with SN unchanged.

Corresponding CR with above changes is provided in R3-21xxxx.

# References

[1] 3GPP TS 37.340 Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity