3GPP TSG-RAN WG3 Meeting #114-e	R3-215240

Online, 1 - 11 November 2021

Agenda item:	  22.2.2

Source:	  Qualcomm Incorporated, Huawei, Lenovo, Motorola Mobility

Title:		(TP to TS 38.470 BL CR) Multicast Session Management

Document for:		Other

# 1	Introduction

- As discussed in [1], new group paging procedure needs to be introduced:

Proposal 7: Introduce non UE associated Class 2 F1AP: Multicast Group Paging procedure to support group paging from gNB-CU to gNB-DU. And the F1AP: MULTICAST GROUP PAGINGE message includes:

- MBS Session ID

- Paging Cell List

- (Optional) Paging UE List { UE Identity Index value, DRX value, Paging UE Cell List}

- In this paper, we provide the text proposal to BL CR of TS 38.470 in Section 3.

# 2 Reference

- R3-215138	Consideration on Multicast Session Management	Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

- R3-215139	(TP to TS 38.410 BL CR) Multicast Session Management	Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

- R3-215140	(TP to TS 38.413 BL CR) Multicast Session Management	Huawei, Lenovo, Motorola Mobility, Qualcomm Incorporated

- R3-215272	(TP to TS 38.460 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

- R3-215273	(TP to TS 38.463 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

- R3-215271	(TP to TS 38.423 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

# 3	Text Proposal

-----------------Start of the Changes-------------------

### 6.1.x	NR MBS procedures

The F1 MBS procedures are listed below:

- Multucast Group Paging procedure