3GPP TSG-RAN WG3 Meeting #114-e	R3-215373

E-meeting, 1-11 Nov 2021

Agenda item:	  22.2.4

Source:	  Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

Title:		(TP to TS 38.463 BL CR) Support of Multicast Distribution Setup and Release

Document for:		Other

1	Introduction

- As discussed in [1], the following proposals are achieved:

Proposal 9. Introduce  non-UE associated Class1 E1AP procedures：

- Multicast Distribution Setup procedure, triggered by the gNB-CU-UP

  - in case the gNB-CU-UP decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

  - in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the gNB-CU-CP shall provide IP multicast DL tunnel info to the gNB-CU-UP, to enable IP multicast transport.

- Multicast Distribution Release procedure, triggered by the gNB-CU-UP or gNB-CU-CP

- In this paper, we provide the text proposal to BL CR of TS 38.463 in Section 3.

2 Reference

[1] R3-215138 Consideration on Multicast Session Management. Huawei, CBN, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Unicom, China Telecom.

3	Text Proposal

-----------------Start of the Changes-------------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[4]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error".

[6]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[7]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[8]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[9]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specificaiton”.

[11]	3GPP TS 23.401: “General Packet Radio Service (GPRS) Enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[12]	3GPP TS 23.203: “Policy and Charging Control Architecture”.

[13]	3GPP TS 33.501: “Security Architecture and Procedures for 5G System”.

[14]	IETF RFC 5905: “Network Time Protocol Version 4: Protocol and Algorithms Specification”.

[15]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[16]	3GPP TS 38.414: “NG-RAN; NG Data Transport”.

[17]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[18]	3GPP TS 38.462: "NG-RAN; E1 Signalling Transport".

[19]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[20]	3GPP TS 23.501: "System Architecture for the 5G System".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[22]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[23]	3GPP TS 23.003: "Numbering, addressing and identification".

[24]	3GPP TS 32.422: "Trace control and configuration management".

[25]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[26]	3GPP TS 32.425: "Performance measurements; Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[27]	3GPP TS 37.320: "Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); Radio measurement collection for Minimization of Drive Tests (MDT);Overall description; Stage 2".

[28]	3GPP TS 38.474: "NG-RAN; F1 data transport".

[xx]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2”.

-----------------Start of the next Change-------------------

## 8.1	List of E1AP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure |  | Initiating Message |  | Successful Outcome |  | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Response message |  | Response message |  |
| Reset |  | RESET |  | RESET ACKNOWLEDGE |  |  |  |
| gNB-CU-UP E1 Setup |  | GNB-CU-UP E1 SETUP REQUEST |  | GNB-CU-UP E1 SETUP RESPONSE |  | GNB-CU-UP E1 SETUP FAILURE |  |
| gNB-CU-CP E1 Setup |  | GNB-CU-CP E1 SETUP REQUEST |  | GNB-CU-CP E1 SETUP RESPONSE |  | GNB-CU-CP E1 SETUP FAILURE |  |
| gNB-CU-UP Configuration Update |  | GNB-CU-UP CONFIGURATION UPDATE |  | GNB-CU-UP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-UP CONFIGURATION UPDATE FAILURE |  |
| gNB-CU-CP Configuration Update |  | GNB-CU-CP CONFIGURATION UPDATE |  | GNB-CU-CP CONFIGURATION UPDATE ACKNOWLEDGE |  | GNB-CU-CP CONFIGURATION UPDATE FAILURE |  |
| E1 Release |  | E1 RELEASE REQUEST |  | E1 RELEASE RESPONSE |  |  |  |
| Bearer Context Setup |  | BEARER CONTEXT SETUP REQUEST |  | BEARER CONTEXT SETUP RESPONSE |  | BEARER CONTEXT SETUP FAILURE |  |
| Bearer Context Modification (gNB-CU-CP initiated) |  | BEARER CONTEXT MODIFICATION REQUEST |  | BEARER CONTEXT MODIFICATION RESPONSE |  | BEARER CONTEXT MODIFICATION FAILURE |  |
| Bearer Context Modification Required (gNB-CU-UP initiated) |  | BEARER CONTEXT MODIFICATION REQUIRED |  | BEARER CONTEXT MODIFICATION CONFIRM |  |  |  |
| Bearer Context Release (gNB-CU-CP initiated) |  | BEARER CONTEXT RELEASE COMMAND |  | BEARER CONTEXT RELEASE COMPLETE |  |  |  |
| Resource Status Reporting Initiation |  | RESOURCE STATUS REQUEST |  | RESOURCE STATUS RESPONSE |  | RESOURCE STATUS FAILURE |  |
| IAB UP TNL Address Update |  | IAB UP TNL ADDRESS UPDATE |  | IAB UP TNL ADDRESS UPDATE ACKNOWLEDGE |  | IAB UP TNL ADDRESS UPDATE FAILURE |  |
| Multicast Distribution Setup |  | MULTICAST DISTRIBUTION SETUP REQUEST |  | MULTICAST DISTRIBUTION SETUP RESPONSE |  | MULTICAST DISTRIBUTION SETUP FAILURE |  |
| Multicast Distribution Release |  | MULTICAST DISTRIBUTION RELEASE REQUEST |  | MULTICAST DISTRIBUTION RELEASE RESPONSE |  |  |  |

Editor’s Note: It is FFS that whether the Multicast Session Activation procedure is needed or not.

Table 2: Class 2 procedures

| Elementary Procedure |  |  | Message |  |  |
| --- | --- | --- | --- | --- | --- |
| Error Indication |  |  | ERROR INDICATION |  |  |
| Bearer Context Release Request (gNB-CU-UP initiated) |  |  | BEARER CONTEXT RELEASE REQUEST |  |  |
| Bearer Context Inactivity Notification |  |  | BEARER CONTEXT INACTIVITY NOTIFICATION |  |  |
| DL Data Notification |  |  | DL DATA NOTIFICATION |  |  |
| UL Data Notification |  |  | UL DATA NOTIFICATION |  |  |
| Data Usage Report |  |  | DATA USAGE REPORT |  |  |
| gNB-CU-UP Counter Check |  |  | GNB-CU-UP COUNTER CHECK |  |  |
| gNB-CU-UP Status Indication |  |  | GNB-CU-UP STATUS INDICATION |  |  |
| MR-DC Data Usage Report |  |  | MR-DC DATA USAGE REPORT |  |  |
| Trace Start |  |  | TRACE START |  |  |
| Deactivate Trace |  |  | DEACTIVATE TRACE |  |  |
| Resource Status Reporting |  |  | RESOURCE STATUS UPDATE |  |  |
| Early Forwarding SN Transfer |  |  | EARLY FORWARDING SN TRANSFER |  |  |
| GNB-CU-CP Measurement Results Information |  |  | GNB-CU-CP MEASUREMENT RESULTS INFORMATION |  |  |

-----------------Start of the next Change-------------------

8.X	MBS Session Management Procedures

8.X.1	Multicast Distribution Setup

8.X.1.1	General

The purpose of the Multicast Distribution Setup procedure is to assign NG-U resources for one MBS session and the corresponding MBS QoS flows. The procedure uses non-UE-associated signalling.

8.X.1.2	Successful Operation

Figure 8.X.1.2-1: Multicast Distribution Setup procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending a MULTICAST DISTRIBUTION SETUP REQUEST message to the gNB-CU-CP. The gNB-CU-CP responds with a MULTICAST DISTRIBUTION SETUP RESPONSE message.

If the DL Transport Layer Information IE is included in the MULTICAST DISTRIBUTION SETUP REQUEST message, the gNB-CU-CP shall handle it as specified in TS 38.401 [2].

8.X.1.3	Unsuccessful Operation

Figure 8.X.1.3-1: Multicast Distribution Setup procedure: Unsuccessful Operation.

In case the Multicast Distribution cannot be setup successfully, the gNB-CU-CP shall response with the MULTICAST DISTRIBUTION SETUP FAILURE message to the gNB-CU-UP with an appropriate cause value in the Cause IE.

8.X.1.4	Abnormal Conditions

Not Applicable.

8.Y.1	Multicast Distribution Release

8.Y.1.1	General

The purpose of the Multicast Distribution Release procedure is to enable the release of already established NG-U resources for a given MBS session. The procedure uses non-UE-associated signalling.

8.Y.1.2	Successful Operation

Figure 8.X.1.2-1: Multicast Distribution Release procedure: Successful Operation.

The gNB-CU-UP or the gNB-CU-CP initiates the procedure by sending a MULTICAST DISTRIBUTION SETUP REQUEST message to the peer node.

Upon receipt of the MULTICAST DISTRIBUTION RELEASE REQUEST message, the receiving node shall send the MULTICAST DISTRIBUTION RELEASE RESPONSE message after successfully remove the corresponding NG-U resource for the MBS session.

8.Y.1.3	Unsuccessful Operation

Not Applicable.

8.Y.1.4	Abnormal Conditions

Not Applicable.

-----------------Start of the next Change-------------------

9.2.x	MBS Messages

9.2.x.1a	MULTICAST DISTRIBUTION SETUP REQUEST

This message is sent by the gNB-CU-UP to request the setup of the NG-U transport for a MBS Session.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.y |  | YES | ignore |
| DL Transport Layer Information | O |  | UP Transfer Layer Information 9.3.2.1 |  | YES | ignore |

9.2.x.1b	MULTICAST DISTRIBUTION SETUP RESPONSE

This message is sent by the gNB-CU-CP to confirm the setup of the NG-U transport for a MBS Session.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.y |  | YES | ignore |
| IP Multicast Address Information | O |  | 9.3.2.1 UP Transfer Layer Information |  | YES | ignore |
| >IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >GTP DL TEID | M |  | GTP-TEID 9.3.2.3 |  |  |  |

9.2.x.1c	MULTICAST DISTRIBUTION SETUP FAILURE

This message is sent by the gNB-CU-CP to indicate the setup of the NG-U transport for a MBS Session was failed.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.y |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

9.2.x.2a	MULTICAST DISTRIBUTION RELEASE REQUEST

This message is sent by the gNB-CU-CP or the gNB-CU-UP to request the release of the NG-U transport for a MBS Session.

Direction: gNB-CU-CP  gNB-CU-UP and gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.y |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

9.2.x.2b	MULTICAST DISTRIBUTION RELEASE RESPONSE

This message is sent by the gNB-CU-CP or the gNB-CU-UP to confirm the release of the NG-U transport for a MBS Session.

Direction: gNB-CU-CP  gNB-CU-UP and gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.x |  | YES | reject |
| Area Session ID | O |  | 9.3.1.y |  | YES | ignore |

-----------------Start of the next Change-------------------

9.3.1.x	MBS Session ID

This IE identifies a MBS Session. The definition and use of the MBS Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS Session ID | M |  | FFS |  |

-----------------Start of the next Change-------------------

9.3.1.y	Area Session ID

This IE identifies a Area Session. The definition and use of the Area Session ID is specified in TS 23.247 [xx].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Area Session ID | M |  | FFS |  |