3GPP TSG-RAN WG3#114-e                                             	                     R3-215322

E-Meeting, 1 – 11 November 2021

Agenda item:	9.3.6.1

Source:	Nokia, Nokia Shanghai Bell, Orange

Title: 	Dual connectivity Support at inter-MN handover

Document for:	Approval

# Discussion

Currently, a serving gNB receives the Allowed NSSAI from the beginning within the UE context in the Initial Context Setup Request message.

Whenever this serving gNB (MN) wants to add a secondary node (SN) or secondary cell it may have several candidates allowing to offload the particular PDU session and slice. The MN can use the Allowed NSSAI to pick the one candidate which would maximize the support of the other slices which are not yet active but which UE can use. The Allowed NSSAI represents the list of slices which the UE has requested and have been allowed. They are therefore likely to be used, even if not currently “active” (e.g. PDU session currently inactive).

There is however one case where the above function is broken, which is the SN addition during the inter-MN handover.

In case of inter-MN handover, the current Xn Handover Request (step 1 of the following call flow) does not provide the Allowed NSSAI of the UE to the target MN, which means that at step 2 of the following call flow (see section 10.7.2 of TS 37.340), the target MN may setup DC with a neighbour SN cell without knowing the Allowed NSSAI.

The target MN would typically receive the Allowed NSSAI only later, in the Path Switch Request Acknowledge message, but this would be too late: see TS 38.413 section 9.2.3.9:

#### 9.2.3.9	PATH SWITCH REQUEST ACKNOWLEDGE

This message is sent by the AMF to inform the NG-RAN node that the path switch has been successfully completed in the 5GC.

Direction: AMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| UE Security Capabilities | O |  | 9.3.1.86 |  | YES | reject |
| Security Context | M |  | 9.3.1.88 |  | YES | reject |
| … | O |  | 9.3.1.55 |  | YES | reject |
| Allowed NSSAI | M |  | 9.3.1.31 | Indicates the S-NSSAIs permitted by the network. | YES | reject |
| Core Network Assistance Information for RRC INACTIVE | O |  | 9.3.1.15 |  | YES | ignore |
| … | O |  | 9.3.1.91 |  | YES | ignore |

In order to have the Allowed NSSAI at the proper time, i.e. before the SN Addition, it must be provided to target MN together with the UE context within the Handover Request message.

Proposal 1: add the Allowed NSSAI IE to the Handover Request message to cover the missing case of SN addition during the inter-MN handover.

# Conclusion and Proposal

This paper has explained the shortcomings of the SN addition during the inter-MN handover and make following proposal:

Proposal 1: add the Allowed NSSAI IE to the Handover Request message to cover the case of SN addition during the inter-MN handover.

Proposal 2: agree the CR in [2].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO, Inc.

- R3-215323, Support of Dual Connectivity with Slicing