3GPP TSG-RAN WG3 Meeting #114e	R3-215331

Online, 1 – 11 November 2021

Agenda item:	 18.4.2

Source:		Lenovo, Motorola Mobility

Title:		Discussion on traffic load prediction

Document for:		Discussion and Decision

1	Introduction

In RAN3#110e [1], some agreements were achieved as below:

- As a starting point, focus on at least the following use cases: Energy saving, load balancing, traffic steering/mobility optimization (other use cases, e.g. optimization of physical layer parameters, are not precluded)

- Augmented information should be studied case by case, e.g. history info, info needed for prediction, etc.

- Study the enhancement of network interfaces to support AI enabled RAN intelligence based on the agreed use cases.

- Coordinate with other working groups later for NRM enhancement when needed.

- Detailed AI functionality and interface impacts could be studied case by case for the agreed use cases later.

- Reuse the existing procedures for SON/MDT as the baseline for data collection or SON related use case where it fits. And additional enhancement/new signaling is studied when needed.

In RAN3#112e [2], further agreements were achieved:

- Progress the prioritized use cases on energy saving, load balancing, traffic steering/mobility optimization, i.e. at least by identifying their impact on the specifications, before considering any new use case.

- Where ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases.

- Security aspects should be considered and coordinated with other working groups later if needed.

In RAN3#113e [3], more were agreed:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

- AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

- To improve the load balancing decisions at a gNB (gNB-CU), a gNB can request load predictions from a neighbouring node. Details of the procedure are FFS.

- In case of CU-DU split architecture:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

- Other possible locations of the AI/ML Model Training and AI/ML Model Inference are FFS.

In this paper, we would further discuss the FFS issues for AI/ML based load prediction.

2	Discussion

Last RAN3 meeting agreed that:

- AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

- AI/ML Model Training and Model Inference are both located in the gNB-CU.

In case of split architectures, since CU has an overall view for the resource status of the related DUs via the existing F1/E1 interface, and CU has more computation resource, it is common understanding that it is suitable for the gNB-CU rather than the gNB-DU to perform AI/ML Model training, but which one performing AI/ML Model Inference needs further discussion.

As the figure 1 shows, if it is the gNB-DU performs AI/ML Model Inference, the gNB-CU needs to send the AI/ML Model to the gNB-DU after model training is complete. After the gNB-DU gets the AI/ML Model, it can use its current/historical resource information for load prediction. Since gNB-CU requests load prediction from gNB-DU to exchange between the serving and neighbor gNB-CUs for load balancing decision, the gNB-DU would send the load prediction to gNB-CU based on the request. When the size of AI/ML Model is huge or AI/ML Model re-training is frequent, the impacts on F1 interface need further consideration, e.g. how to transfer AI/ML Model between the gNB-CU and gNB-DU.

Figure 1 AI/ML Model Inference at gNB-DU

Observation 1: When gNB-DU performs AI/ML Model Inference for load prediction, the impact on F1 interface needs further consideration, e.g. transfer AI/ML Model between the gNB-CU and gNB-DU.

As the figure 2 shows, if gNB-CU predicts load information, the gNB-CU can obtain input for prediction, e.g. current/historical resource information, from the gNB-DU via RESOURCE STATUS REQUEST/ RESOURCE STATUS RESPONSE/ RESOURCE STATUS UPDATE procedure. After the gNB-CU gets the inference input, it can use the AI/ML Model to predict load information.

Figure 2 AI/ML Model Inference at gNB-CU

Observation 2: The gNB-CU can obtain inputs for load prediction, e.g. current/historical resource information, from the gNB-DU via RESOURCE STATUS REQUEST/ RESOURCE STATUS RESPONSE/ RESOURCE STATUS UPDATE procedure.

Based on above analysis, RAN3 can support AI/ML Model Inference for load prediction at gNB-DU, and the details need further study e.g. how to transfer AI/ML Model via the F1 interface.

Proposal 1: Support AI/ML Model Inference for load prediction at gNB-DU, and the details need further study e.g. how to transfer AI/ML Model via the F1 interface.

Currently, the resource status exchange can be supported between two RAN nodes, i.e. current/historical load/resource information is achieved via RESOURCE STATUS REQUEST/ RESOURCE STATUS RESPONSE/ RESOURCE STATUS UPDATE procedure. For load balancing decisions at a gNB or gNB-CU, a gNB/ gNB-CU can request load predictions from a neighbouring gNB/ gNB-CU, as Figure 3 shows, current procedure for resource status exchange can be as baseline for exchanging predicted load information.

Figure 3: exchange for traffic load prediction

Step 1: RAN node 1 requests RAN node 2 to provide the predicted load/resource information, e.g. via the RESOURCE STATUS REQUEST message. Optionally, the demanded accuracy for load/resource prediction may be indicated to RAN node 2, i.e. only when accuracy of the predicted load/resource information meets the demanded value, RAN node 2 can feedback its load/resource prediction to RAN node 1.

Step 2: RAN node 2 decides whether it can accept the request from RAN node 1, if so, it can respond RAN node 1 e.g. via the RESOURCE STATUS RESPONSE message.

Step 3: According to the request from RAN node 1, RAN node 2 can use ML model to predict load/resource information based on the input of the neighbor nodes and itself.

The input for load/resource prediction can at least include:

•	current/historical number of active UEs;

•	current/historical resource utilization, i.e. the usage of the PRBs per cell and per SSB area for DL/UL traffic;

•	current/historical number of RRC Connections.

After model inference in RAN node 2, the output of load/resource prediction can at least include:

•	predicted number of active UEs within a certain time window in the future;

•	predicted resource utilization, i.e. the usage of the PRBs per cell and per SSB area for DL/UL traffic within a certain time window in the future;

•	predicted number of RRC Connections within a certain time window in the future.

Step 4: According to the request from RAN node 1 in Step 1, RAN node 2 provides the output of load/resource prediction to the RAN node 1 e.g. via the RESOURCE STATUS UPDATE message. Optionally, the actual accuracy of the predicted load/resource information may also be provided to RAN node 1.

Similarly, RAN node 1 may predict its load/resource information. Based on the load/resource prediction of itself and neighbor cells that achieved from neighbor node(s), when the predicted traffic is overloaded in RAN node 1, it can make load balancing decision e.g. select a proper neighbor cell for handover or cell reselection.

Proposal 2: Input for load/resource prediction can at least include:

•	current/historical number of active UEs;

•	current/historical resource utilization, i.e. the usage of the PRBs per cell and per SSB area for DL/UL traffic;

•	current/historical number of RRC Connections.

Proposal 3: Output of load/resource prediction can at least include:

•	predicted number of active UEs within a certain time window in the future ;

•	predicted resource utilization, i.e. the usage of the PRBs per cell and per SSB area for DL/UL traffic within a certain time window in the future;

•	predicted number of RRC Connections within a certain time window in the future.

Proposal 4: A RAN node can request its neighbor RAN node to provide load/resource prediction via RESOURCE STATUS REQUEST message, and the demanded prediction accuracy may be indicated to the neighbor RAN node.

Proposal 5: The neighbor RAN node should respond whether the demanded prediction for traffic load/resource can be provided via RESOURCE STATUS RESPONSE message.

Proposal 6: The neighbor RAN node should provide the predicted load/resource information that demanded by the peer RAN node as well as accuracy of the predicted load/resource information via RESOURCE STATUS UPDATE message.

3	Conclusion

In this contribution, the details of AI/ML based load prediction are discussed. We have the following observations and proposals:

Observation 1: When gNB-DU performs AI/ML Model Inference for load prediction, the impact on F1 interface needs further consideration, e.g. transfer AI/ML Model between the gNB-CU and gNB-DU.

Observation 2: The gNB-CU can obtain inputs for load prediction, e.g. current/historical resource information, from the gNB-DU via RESOURCE STATUS REQUEST/ RESOURCE STATUS RESPONSE/ RESOURCE STATUS UPDATE procedure.

Proposal 1: Support AI/ML Model Inference for load prediction at gNB-DU, and the details need further study e.g. how to transfer AI/ML Model via the F1 interface.

Proposal 2: Input for load/resource prediction can at least include:

•	current/historical number of active UEs;

•	current/historical resource utilization, i.e. the usage of the PRBs per cell and per SSB area for DL/UL traffic;

•	current/historical number of RRC Connections.

Proposal 3: Output of load/resource prediction can at least include:

•	predicted number of active UEs within a certain time window in the future ;

•	predicted resource utilization, i.e. the usage of the PRBs per cell and per SSB area for DL/UL traffic within a certain time window in the future;

•	predicted number of RRC Connections within a certain time window in the future.

Proposal 4: A RAN node can request its neighbor RAN node to provide load/resource prediction via RESOURCE STATUS REQUEST message, and the demanded prediction accuracy may be indicated to the neighbor RAN node.

Proposal 5: The neighbor RAN node should respond whether the demanded prediction for traffic load/resource can be provided via RESOURCE STATUS RESPONSE message.

Proposal 6: The neighbor RAN node should provide the predicted load/resource information that demanded by the peer RAN node as well as accuracy of the predicted load/resource information via RESOURCE STATUS UPDATE message.

Reference

[1] Chairman notes for RAN3#110e meeting.

[2] Chairman notes for RAN3#112e meeting.

[3] Chairman notes for RAN3#113e meeting.