3GPP TSG-RAN WG3 #114-e	R3-215461

1 - 11 Nov 2021

Online

Title: 	(TP for SON BL CR for TS 36.413) Introduce UE History Information in MR-DC

Source: 	ZTE, Lenovo, Motorola Mobility, China Unicom

Agenda item:	10.2.1.4

Document for:	Approval

1	Introduction and discussion

The discussion part can be seen in R3-215456 UE History Information in MR-DC.

Proposal 7: Both MN and SN UHI shall be sent from source MN to target MN through the HANDOVER REQUEST message during inter-MN handover.

This paper provides the corresponding TP for TS 36.413.

2	TP to TS 36.413

*** Start changed ***

9.2.1.7	Source eNB to Target eNB Transparent Container

The Source eNB to target eNB Transparent Container IE is an information element that is produced by the source eNB and is transmitted to the target eNB. For inter-system handovers to E-UTRAN, the IE is transmitted from the external handover source to the target eNB.

This IE is transparent to the EPC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RRC Container | M |  | OCTET STRING | Includes the RRC Handover Preparation Information message as defined in subclause 10.2.2 of TS 36.331 [16]. | - |  |  |
| E-RABs Information List |  | 0..1 |  |  | - |  |  |
| >E-RABs Information Item |  | 1 .. <maxnoof E-RABs> |  |  | EACH | ignore |  |
| >>E-RAB ID | M |  | 9.2.1.2 |  | - |  |  |
| >>DL Forwarding | O |  | 9.2.3.14 |  | - |  |  |
| >>DAPS Request Information | O |  | 9.2.1.155 |  | YES | ignore |  |
| Target Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |  |
| Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.1.39 |  | - |  |  |
| UE History Information | M |  | 9.2.1.42 |  | - |  |  |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the external handover source provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |  |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [16]) | YES | ignore |  |
| IMS voice EPS fallback from 5G | O |  | ENUMERATED (true, …) |  | YES | ignore |  |
| Additional RRM Policy Index | O |  | 9.2.1.39a |  | YES | ignore |  |
| UE Context Reference at Source | O |  | 9.2.1.144 |  | YES | ignore |  |
| Inter-system measurement Configuration | O |  | 9.2.1.151 |  | YES |  | ignore |
| Source Node ID | O |  | 9.2.1.152 |  | YES |  | ignore |
| Emergency Indicator | O |  | ENUMERATED (true, …) | Indicates an emergency EPS voice fallback | YES |  | ignore |
| UE History Information from SN | O |  | 9.2.1.42 | Used by target eNB for handover preparation as defined in 37.340 [47] |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofE-RABs | Maximum no. of E-RABs for one UE. Value is 256. |

*** End of changed ***