3GPP TSG-RAN WG3 Meeting #114 electronic	R3-215699

Online, 1-11 Nov 2021

Agenda Item:	18.4.3

Source:	CMCC

Title:	Remaining issues for AI based Mobility and Energy Saving

Document for:	Discussion and Decision

1	Introduction

In last RAN3#113 E-meeting, the potential standards impact for the use case of AI based mobility and energy saving on existing nodes, functions, and interfaces were discussed and the initial TP was endorsed [1].

There are still some open issues which have not been agreed due to the limited time. This contribution will address these open issues.

2	Discussion

2.1	Mobility

As captured in the TR [2], mobility aspects of SON that can be enhanced by the use of AI/ML include

- Reduction of the probability of unintended events

- UE Location/Mobility/Performance prediction

- Traffic Steering

Although above there are three aspects, in our understanding both the first aspect and third one are handover related use cases, since the three unintended events of the first aspect are too late HO, too early HO and HO to wrong Cell. So, in the following discussion we would like to classify the three aspects into two types: handover and UE trajectory prediction.

In last RAN3 E-meeting, potential standard impacts discussion on AI based mobility use cases, including the input information from UE, neighbouring RAN nodes, local node, as well as output data, were discussed together, without distinguishing the use case of handover from UE trajectory prediction. Actually, during the discussion, it is found that the detailed information could be different for these two cases, which to some extend made some information cannot achieve agreements since it is may be needed for one use case, but not necessary for the other one. So, we suggest to point out the related use case if the information is not necessary for all aspects.

Proposal 1: For the input/output information that not common for handover and UE trajectory prediction, the related use case should be indicated.

- Input Information from UE:

For the input Information from UE, currently the UE historical location information from MDT and predicted traffic are FFS:

The following data is required as input data for mobility optimization.

Input Information from UE:

- FFS UE historical location information from MDT, e.g., Latitude, longitude, altitude, cell ID

- Radio measurements related to serving cell and neighbouring cells associated with UE location information, e.g., RSRP, RSRQ, SINR

- UE historical serving cells and their locations

- Moving velocity

- FFS predicted traffic

In our understanding, the UE historical location information is quite helpful for the prediction of UE’s trajectory, also beneficial for the handover decision based on the trajectory prediction.

Therefore, we propose that:

Proposal 1: UE historical location information from MDT, e.g., Latitude, longitude, altitude, cell ID, could be used as the input information for both handover and UE trajectory prediction.

As for the predicted traffic, the information seems not so useful for UE mobility prediction, but it is beneficial for handover performance, e.g., help the source gNB to choose the right target cell to meet the QoS requirement of the predicted traffic, realize the propriate traffic steering for the purpose of load balancing, etc. Therefore, we propose that:

Proposal 2: UE predicted traffic information could be used as the input information for the use case of handover.

- Input Information from the neighbouring RAN nodes

For the input information from neighbouring RAN node, there are two FFS: QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc) and Information about the performance of handed over UEs

Input Information from the neighbouring RAN nodes:

- UE’s successful handover information in the past and received from neighboring RAN nodes

- UE’s history information from neighbor

- Position, resource status, FFS QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc.)

- Resource status and utilization prediction/estimation

- SON Reports of handovers that are successful, too-early, too-late, or handover to wrong (sub-optimal) cell

- FFS Information about the performance of handed over UEs

The QoS parameters of historical HO-ed UE of neighbouring nodes is useful for the handover use case, e.g., to help the source node to judge whether it is proper to handover specific UE with specific services to the neighbouring RAN node to meet the QoS requirement of the service. Whereas, the information is not useful for UE’s trajectory prediction.

Proposal 3: QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc) from neighbouring RAN nodes could be used as the input information for the use case of handover.

As for the second FFS, i.e., information about the performance of handed over UEs, it is not clear what kind of performance information it refers to and could be used, and clarification is needed.

Proposal 4: Clarification is needed for the information about the performance of handed over UEs from Neighbouring RAN node.

- Input Information from the local node

As for the input information from the local node, it is FFS whether new UE measurements are needed besides existing MDT and RRM measurements.

Input Information from the local node:

- UE trajectory prediction output (will be used by the RAN node internally)

- Local load prediction

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements). FFS on whether new UE measurements are needed.

In Rel-16 MDT, some new measurements about beam and sensor are introduced, as captured in TS37.320 [3]:

For NR, in addition to the logged measurement quantities of the camped cell, the best beam index (SSB Index) as well as best beam RSRP/RSRQ is logged as well as the 'number of good beams' associated to the cells within the R value range (which is configured by network for cell reselection) of the highest ranked cell as part of the beam level measurements. Sensor measurements are logged if available.

In our understand, these measurements are sufficient for both the use case of handover and UE trajectory prediction, and it seems that no more UE measurements could be provided.

Proposal 5:  No new UE measurements from the local node are needed for AI based mobility use cases.

- Output data

As for the output data, controversial lies in the UE trajectory prediction:

- FFS UE trajectory prediction (Latitude, longitude, altitude of UE over a future period of time)

- Estimated arrival probability in CHO and relevant confidence interval

- Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication

For the use case of UE trajectory prediction, the information is necessary seems it is the primary purpose of the use case. While for the use case of handover, the information is not so essential and the information of predicted handover target node and candidate cells in CHO seems sufficient for the decision of handover.

Proposal 6:  The information of UE trajectory prediction (Latitude, longitude, altitude of UE over a future period of time) could be the output data for the use case of UE trajectory prediction.

2.2	Energy saving

- Input Information from UE:

In last RAN3#113 E-meeting, following information has been agreed as the input Information for AI based Energy Saving:

- Current/Predicted resource status of ES-Cell and its neighbor nodes

- Current/Predicted energy information of ES-Cell and its neighbor nodes

- UE measurement report (e.g. UE RSRP, RSRQ, SINR measurement, etc)

If existing UE measurements are needed by a gNB for AI/ML-based network energy saving, RAN3 shall reuse the existing framework (including MDT and RRM measurements). FFS on whether new UE measurements are needed.

Editor’s Note: FFS other input information required for AI/ML-based network energy saving. FFS energy information is exact energy consumption value or energy efficiency gain.

As for the input information from UE side, we have agreed UE measurement report. In our understanding, cell level measurements are the baseline, and furthermore, the beam level measurements are also beneficial for the decision of the energy saving decision, i.e., handover UE to the proper target cell. Furthermore, the QoS requirement of UE’s service is also helpful for the energy saving, so that the energy saving solution would not deteriorate UE’s QoS performance.

Proposal 7: Use cell level and beam level UE measurements, as well as UE’s QoS requirement are used as input for AI based energy.

- Output data

It has been agreed that AI/ML-based network energy saving model can generate following information as output:

- Energy saving strategy

- Handover strategy, including recommended candidate cells for taking over the traffic

- Predicted energy information

Editor’s Note: FFS other output information expected from AI/ML-based network energy saving. FFS detailed granularity and action of energy saving strategy. FFS on accuracy of predicted energy saving decision.

AS for the details of the energy saving strategy, we suggest to at least include various granularity of energy saving, e.g., subframe shutdown, channel shutdown, carrier shutdown, deep sleep, device shutdown, as well as time span of the energy saving state.

Proposal 8: The energy saving strategy includes various granularity of energy saving, e.g., subframe shutdown, channel shutdown, carrier shutdown, deep sleep, device shutdown, as well as time span of the energy saving state.

3	Summary

This contribution addressed the open issues for AI based Mobility, and made following proposals:

Proposal 1: UE historical location information from MDT, e.g., Latitude, longitude, altitude, cell ID, could be used as the input information for both handover and UE trajectory prediction.

Proposal 2: UE predicted traffic information could be used as the input information for the use case of handover.

Proposal 3: QoS parameters of historical HO-ed UE (e.g., loss rate, delay, etc) from neighbouring RAN nodes could be used as the input information for the use case of handover.

Proposal 4: Clarification is needed for the information about the performance of handed over UEs from Neighbouring RAN node.

Proposal 5:  No new UE measurements from the local node are needed for AI based mobility use cases.

Proposal 6:  The information of UE trajectory prediction (Latitude, longitude, altitude of UE over a future period of time) could be the output data for the use case of UE trajectory prediction.

Proposal 7: Use cell level and beam level UE measurements, as well as UE’s QoS requirement are used as input for AI based energy.

Proposal 8: The energy saving strategy includes various granularity of energy saving, e.g., subframe shutdown, channel shutdown, carrier shutdown, deep sleep, device shutdown, as well as time span of the energy saving state.

4	Reference

[1] 	R3-214484, TP for AIRAN_MobilitySolution, CMCC, ZTE

[2]         3GPP TR 37.817-030, CMCC

[3]         3GPP TS 37.320-g20