3GPP TSG-RAN WG3 #114-e	R3-215718

1 - 11Nov 2021

Online

Title: 	 (TP for SON BL CR for TS 36.413) SN RACH REPORT

Source: 	ZTE

Agenda item:	8.1

Document for:	Approval

1	Introduction and discussion

This contribution provide text proposal for SON BL CR for TS 36.413, to capture the following proposal in discussion paper.

Proposal 3: Adding RA report into EN-DC SON Configuration Transfer IE contained in eNB CONFIGURATION TRANSFER message and MME CONFIGURATION TRANSFER message to support forward RA report between two eNBs (without X2 interface) via EPC.

2	TP to TS 36.413

*** Start changed ***

9.2.3.39	SON Information Report

This IE contains the configuration information to be transferred to the eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE SON Information Report | M |  |  |  |
| >RLF Report Information |  |  |  |  |
| >>RLF Report Information | M |  | 9.2.3.40 |  |
| >RACH Report information |  |  |  |  |
| >> RACH Report of en-gNB | M |  | 9.2.3.X |  |

*** Next changed ***

9.3.3.X	RACH Report of en-gNB

This IE contains the random access  information of en-gNB to be transferred.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [50]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

*** End of changed ***