3GPP TSG-RAN WG3 #114-e	R3-215719

1 - 11 Nov 2021

Online

Title: 	 (TP for SON BL CR for TS 36.423) SN RACH REPORT

Source: 	ZTE

Agenda item:	8.1

Document for:	Approval

1	Introduction and discussion

This contribution provide text proposal for SON BL CR for TS 36.423, to capture the following proposal in discussion paper.

Proposal 2: For EN-DC, to introduce ACCESS AND MOBILITY INDICATION message in X2AP.

2	TP to TS 36.423

*** Start changed ***

9.1.4	Messages for E-UTRAN-NR Dual Connectivity Procedures

9.1.4.3X	ACCESS AND MOBILITY INDICATION

This message is sent by MeNB to transfer access and mobility related information to en-gNB.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| RACH Report List |  | 0..1 |  |  | YES | ignore |
| >RACH Report List Item |  | 1 .. <maxnoofRACHReports> |  |  | EACH | ignore |
| >>RACH Report Container | O |  | OCTET STRING | RACH-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [31]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofRACHReports | Maximum no. of RACH Reports, the maximum value is 64. |

*** End of changed ***