3GPP TSG-RAN WG3 #114-e							R3-215722

1-11 November 2021

Online

Source:               ZTE

Title:                    Consideration on User Plane Integrity Protection for eUTRA connected to EPC

Agenda item:      8.1

Document for:    Discussion and Decision

Introduction

LS from SA3[1] requests RAN3 to  modify the specifications to ensure that all of MME, eNB, AMF and ng-RAN node copy on the complete UE security capabilities.

In this contribution, we will discuss this issue and give our proposals.

Discussion

Based on the following description in the LS[3] from SA3, it has been specified in TS 33.401 that MME should leliver the UE EPS security capabilities to eNB when AS security context is established in the eNB.

SA3 asks RAN3 to modify their specifications to ensure that all of MME, eNB, AMF and ng-RAN node copy on the complete UE security capabilities. This is to prevent the network not selecting what would be the preferred security algorithm if the full UE security capabilities were available at the eNB or ng-RAN node. Such a change is justified by the following stage text (from clause 7.2.4.2.1 of TS 33.401). “When AS security context is established in the eNB, the MME shall send the UE EPS security capabilities to the eNB.”

The UE EPS security capabilities includes EEA(EPS Encryption Algorithm), EIA(EPS Integrity Algorithm) and User plane integrity protection support capability, but only the EEA and EIA are independent of the UP IP work. Considering that the EEA and EIA can already been sent to eNB by UE Security Capabilities IE in the message of INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST, HANDOVER REQUEST, and UE Security Capabilities IE is “M” type in PATH SWITCH REQUEST message[2]. It means that eNB can always obtain the EEA and EIA capability when AS security context is established in the eNB.

Observation 1: Among UE EPS security capabilities, the EEA and EIA are independent of the UP IP work, and eNB can already obtain the EEA and EIA when AS security context is established.

Since the UE Security Capabilities IE including EEA and EIA is mandatory present in HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE message of X2AP and XnAP, the source eNB shall always transfer it to the target eNB[3].

Observation 2: The UE Security Capabilities IE is mandatory present in HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE message of X2AP and XnAP.

The only concern is that it is not clear whether the UE Security Capabilities IE in X2AP/XnAP is copy-paste from the S1AP/NGAP signalling without modification and whether the UE Security Capabilities IE in S1AP/NGAP signalling is copy-paste from the NAS signalling[4]. Considering that whether the UE Security Capabilities IE in S1AP/NGAP signalling is copy-paste from the NAS signalling is CN behaviour and should be specified in SA2 specification, RAN3 should only clarify that the UE Security Capabilities IE in X2AP/XnAP is copy-paste from the S1AP/NGAP signalling without modification, which can be described in stage 2(e.g. TS 36.300 and TS 38.300) or stage 3 specification(e.g. TS 36.423 and TS 38.423). For simplicity, we slightly prefer to clarify it in stage 2 specification. Nevertheless, we provide the draft CR for both stage 2 specification and stage 3 specification for discussion and selection.

Proposal 1: RAN3 clarifies that the UE Security Capabilities IE in X2AP/XnAP is copy-paste from the S1AP/NGAP signalling without modification, and whether it is clarified in stage 2 specification or stage 3 specification can be further discussed.

Considering that the User plane integrity protection support capability is only related to UP IP work, and there are concerns to support UP IP for eUTRA connected to EPC, which had been discussed in RAN#92e and RAN#93e, but there is not decision yet[5]. Whether to support UP IP for eUTRA connected to EPC will be further discussed in RAN#94e, and RAN3 can wait for the RAN’s conclusion on whether to deal with the issue related UP IP for eUTRA connected to EPC.
Observation 3: The User plane integrity protection support capability is only related to UP IP work.

Proposal 2: RAN3 waits for the RAN’s conclusion on whether to deal with the issue related UP IP for eUTRA connected to EPC.

Conclusions

In this contribution, we make the following observations and proposals:

Observation 1: Among UE EPS security capabilities, the EEA and EIA are independent of the UP IP work, and eNB can already obtain the EEA and EIA when AS security context is established.

Observation 2: The UE Security Capabilities IE is mandatory present in HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE message of X2AP and XnAP.

Observation 3: The User plane integrity protection support capability is only related to UP IP work.

Proposal 1: RAN3 clarifies that the UE Security Capabilities IE in X2AP/XnAP is copy-paste from the S1AP/NGAP signalling without modification, and whether it is clarified in stage 2 specification or stage 3 specification can be further discussed.

Proposal 2: RAN3 waits for the RAN’s conclusion on whether to deal with the issue related UP IP for eUTRA connected to EPC.

Based on the Proposal 1, we provide the draft CRs[6][7][8][9] for reference.

- References

- 3GPP, R3-214714, LS on User Plane Integrity Protection for eUTRA connected to EPC, November 2021

- 3GPP, TS 36.413, V16.7.0 (2021-10), Evolved Universal Terrestrial Radio Access Network(E-UTRAN);S1 Application Protocol (S1AP)

- 3GPP, TS 36.423, V16.7.0 (2021-10), Evolved Universal Terrestrial Radio Access Network(E-UTRAN);X2 application protocol (X2AP)

- 3GPP, R3-212608_SOD_CB11_UP-IP-EPS_v02, May, 2021

- 3GPP, draft_MeetingReport_RAN_93e_210920_eom, September, 2021

- 3GPP, R3-215723 CR for TS 36.300 on UE Security Capabilities clarification

- 3GPP, R3-215724 CR for TS 38.300 on UE Security Capabilities clarification

- 3GPP, R3-215725 CR for TS 36.423 on UE Security Capabilities clarification

- 3GPP, R3-215726 CR for TS 38.423 on UE Security Capabilities clarification