3GPP TSG-RAN WG3 Meeting #114-e 	R3-215937

- E-Meeting, November 1-12, 2021

Agenda item:	9.3.6.1

Source:	Nokia (moderator)

Title:	CB: # 106_NodeName – Summary of e-mail discussion

Document for:	Decision

# 1	Introduction

CB: # 106_NodeName

- Find the possible BC way to solve the issue?

(Nok - moderator)

Rev in R3-215937

# 2	For the Chair’s Notes

To be updated after discussion

# 3	Discussion

R3-215079 pointed out that the Extended gNB-CU-Node-Name and gNB-DU-Node-Name are incorrectly placed in ASN.1 of F1 SETUP REQUEST and F1 SETUP RESPONSE messages.

Several alternatives were discussed to address this issue during the online discussion

- Solution 1 - Replace incorrect identifiers: Described in R3-215079. Introduces Extended-gNB-DU-Node-Name in F1 SETUP REQUEST and Extended-gNB-CU-Node-Name in F1 SETUP RESPONSE messages and removes the incorrect Information Elements accordingly. This is an ASN.1 Non-Backwards Compatible change.

- Solution 2 – Deprecate the existing incorrect identifier and newly introduce the correct ones: In this alternative, the existing “incorrect” identifiers in ASN.1 are left as they are. However, they are deprecated in the semantics in tabular with a semantics description that “This IE is not used in this version of the specification”. In addition, the correct identifiers are newly introduced in the F1 SETUP REQUEST/RESPONSE messages. This is an ASN.1 Backwards Compatible change.

- Solution 3 - Replace incorrect identifiers and swap protocol IDs: In this alternative, in addition to the changes in Solution 1, it is proposed to swap the protocol IDs for the Extended-gNB-CU-Name and gNB-DU-Name IEs. The motivation for this change is that both IEs definition is identical, and hence the messages would be kept as pointing toward the same protocol ID as in the earlier version of the specification. Proponents of these changes claim this would change the CR to be Backwards Compatible.

  - id-Extended-GNB-DU-Name                             ProtocolIE-ID ::= 426 à 427

  - id-Extended-GNB-CU-Name                             ProtocolIE-ID ::= 427 à 426

Q1: Please provide your view on whether Solution 3 is backwards compatible or non-backwards compatible.

| Company | Comment |
| --- | --- |
| Nokia | In our view, swapping the protocol IDs in Solution 3 is NOT backwards compatible. Even though the messages would point the same protocol ID after the CR, semantically the IE in the F1 SETUP REQUEST and F1 SETUP RESPONSE messages have changed from a CU to DU, and from DU to CU. Thus, regardless of both IEs having the same definition, the IEs have been in practice swapped. For instance, if a given DU has this change implemented, but a CU does had not, the legacy CU would still expect an Extended-gNB-CU name, yet the DU is instead providing an Extended-gNB-DU-Name. Therefore, if a backwards compatible solution is to be pursued, it should be based on Solution 2. That is, deprecate the incorrect IEs and newly introduce the correct ones. |

Q1: Please indicate your preferred solution.

| Company | Comment |
| --- | --- |
| Nokia | Solution 2, if backwards compatibility is to be mandated. However, as indicated online, we think Solution 1 provides a clean fix, even if it introduces NBC ASN.1 change. Therefore, it remains our preferred approach. |

# 4	Conclusions

To be updated after discussion

# 5	References

| R3-215079 | Incorrect Node Name IE in ASN.1 (Nokia, Nokia Shanghai Bell, Verizon Wireless) |
| --- | --- |