3GPP TSG-RAN WG3 #114-e	R3-215813

Online, 1 – 11 Nov 2021

Agenda Item:	8.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CHO with DC opearation (CB #10)

Document for:	Approval

# Introduction

RAN3 has received a response LS from RAN2 concerning the CHO with SCG configuration [1]. RAN2 requests RAN3 to enable the CHO with SCG config as part of Rel.17 development. To agree how this shall be done, an offline discussion has been scheduled at RAN3 #114:

CB: # 10_CHOwithSCGConfig

- Treat it in MR-DC WI or TEI17?

- Identify the scenarios and the corresponding solution

- Any further enhancements on early data forwarding for CPAC needed?

- Reply LS to RAN2?

(Nok - moderator)

Summary of offline disc R3-215813

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion (1st round)

## Framework for the work

In [2] and [6] it is discussed how to organise the requested work within the Rel.17 framework.

- In [2], it is proposed to include the work in the DCCA WI, in the part on CPAC.

- In [6], it is proposed to conduct the work as a dedicated sub-agenda within TEI17.

Question 1-1: Companies are requested to comment on the preferred way to conduct the discussion.

| Company | Preferred option | Comment |
| --- | --- | --- |
| Nokia | 2 | As discussed in [6], both options are perfectly all right. However, from the formal perspective, extending the scope of the WI could require approval from the plenary. Hence, having it within TEI17 may be simpler. |

## CHO indicator

All submitted papers [2,6,10,16] propose to enable an indicator in the Addition to make the SN aware the UE will arrive late.

Furthermore, in [10], it is observed that such indicator is needed in the MN-initiated modification procedure, for the case where CHO is prepared in the same MN.

Question 2-1: Can RAN3 agree to have the CHO indicator added in both, the Addition and the MN-initiated Modification procedures?

| Company | ADD | MOD | Please, comment, if the answer to either is ‘No’. |
| --- | --- | --- | --- |
| Nokia | Yes | Yes |  |

## Resource optimisation at the target SN

In [6], the list of the needed changes, as agreed at #112e, is reminded. One of the problems identified then was possible over-allocation of resources in the SN, if multiple target cells are prepared for the same UE, all or some of which with the same target SN.

Question 3-1: Companies are requested to confirm that the problem of possible over-reservation of resources at the SN, identified at RAN3 #112, is to be addressed.

| Company | Answer | Please, comment, what has changed since RAN3 #112, if the answer is ‘no’. |
| --- | --- | --- |
| Nokia | Yes, it is part of the problem |  |

## Release of the source

In [10] and in [16], the moment when the source MN sends the Release is initiated towards the source SN: only after the Handover Success is received at the source MN.

On the other hand, CHO with DC at the source (no DC at the target) has already been enabled in Rel.16. The question is therefore if keeping DC operation at the target changes anything in the source MN’s behaviour?

Question 4-1: Companies are requested to comment if the Rel.16 behaviour of the source MN (in Rel.16, the source MN could be configured with DC, but it had to be released in the course of CHO) is different than the behaviour proposed at this meeting.

| Company | Comment |
| --- | --- |
| Nokia | Rel.16 behaviour is the same as proposed in [10] and [16]: the source MN releases the source SN only when it receives the HO Success. |

## Data forwarding

In [2] and [10], it is analysed that the existing signalling prepared for CHO with DC at source, possibly with the enhancements discussed in Rel.17 for CPAC, is enough and thus no further work in required specifically for the CHO with DC.

In [16], the solution for data forwarding is presented, but it also seems to be the same as the Rel.16 solution for CHO with DC at the source.

Question 5-1: Companies are requested to comment if any work related to the data forwarding is needed specifically for CHO with DC operation kept (on top of what is enabled in Rel.16 and being worked on in Rel.17 in the scope of CPAC).

| Company | Comment |
| --- | --- |
| Nokia | No such specific work is needed. |

## E1 and F1 impacts

In [10], it is analysed that the existing signalling on E1 and F1 can be reused for the CHO with DC kept at the target.

Question 6-1: Companies are requested to comment if the F1 or E1 requires any work specifically for CHO with DC operation kept (on top of what is enabled in Rel.16 and being worked on in Rel.17 in the scope of CPAC).

| Company | Comment |
| --- | --- |
| Nokia | No such specific work is needed. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-214691, Response LS on CHO with SCG configuration (RAN2)

- R3-214743, Discussion on CHO with SCG configuration (ZTE)

- R3-214744, (TP for CPAC BLCR to TS36.423) Support CHO with SCG configuration (ZTE)

- R3-214745, (TP for CPAC BLCR to TS38.423) Support CHO with SCG configuration (ZTE)

- R3-214746, (TP for CPAC BLCR to TS37.340) Support CHO with SCG configuration (ZTE)

- R3-214797, Continuation of the work on CHO with SCG configuration (Nokia, Nokia Shanghai Bell)

- R3-214798, Enabling CHO with SCG configuration (Nokia, Nokia Shanghai Bell)

- R3-214799, Enabling CHO with SCG configuration (Nokia, Nokia Shanghai Bell)

- R3-214800, Response LS on Conditional Handover with SCG configuration scenarios (Nokia, Nokia Shanghai Bell)

- R3-215024, Discussion on RAN2 LS on Conditional Handover with SCG configuration scenarios (Huawei)

- R3-215025, Support of CHO with SCG configuration (Huawei)

- R3-215026, Support of CHO with SCG configuration (Huawei)

- R3-215027, Support of CHO with SCG configuration (Huawei)

- R3-215028, Support of CHO with SCG configuration (Huawei)

- R3-215085, Support of CHO with SCG configuration (Huawei)

- R3-214922, CHO with SCG configuration (Qualcomm Incorporated)