3GPP TSG-RAN WG3 #114-e	R3-215820

Online, 1-11 Nov 2021

Agenda Item:	8.3.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: #17_BearerPreemptionRateLimit

Document for:	Approval

# Introduction

CB: # 17_BearerPreemptionRateLimit

- More clarification is needed? HW

- Introduce a new cause value “Exceed limitation of bearer pre-emption rate” in S1AP, NGAP, W1AP, F1AP? (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T) or only for S1AP and W1AP? E///

- Introduce E-RAB Pending List and Back off time to avoid this issue? Samsung

- Reply LS to SA6 if agreements achieved

(Nok - moderator)

Summary of offline disc R3-215820

The discussion has two phases:

Phase 1: Identify potentially achievable agreements.

Phase 2: Based on Phase 1 discussion, finalize the CRs/LS

The deadline for Phase 1 is Thursday, Nov 4th, 23:59:59 UTC.

The deadline for Phase 2 is the same as for all email discussions, i.e., Tuesday, Nov 9th, 12:00:00 UTC.

# For the Chairman’s Notes

Propose to capture the following Agreement:

…

# Phase 1 Discussion

[Suggestion from Moderator]: It is understood that MBMS may help for DL (also SA6 knows MBMS 😊). But SA6 issue is for the scenario when E-RAB is used for MC service. Considering last meeting’s minutes “Issue acknowledged and to be continued...”, Moderator suggest focus the discussion on how to address this issue when E-RAB is used for MC services, e.g. via existing RAN3 specification(s), or introducing enhancements, etc.

Contribution ([1][7][9]) proposes the RAN3 specification need to be enhanced.

Contribution ([13]) proposes the existing cause value “Radio resources not available” can be reused, with a reason “as long as pre-emption action is configured, resource status indication should be reported after pre-emption is taken, i.e. pre-emption is attempted but no results.”

When the E-RAB setup request for MC services is failed, the CN node (e.g. MME, MC server) need to differentiate following 2 cases:

- Case 1: there is radio resource used by low priority E-RABs and can be pre-empted. But due to the eNB’s limitation (e.g. number of pre-emption caused by the MC services exceed the eNB’s processing limitation), the eNB cannot process the E-RAB setup request for MC services. (This is the case from SA6 LS)

- Case 2: there is NO radio resource that can be pre-empted for MC services, e.g. all low priority E-RABs’ Pre-emption Vulnerability IE is set to “not pre-emptable”.

Q1: Do you agree that RAN3 specification need to be enhanced? If “No”, please explain which cause value is used in above 2 cases and how CN node can differentiate the 2 cases?

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T | Yes. We think RAN3 specification need to be enhanced to inform the CN (e.g. MME, and further to MC server) the exact failure cause for the E-RAB setup request for the MC services. As described in our contribution ([1]), current RAN3 specification cannot give a clear indication to MME (and MC server) for Case 1. Using the existing cause value, e.g. “Radio resources not available”, for Case 1 will give an incorrect indication to the MME (and further to the MC server) and may prevent the CN node (e.g. MME or MC server) to take appropriate action. |

In case RAN3 specification(s) need to be enhanced, there are 2 solutions:

- Solution 1: introducing a new cause value, as proposed in ([1][7])

Current 3GPP interfaces support propagating the S1-Cause value to multiple CN nodes (e.g. PGW, PCRF, P-CSCF, MC server, etc)

- Solution 2: introducing a new E-RAB pending list in E-RAB SETUP RESPONSE message to indicate the E-RAB will be setup after a pending time, and a new back off time included in the E-RAB list to notify the suggested retry time.

Please Note: the new E-RAB pending list and new back off time are not supported in other interfaces (e.g. S11, S5, Gx, Rx, etc).

Q2: In case RAN3 specification(s) need to be enhanced, which above solution is your preference?

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T | Solution 1 is preferred. Via the new cause value, the CN node (e.g. MME, MC server) can know the actual failure reason and take appropriate action. For Solution 2, as described in our contribution ([1]), Solution 2 is complex and require more changes. For example, SA6 specification describes the MC server takes appropriate action based on the information received from eNB. However, in Solution 2, it is not possible for MC server to know the E-RAB Pending List without changes to all major interfaces, e.g. S11, S5, Gx, Rx, etc. |

In case RAN3 specification(s) need to be enhanced, contribution [1] propose to introduce same change also for NR specifications since the MC services can be provided over NR system. Contribution ([7]) proposes to only consider LTE specifications, with a reason “the evolution of hardware and virtualization platforms for 5G”

Q3: In case RAN3 specification(s) need to be enhanced, which following option is your preference?

- Option 1: LTE specifications only (i.e. TS 36.413 and TS 37.473)

- Option 2: LTE specifications and NR specifications (i.e. TS 36.413 and TS 37.473 for LTE, TS 38.413 and TS 38.473 for NR)

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T | Option 2 is preferred. |

Summary:

Suggest following proposal:

Proposal

# Phase 2 Discussion

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-215338, Discussion on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	discussion

- R3-215339, S1AP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR1824r1, TS 36.413 v16.7.0, Rel-16, Cat. F

- R3-215340, NGAP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR0629r1, TS 38.413 v16.7.0, Rel-16, Cat. F

- R3-215341, W1AP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR0011r1, TS 37.473 v16.6.0, Rel-16, Cat. F

- R3-215342, F1AP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR0786r1, TS 38.473 v16.7.0, Rel-16, Cat. F

- R3-215343, [Draft] Reply LS on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Nokia)	LS out To: SA6 CC: RAN, RAN2

- R3-215400, Bearer Pre-Emption for Mission-Critical Services in LTE (Ericsson LM)	discussion

- R3-215463, [DRAFT] Reply LS on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Ericsson LM)	LS out To: SA6, SA2 CC: RAN2, RAN

- R3-215549, Discussion on Pre-emption limit issue in MC systems (Samsung)	discussion

- R3-215550, (CR for TS 36.413) Support to address pre-emption limit issue in MC systems (Samsung)	CR1847r, TS 36.413 v16.7.0, Rel-16, Cat. F

- R3-215551, (CR for TS 38.413) Support to address pre-emption limit issue in MC systems (Samsung)	CR0704r, TS 38.413 v16.7.0, Rel-16, Cat. F

- R3-215552, ReplyLS_Pre-emption limit issue in MC systems (Samsung)	LS out To: SA6 CC: RAN2, SA2

- R3-215655, Further discussions on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Huawei)	discussion

- R3-215656, Draft reply LS on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Huawei)	LS out To: SA6 CC: RAN2, RAN