3GPP TSG-RAN WG3 #114-e	draft R3-215820

Online, 1-11 Nov 2021

Agenda Item:	8.3.2

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on CB: #17_BearerPreemptionRateLimit

Document for:	Approval

# Introduction

CB: # 17_BearerPreemptionRateLimit

- More clarification is needed? HW

- Introduce a new cause value “Exceed limitation of bearer pre-emption rate” in S1AP, NGAP, W1AP, F1AP? (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T) or only for S1AP and W1AP? E///

- Introduce E-RAB Pending List and Back off time to avoid this issue? Samsung

- Reply LS to SA6 if agreements achieved

(Nok - moderator)

Summary of offline disc R3-215820

The discussion has two phases:

Phase 1: Identify potentially achievable agreements.

Phase 2: Based on Phase 1 discussion, finalize the CRs/LS

The deadline for Phase 1 is Thursday, Nov 4th, 23:59:59 UTC.

The deadline for Phase 2 is the same as for all email discussions, i.e., Tuesday, Nov 9th, 12:00:00 UTC.

# For the Chairman’s Notes

Propose to capture the following Agreement:

…

# Phase 1 Discussion

[Suggestion from Moderator]: It is understood that MBMS may help for DL (also SA6 knows MBMS 😊). But SA6 issue is for the scenario when E-RAB is used for MC service. Considering last meeting’s minutes “Issue acknowledged and to be continued...”, Moderator suggest focus the discussion on how to address this issue when E-RAB is used for MC services, e.g. via existing RAN3 specification(s), or introducing enhancements, etc.

Contribution ([1][7][9]) proposes the RAN3 specification need to be enhanced.

Contribution ([13]) proposes the existing cause value “Radio resources not available” can be reused, with a reason “as long as pre-emption action is configured, resource status indication should be reported after pre-emption is taken, i.e. pre-emption is attempted but no results.”

When the E-RAB setup request for MC services is failed, the CN node (e.g. MME, MC server) need to differentiate following 2 cases:

- Case 1: there is radio resource used by low priority E-RABs and can be pre-empted. But due to the eNB’s limitation (e.g. number of pre-emption caused by the MC services exceed the eNB’s processing limitation), the eNB cannot process the E-RAB setup request for MC services. (This is the case from SA6 LS)

- Case 2: there is NO radio resource that can be pre-empted for MC services, e.g. all low priority E-RABs’ Pre-emption Vulnerability IE is set to “not pre-emptable”.

Q1: Do you agree that RAN3 specification need to be enhanced? If “No”, please explain which cause value is used in above 2 cases and how CN node can differentiate the 2 cases?

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T | Yes. We think RAN3 specification need to be enhanced to inform the CN (e.g. MME, and further to MC server) the exact failure cause for the E-RAB setup request for the MC services. As described in our contribution ([1]), current RAN3 specification cannot give a clear indication to MME (and MC server) for Case 1. Using the existing cause value, e.g. “Radio resources not available”, for Case 1 will give an incorrect indication to the MME (and further to the MC server) and may prevent the CN node (e.g. MME or MC server) to take appropriate action. |
| Samsung | Yes. But we don’t think a new cause value can solve the issue, in our understanding, if the E-RAB request for the MC services is failed, the server will retry anyway no matter what the cause value is, as this kind of service is very important and critical. If companies have doubt on this understanding, we suggest RAN3 send LS to SA2 and SA6 to check their views, if the request is failed, whether the retry is based on the cause value or not. The most important thing that matters in this case is to ensure the E-RAB to be setup successfully and timely. If just blindly retry, it may be failed again, and the E-RAB setup latency will be increased due to more signaling interactions, we think failed request and long latency setup time are not MC services want, so the enhancements are needed. |
| Ericsson | No, but we are available to accept majority view, which seems to follow the approach of introducing a new cause value. With the understanding that this new cause value is only a part of the whole implementation, which is assumed to be outside of standardization scope. As explained in our papers, we believe our approach should be pragmatic and minimalistic. |

In case RAN3 specification(s) need to be enhanced, there are 2 solutions:

- Solution 1: introducing a new cause value, as proposed in ([1][7])

Current 3GPP interfaces support propagating the S1-Cause value to multiple CN nodes (e.g. PGW, PCRF, P-CSCF, MC server, etc)

- Solution 2: introducing a new E-RAB pending list in E-RAB SETUP RESPONSE message to indicate the E-RAB will be setup after a pending time, and a new back off time included in the E-RAB list to notify the suggested retry time.

Please Note: the new E-RAB pending list and new back off time are not supported in other interfaces (e.g. S11, S5, Gx, Rx, etc).

Q2: In case RAN3 specification(s) need to be enhanced, which above solution is your preference?

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T | Solution 1 is preferred. Via the new cause value, the CN node (e.g. MME, MC server) can know the actual failure reason and take appropriate action. For Solution 2, as described in our contribution ([1]), Solution 2 is complex and require more changes. For example, SA6 specification describes the MC server takes appropriate action based on the information received from eNB. However, in Solution 2, it is not possible for MC server to know the E-RAB Pending List without changes to all major interfaces, e.g. S11, S5, Gx, Rx, etc. |
| Samsung | Solution 2 is preferred. Actually, solution 2 can be divided into 2 sub-options, option 1 is suspending the request in eNB and notify the MME, option 2 is include a backoff time in the failure message if the E-RAB cannot be setup due to pre-emption limit. We think either option 1 or option 2 or both can be used to solve the issue, and they are better than solution 1, which can ensure the E-RAB to be setup successfully and timely. Option 1 may have more impact on CN, but it has best performance to solve the issue, it can make sure the E-RAB to be setup successfully and timely. One company has doubt on option2 by saying that the back off time is too restrictive, but we think that’s actually the upside of this option, which can ensure a successful retry. |
| Ericsson | Solution 1 would be acceptable for us (although we wouldn’t call it a solution, but an “approach”). Anything that complicates the standards (like e.g. Sol. 2) is counterproductive: as explained by Nokia et al., Sol. 2 will require specification impacts all the way to the MC server, so it should be avoided. |

In case RAN3 specification(s) need to be enhanced, contribution [1] propose to introduce same change also for NR specifications since the MC services can be provided over NR system. Contribution ([7]) proposes to only consider LTE specifications, with a reason “the evolution of hardware and virtualization platforms for 5G”

Q3: In case RAN3 specification(s) need to be enhanced, which following option is your preference?

- Option 1: LTE specifications only (i.e. TS 36.413 and TS 37.473)

- Option 2: LTE specifications and NR specifications (i.e. TS 36.413 and TS 37.473 for LTE, TS 38.413 and TS 38.473 for NR)

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T | Option 2 is preferred. |
| Samsung | Option 2 is preferred. |
| Ericsson | Option 1 is preferred. As explained in our contribution, given the evolution of hardware and virtualization platforms this is not expected to be a problem in NR (and let’s also keep in mind that even in E-UTRAN this seems to affect a very specific implementation). But we would also like to point out that strictly speaking, only part of Option 1, i.e. the S1AP cause value, is appropriate. Adding a cause in W1AP is functionally wrong, as W1AP is only applicable in the context of NG-RAN, which does not seem to be the case here. |

Summary:

Suggest following proposal:

Proposal

# Phase 2 Discussion

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-215338, Discussion on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	discussion

- R3-215339, S1AP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR1824r1, TS 36.413 v16.7.0, Rel-16, Cat. F

- R3-215340, NGAP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR0629r1, TS 38.413 v16.7.0, Rel-16, Cat. F

- R3-215341, W1AP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR0011r1, TS 37.473 v16.6.0, Rel-16, Cat. F

- R3-215342, F1AP Cause value for exceeding the bearer pre-emption rate limitation (Nokia, Nokia Shanghai Bell, BT, BDBOS, BMWi, Home Office, Firstnet, UIC, AT&T)	CR0786r1, TS 38.473 v16.7.0, Rel-16, Cat. F

- R3-215343, [Draft] Reply LS on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Nokia)	LS out To: SA6 CC: RAN, RAN2

- R3-215400, Bearer Pre-Emption for Mission-Critical Services in LTE (Ericsson LM)	discussion

- R3-215463, [DRAFT] Reply LS on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Ericsson LM)	LS out To: SA6, SA2 CC: RAN2, RAN

- R3-215549, Discussion on Pre-emption limit issue in MC systems (Samsung)	discussion

- R3-215550, (CR for TS 36.413) Support to address pre-emption limit issue in MC systems (Samsung)	CR1847r, TS 36.413 v16.7.0, Rel-16, Cat. F

- R3-215551, (CR for TS 38.413) Support to address pre-emption limit issue in MC systems (Samsung)	CR0704r, TS 38.413 v16.7.0, Rel-16, Cat. F

- R3-215552, ReplyLS_Pre-emption limit issue in MC systems (Samsung)	LS out To: SA6 CC: RAN2, SA2

- R3-215655, Further discussions on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Huawei)	discussion

- R3-215656, Draft reply LS on Bearer pre-emption rate limit issue for GBR bearer establishment in MC systems (Huawei)	LS out To: SA6 CC: RAN2, RAN