3GPP TSG-RAN WG3 Meeting #114-e                                                      	R3-215807

E-meeting, 01-11 Nov 2021

Agenda Item:	8.1

Source:	CATT (moderator)

Title:	SoD for CB: # 4_PositioningDLAOD

Document for:	Approval

# Introduction

This is the SoD of the following CB:

CB: # 4_PositioningDLAOD

- The necessary information can be assumed to be configured into the LMF through OAM, consistently with the RAN? E///

- Prefer Option 2.2 mentioned in LS from RAN1? CATT

- Wait for RAN1 to make decisions on the two options and other FFSs, before RAN3 to discuss how to support in RAN3 specifications? HW

(CATT - moderator)

[NWM] Summary of offline disc R3-215807

Draft LS to RAN1 in R3-216043

# For the Chairman’s Note

Majority of companies (4/5) agree to reply the LS to RAN1. The draft LS is to reflect the majorities’ view in RAN3.

Proposed Way Forward:

Further check online whether the draft reply LS in R3-216043 is agreeable or not.

# Discussion (1st round)

An LS from RAN1 on beam/antenna information for DL AOD has been received [1]. The LS asks signaling/procedures on how the LMF receives this information from the gNBs.

According to the company’s contributions, one said that “antenna directivity characteristics is part of the node configuration, then, any configuration involving TRP antenna directivity will be performed via OAM”[3].

However, the configuration information for TRP already exists in the existing NRPPa specification, e.g, the spatial direction information of the DL PRS resources for the TRP. Also, RAN1 had agreed to list two options about providing antenna/beam information and just need a down-selection.

Please feedback your views below.

Feedback Form 1: Do you agree that the Beam/Antenna Information for DL AoD in DL positioning can be signalled to LMF from gNB via NRPPa or configured by OAM?

1 – Ericsson LM As already explained in our paper, the beam/antenna information is not like all other TRP-related param- eters: unlike such parameters, it cannot be changed on the fly without profoundly affecting the coverage area. Change the beam pattern ”on the fly” and you risk losing active UE connections. For this reason, we believe adding this information in signaling is not justified: any such changes will be done in a supervised manner to avoid service interruptions. We strongly disagree that ”RAN1 only needs a down-selection”: we are always allowed to disagree with another WG’s decision, especially when it concerns something in our scope. In particular, it seems RAN1 may have considered the OAM option, but somehow they didn’t capture it in the LS.

2 – Huawei Tech.(UK) Co.. Ltd Yes, we do it is a requirement from RAN1.

3 – Qualcomm Technologies Int Yes - we prefer an open standard and signalling via NRPPa.

4 – ZTE Corporation Agree

5 – Nokia NRPPa should be enhanced to support signalling of the beam/antenna information from gNB to LMF, as agreed by RAN1.

6 – CATT Option 2.1 RAN1 has discussed the two options, and most companies support Option 2.1. Since signaling is related to RAN3, opinions of RAN3 are required. RAN3 may provide some analysis results at least from the perspective of signaling design.

7 – CATT Beam/Antenna Information for DL AoD in DL positioning should be signaled to LMF from gNB via NRPPa.

Moderator summary:

6 companies participated in the discussion, only 1 company assumed that the beam/antenna information should be configured by OAM.

To follow the views of majority, moderator proposes for RAN3 to agree with signaling of the beam/antenna information from gNB to LMF via NRPPa.

Proposal 1: RAN3 to support signaling of the beam/antenna information from gNB to LMF via NRPPa.

In the LS [1], two options are provided on how to provide the beam/antenna information from the gNB the LMF, as below:

Option 2.1: The gNB reports quantized version of the relative Power/Angle response per PRS resource per TRP

−

The relative power is defined with respect to the peak power of that resource

○

FFS: How many relative power levels can be included (e.g., single -3 dB power-levels, multiple power-levels, etc).

Option 2.2: The gNB reports quantized version of the relative Power between PRS resources per angle per TRP.

−

The relative power is defined with respect to the peak power in each angle

- For each angle, at least two PRS resources are reported.

On how to signal this over NRPPa (including e.g. the number of quantized power levels, granularity of the angle axis, etc.), the consideration of RAN3 is required for this issue.

Based on the existing NRPPa specification, it has already had the definition of PRS Resource IE[Sec. 9.2.44 of TS 38.455], if Option 2.1 is used, the relative Power/Angle response per PRS resource can be directly

introduced into the table. But if Option 2.2 is used, it is necessary to define a new Angle IE, and then

introduce the relative Power between different PRS resource ids into the new defined Angle IE. it seems to have more standard efforts. Therefore, from the information element design point of view, Option 2.1 seems simpler and more intuitive.

On the other hand, from looking at the RAN1 discussion [2], it seems that Option 2.1 has advantage in terms of signaling overhead, and the option has the majority support in RAN1.

Please feedback your views below.

Feedback Form 2: What is your preferred option from a RAN3 point of view, Option 2.1 or Option 2.2? Why ?

1 – Ericsson LM Neither option is justified, as explained in the previous comment. Furthermore, the fact that an option has majority support in RAN1 is totally irrelevant, considering that NRPPa signaling details are out of their scope! If RAN3 were to converge on a 3rd option not considered by RAN1 that would be totally acceptable, precisely for that reason.

2 – Huawei Tech.(UK) Co.. Ltd The both Options are possible with different impacts for RAN3. We have preference to wait RAN1 progress, there is from our understanding still some discussion on-going.

3 – Qualcomm Technologies Int Of course, both options can be supported from RAN3 point of view. The decision should be made in RAN1.

4 – ZTE Corporation Wait for RAN1 further progress, and then discuss the impacts in RAN3.

5 – Nokia RAN3 should wait for further RAN1 progress, since they are still discussing the two options.

6 – CATT Option 2.1 RAN1 has discussed the two options, and most companies support Option 2.1. Since signaling is related to RAN3, opinions of RAN3 are required. RAN3 may provide some analysis results at least from the perspective of signaling design.

Moderator summary:

6 companies participated in the discussion, 4 companies assumed that it is pending RAN1. 1 company

assumed that 3rd option should be considered. Another company assumed that RAN3 may provide some analysis results of NRPPa signaling.

To follow the views of majority, moderator proposes for RAN3 to wait for RAN1 further progress.

Proposal 2: RAN3 to wait for RAN1 further progress for the choice of option 2.1 and 2.2.

# Discussion (2nd round)

Based on the discssion of the first round, moderator would propose to reply the LS to RAN1, reflecting our agreements/considerations as been proposed in Proposal 1 and Proposal 2 above.

The draft LS is in the inbox, please provide your views below.

Feedback Form 3: Do you agree to reply the LS to RAN1 indicating our agreement made in RAN3?

1 – Ericsson LM We should of course communicate the status of our discussion to RAN1. However, we believe the proposed agreement (”wait for further progress in RAN1”) is the absolute worst option: in this way we will be letting RAN1 do signaling design for NRPPa. We strongly oppose such way forward. Also as a way to compromise, we propose to capture something like ”the majority of companies in RAN3 believe it is beneficial to provide beam/antenna information from gNB to LMF via NRPPa, with the understanding that OAM configuration is always possible. From a pure signaling point of view, Option 2.1 is more efficient.”

2 – Ericsson LM Furthermore, we should also capture in the LS that, unlike other TRP configuration information, beam/an- tenna information is not expected to be changed on the fly.

3 – Nokia A reply LS is fine as long as RAN3 can reach consensus on content that is ”clear” and ”useful” (i.e. avoid confusing RAN1 with details of RAN3 discussion that is not relevant to their decision making). Our un- derstanding is that RAN1 will decide between option 2.1 and option 2.2 at their next meeting, so we could

indicate RAN3 preference for option 2.1. So a short Reply LS that simply states: ”RAN3 has analysed the signaling impact of the two options, and concluded that option 2.1 is preferable from a signaling perspec- tive.”

4 – Ericsson LM The whole point is that RAN1 should not decide on those options by themselves. We believe the LS revision we propose captures the facts: 1) that a majority of companies in RAN3 believe it is beneficial, 2) that OAM is always possible, and that 3) this is not expected to change on the fly. This is our part of the compromise (as we still believe this information is not needed at all). Please check the draft.

5 – Qualcomm Technologies Int We fully support Nokia. Saying that OAM is possible is somewhat pointless in many contexts, and in this context, actually gives an incorrect Impression of where RAN3 stands. The current LS spends most of the text on caveats rather than expressing a RAN3 comment on the options.

6 – Ericsson LM The main message is about the acceptable option, and it’s currently there. In addition RAN1 needs to be reminded about two very important principles that so far were not disputed in RAN3 (and were overlooked in RAN1). We still believe that this discussion (and a few others) in RAN1 on positioning were grossly misguided, so we think the current text should be regarded as an acceptable compromise.

7 – Huawei Tech.(UK) Co.. Ltd We do not support the LS. The main feedback from the companies is that we should wait RAN1 decision. Unless I miss something, we did not have any agreement on solution. The current LS just list the solutions. The OAM solution if any could be bring by Ericsson in RAN1. All thoughts for beam power, angles etc... OAM should be able to provide a set of configurations ....

8 – CATT Share the view with Nokia and Qualcom. We just need to simply reply the LS to indicate our views on the solutions, just like the texts in Nokia’s version ”RAN3 has analysed the signalling impact of option 2.1 and 2.2, and concluded that option 2.1 is preferable from a RAN3 signalling perspective although both are feasible.”

Moderator’s summary:

- Companies participated in the 2nd round discussion. 4/5 support to reply the LS to RAN1, while 1/5 does not support the LS.

Due to the content of the reply LS, 3 companies prefer only to provide RAN3 analysis of the option 2.1 and 2.2, 1 company further indicate the OAM is feasible and emphasis that beam/antenna configuration is not

expected to be changed on the fly.

Base on current situation, the moderator would propose to follow the majorities, we just simply indicate

RAN1 that option 2.1 is preferable from a signalling perspective. The final decision should be made in RAN1.

# 5	Reference

- R1-2108646, LS on beam/antenna information for DL AOD in NR positioning, Ericsson

- R1- 2110625, FL summary #3 for AI 8.5.3 Accuracy improvements for DL-AoD positioning

- R3-215255, Beam/Antenna Information for DL AoD, Ericsson LM

- R3-215256, [DRAFT] Reply LS on Beam/Antenna Information for DL AoD in NR Positioning, Ericsson LM

- R3-215603, Consideration on the beam/antenna information for DL AOD, CATT

- R3-215604, Reply LS on beam antenna information for DL AOD in NR positioning, CATT

- R3-215378, Discussion on beam/antenna information for DL-AOD in NR positioning, Huawei