3GPP TSG-RAN WG3 #114-e							                                    R3-215974

1-11 Nov 2021

Agenda item:	22.2.2.

Source:			Samsung

Title:				(TP for BL CR for 38.473) Multicast Group Paging procedure in F1

Document for:	Discussions & Approval

# 1	Introduction

The discussion part is showed in R3-215508 with the below proposals.

Proposal 1: 	It is proposed RAN3 to agree UE Identity Index value to be used as UE identify for paging in NG group paging message for multicast session. The NG group paging message contains a list of UE Identity Index value, a common DRX and TMGI.

Proposal 2: 	It is proposed RAN3 to agree RAN group paging message includes a list of UE Identity Index value.a common DRX and TMGI.

Proposal 3: 	It is proposed RAN3 to agree the F1 group paging message is applied to RRC inactive mode and RRC idle mode UEs by including a list of UE Identity Index value, a common DRX and TMGI.

In this document, text proposal for F1 TS38.473 is proposed in section 2 against the BL CR for 38.473

# 2	Text Proposal

/*--------------------------------Start of the First Change----------------------------*/

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| DU-CU Radio Information Transfer | DU-CU RADIO INFORMATION TRANSFER |
| CU-DU Radio Information Transfer | CU-DU RADIO INFORMATION TRANSFER |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| Reference Time Information Reporting Control | REFERENCE TIME INFORMATION REPORTING CONTROL |
| Reference Time Information Report | REFERENCE TIME INFORMATION REPORT |
| Access Success | ACCESS SUCCESS |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Positioning Assistance Information Control | POSITIONING ASSISTANCE INFORMATION CONTROL |
| Positioning Assistance Information Feedback | POSITIONING ASSISTANCE INFORMATION FEEDBACK |
| Positioning Measurement Report | POSITIONING MEASUREMENT REPORT |
| Positioning Measurement Abort | POSITIONING MEASUREMENT ABORT |
| Positioning Measurement Failure Indication | POSITIONING MEASUREMENT FAILURE INDICATION |
| Positioning Measurement Update | POSITIONING MEASUREMENT UPDATE |
| Positioning Deactivation | POSITIONING DEACTIVATION |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Positioning Information Update | POSITIONING INFORMATION UPDATE |
| Multicast Group Paging | MULTICAST GROUP PAGING |

/*--------------------------------Omit unchanged part---------------------------*/

## 8.x	NR MBS Procedures

- 8.x.y	Multicast Group Paging

- 8.x.y.1	General

The purpose of the Multicast Group Paging procedure is used to provide the paging information to enable the gNB-DU to multicast group page UEs which have joined an MBS Session about its activation. The procedure uses non-UE associated signalling.

Editor’s Note:	 procedure text is FFS

- 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: Multicast Group Paging

The gNB-CU initiates the Multicast Group Paging procedure by sending the MULTICAST GROUP PAGING message to the gNB-DU.

At the reception of the MULTICAST GROUP PAGING message, the he gNB-DU shall perform paging of the MBS Session identified by the MBS Session ID IE .

If the  UE Paging DRX IE is included in the MULTICAST GROUP PAGING message gNB-DU may (may?) use it to determine appropriate POs  according to TS 38.304 [24].

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

Editor’s Note:	 parameterisation of the MULTICAST GROUP PAGING message is FFS in general

- 8.x.y.3	Abnormal Conditions

Void.

/*--------------------------------Omit unchanged part---------------------------*/

#### 9.2.13.x	MULTICAST GROUP PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to multicast group page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| MBS Session ID | M |  | FFS |  | YES | reject |
| UE Identity List for Paging |  | 0..1 |  |  | YES | reject |
| >UE Identity for Paging Item |  | 1..<maxnoofUEIDforPaging> |  |  | - |  |
| >> UE Identity Index value | M |  | 9.3.1.39 |  | - |  |
| >>Paging DRX (FFS) | M |  | 9.3.1.40 |  | YES | ignore |
| >>UE Specific Paging Area |  |  |  |  |  |  |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 0..1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEIDforPaging | Maximum no. of UE ID for paging. Value is FFS. |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

/*--------------------------------End of the Changes-----------------------------*/