3GPP TSG-RAN WG3 #114-e							 R3-215812

1-11 Nov 2021

Online

Agenda Item:	8.1

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # 9_UEPowerSaving

Document for:	Approval

# Introduction

CB: # 9_UEPowerSaving

- Introduce UE Paging Subgroup ID over NG, Xn, F1?

- Whether the grouping assistance information from RAN to CN is needed?

- Which node decides the number of supported subgroups? OAM or RAN node/DU, or the subgrouping is limited only to the last visited cell ID? Waiting for progress in other groups?

- Reply LS to RAN2 if needed

(ZTE - moderator)

Summary of offline disc R3-215812

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion- Second round [if needed]

<TBD>

# Discussion-First round

## General

RAN2 has sent an LS R3-214684 [1] to RAN3.

- Overall description: RAN2 discussed UE paging subgrouping as part of the Rel-17 work on UE power saving enhancement (see RP-200938), in order to reduce power consumption in the UE due to false paging alarms (i.e. when the UE receives a paging message on PDSCH on its paging occasion, which is not intended for that UE). This is done by further dividing the UEs within a paging occasion into multiple subgroups. A UE will decode the Paging message in its Paging occasion only if it is received an indication that its subgroup is being paged. Two subgrouping approaches have been agreed, namely CN-assigned subgrouping and UEID-based subgrouping. No CN impact has been identified for UE-ID based subgrouping. The following provides the detailed agreements below for the 2 approaches: CN-assigned subgrouping - CN assigns subgroup ID. - When AMF has assigned a UE with a Paging subgroup, some NAS signalling should be supported between AMF and UE to convey the related information to the UE. Exact information is FFS. The design and procedure are up to SA2/CT1. - When AMF has assigned a UE with a Paging subgroup, some signalling should be supported between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE. Exact information is FFS. The message(s) and associated design are up to RAN3. - It is FFS when a UE in RRC_INACTIVE has been assigned by CN a Paging subgroup, whether some signalling should be introduced between gNBs to inform each other about the UE’s subgroup for RAN paging. - If RAN2 agrees to support UE assistance information to CN in support of paging subgroup assignment, RAN2 will focus on the paging probability and power profile attributes. UEID-based subgrouping - UEID-based subgroup method requires, in addition to the already available information for legacy UEID-based grouping in PO, the total number of supported UEID-based subgroups by the network. - At least for UEID-based subgroup method the total number, Nsg, of supported subgroups by the network is decided by RAN and broadcasted in System Information. - At least for UEID-based subgroup method the total number, Nsg, of supported subgroups is controlled on a cell basis and can be different in different cells. Coexistence of CN-assigned and UEID-based subgrouping - R2 assumes that all the cells within the registration area supports the same number of CN assigned subgroups, i.e. no remapping of CN assigned group ID to RAN subgroup ID (will revisit only if serious issues are found). - For the purpose of continued discussions, R2 assumes that UE has separate UE capabilities for CN assigned and UEID based subgrouping, the actual decision to be taken later. - RAN capability is known based on broadcast information. FFS with explicit indication or implicitly based configuration. RAN2 also discussed TRS/CSI-RS for RRC Idle and Inactive, and agreed to the following: - The TRS/CSI-RS configuration is provided in a new SIB. - RAN2 assumes that TRS/CSI-RS configurations are broadcasted. Potential addition of dedicated signalling can be discussed in a later meeting based on company contributions. - The legacy SI update procedure is used for changing TRS/CSI-RS configurations. - Postpone the topic about TRS/CSI-RS availability until a later meeting when RAN1 also has progressed. - On demand SI should be possible for the SIB with TRS/CSI-RS information. - Postpone the discussion on segmentation of the new SIB until RAN1 has sent the list of the parameters and a potential structure. - Postpone the discussion on splitting the TRS/CSI-RS information to a common and RS-specific part until RAN1 has sent the list of the parameters and a potential structure. 2. Actions: To RAN3: RAN2 respectfully asks RAN3 to take the above information into account for their future work, and provide further information on the following issue once concluded: - Signalling between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE.

We also received reply LS R3-215795[2] from SA2, as below

- Overall description: SA2 thanks RAN2 for the LS on UE Power Saving (S2-2107038/R2-2108917). SA2 has agreed the attached CRs to keep alignment with the RAN2 agreements about introducing the Paging Subgrouping Assistance from the CN side. Based on the reported UE capabilities, the AMF can determine the assistance information for NR Paging Subgroup according to the NR UE characteristics, i.e. local configuration, subscription information and/or statistical information. If the AMF has determined assistance information for Paging Subgroup, the AMF shall deliver the information to the UE during Registration and the NG-RAN in the NGAP Paging Message or RRC Inactive Assistance Information for support of paging a NR UE in CM_IDLE or CM_CONNECTED with RRC Inactive state. 2. Actions: To RAN2, RAN3 and CT1 ACTION: 3GPP SA2 kindly asks RAN2/RAN3/CT1 to take the above information into account and provide feedback if any.

In this email discussion, RAN3 shall discuss the specification impacts to support the UE power saving based on the LS above.

In [4], it is proposed to makes working assumption that the CN provides subgroup ID as the exact subgroup information.

Question 1: Companies are invited to provide views on whether to agree the working assumption that the CN provides subgroup ID as the exact subgroup information.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | The format of subgroup ID is FFS ,and pending to other WGs |

## The potential impact on NGAP/F1AP/XNAP

[3][4][9] all propose to include UE paging subgroup information into the NG Paging message for CN paging.

Question 2-1: Do companies agree to include UE paging subgroup information into the NG Paging message for CN paging?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Introduce subgroup ID IE into NG paging message |

For RAN paging, [3][4] propose to include the UE subgroup information in the NGAP Core Network Assistance Information for RRC_INACTIVE, while [9] proposes to introduce the UE subgroup information into INITIAL CONTEXT SETUP message over NGAP.

Question 2-2: Companies are invited to provide their views on whether to introduce UE subgroup information into NGAP Core Network Assistance Information or INITIAL CONTEXT SETUP message?

| Companies | Comments |
| --- | --- |
| ZTE | The subgroup ID is same for RRC_IDLE and RRC_INACTIVE for the UE, so we prefer to include such information as a separate IE in INITIAL CONTEXT SETUP . but we are ok to include it in NGAP Core Network Assistance Information IE as part of assistance data. |

[3][4][9] all propose to include UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE.

Question 2-3: Do companies agree to introduce UE paging subgroup information into F1 Paging message for paging a UE in RRC_INACTIVE/RRC_IDLE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | Introduce subgroup ID IE into F1 paging message |

[4][9] deem that XnAP impacts should be considered if the Subgrouping is limited only to the last visited cell ID, and then [4][8] propose to introduce UE paging subgroup information into XnAP Paging message. [3] suggest that RAN3 to wait further progress on last used cell limitation for subgrouping and XnAP impacts for Subgrouping can be avoided if the Subgrouping is limited only to the last visited cell ID.

Question 2-4: Companies are invited to provide their views on whether any XnAP impacts should be considered for subgrouping?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | FFS | In RAN2 LS ,“It is FFS when a UE in RRC_INACTIVE has been assigned by CN a Paging subgroup, whether some signalling should be introduced between gNBs to inform each other about the UE’s subgroup for RAN paging.” so, we can wait further progress of RAN2. |

## Others

Regarding whether the grouping assistance information from RAN to CN is needed, [3] propose to examine the motivation of adding any assistance information from RAN to CN, considering the potential gain to complexity or specification impacts ratio, and [4][9] think that is pending to RAN2’s further progress. Hence, moderator tries to capture the proposal as below:

Whether to adding any assistance information from RAN to CN is pending to RAN2’s further progress.

Question 3-1: Companies are invited to provide their views on whether to agree the proposal above?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | pending to other group progress. |

For UE-ID based subgrouping, it is proposed in [4] that for the disaggregated node, the gNB-DU determines the Nsg, of supported subgroups for UE-ID based subgrouping. And [9] proposes that OAM can configure group number per cell, and no RAN3 impact expect at this stage.

Question 3-2: Companies are invited to provide their views on whether there is any impacts in RAN3 with respect to the Nsg, of supported subgroups for UE-ID based subgrouping?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | there is no RAN3 impact foreseen at this stage, can wait for RAN2’s further progress. |

[4] proposes that the RAN/gNB-DU needs to know the UE capability for UE-ID based subgrouping and he UE capability for UE-ID based subgrouping can be sent from the CN to the RAN. For the disaggregated node, the DU needs to know the UE capability for UE-ID based subgrouping.

Question 3-3: Companies are invited to provide their views on whether there is any impacts in RAN3 with respect to UE capability for UE-ID based subgrouping?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| ZTE |  | FFS at this stage |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-214684 LS on UE Power Saving (RAN2) LS in

- R3-215795 LS Reply on UE Power Saving (SA2) LS in

- R3-215416 Discussion on the scope of the UE Subgrouping for paging in RAN3 based on the RAN2 LS (Ericsson) discussion

- R3-215575 Paging subgrouping for UE power saving (Huawei) discussion

- R3-215576 Paging subgrouping for UE power saving (Huawei) CR

- R3-215577 Paging subgrouping for UE power saving (Huawei) CR

- R3-215578 Paging subgrouping for UE power saving (Huawei) CR

- R3-215624 Discussion on UE paging subgroup (ZTE Corporation) discussion

- R3-215625 CR to TS38.413 for UE paging subgroup (ZTE Corporation) CR

- R3-215626 CR to TS38.423 for UE paging subgroup (ZTE Corporation) CR

- R3-215627 CR to TS38.473 for UE paging subgroup (ZTE Corporation) CR

- R3-215628 [Draft] Reply LS on UE Power Saving (ZTE Corporation) LS out