3GPP TSG-RAN WG3 Meeting #114-e	R3-215889

E-meeting, 01-11 Nov 2021

Agenda Item:	22.2.5

Source:	Lenovo, Motorola Mobility (moderator)

Title:	Summary of offline discussion of CB: # MBS3_others

Document for:	Discussion

# Introduction

CB: # MBS3_others

- Support Broadcast Local MBS and Multicast Local MBS? If yes, how?

- MBS service area information transmission during HO?

- Capture agreements and open issues

(Lenovo - moderator)

Summary of offline disc R3-215889

# For the Chairman’s Notes

//to be added

# Discussion

Issue 1: Local dependent broadcast session

In RAN3#113e, it was agreed that Area Session ID may be included in NGAP MBS session resource related signaling to indicate MBS service area information for local dependent multicast session(s), and it is FFS for local dependent broadcast session. Based on the TS 23.247, for location dependent broadcast service, the content data is different between different MBS service areas, therefore the MBS Session ID is used together with a different Area Session ID for each MBS service area.  UEs in different service areas will receive different MBS content data for the same MBS session, while UEs are only aware of the MBS Session ID. Therefore, the shared NG-U tunnel should be established per service area in order to deliver the different content data for the same MBS session. For broadcast service, the NG-U tunnel can be established during MBS session resource setup procedure. So, in order to provide the service area info and the tunnel information related to the corresponding service area, Area session ID should be included in the NGAP MBS session resource setup to indicate MBS service area information for local dependent broadcast session [1].

Question 1: Do companies agree that for a location dependent broadcast session, Area session ID may be included in the NGAP MBS session resource setup procedure to indicate MBS service area information.

| Company | Yes/No | Comment |
| --- | --- | --- |

Question 2: Do companies agree that for location dependent broadcast session, per service area NG-U tunnel should be established.

| Company | Yes/No | Comment |
| --- | --- | --- |

In [1], it is proposed that for location dependent broadcast MBS service, the list of (area session id + service area + (optional, SMF container) IP multicast address) should be provided together with the MBS Session ID in the MBS Session Setup Request. One example of IE structures is provided as:

| > Location Dependent Info List |  |  |  |  |
| --- | --- | --- | --- | --- |
| >> Location Dependent info Item |  | 1..<maxnoofAreaSession> |  |  |
| >>> Area Session ID | M |  | FFS |  |
| >>> Service Area | M |  | FFS |  |
| >>> MBS Session Resource Setup Request Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Setup Request Transfer IE specified in subclause 9.3.4.x |

Question 3: Do companies agree that for a location dependent broadcast MBS session, the list of (area session id + service area + (optional, SMF container) IP multicast address) should be provided together with the MBS Session ID in the MBS Session Setup Request.

| Company | Yes/No | Comment |
| --- | --- | --- |

Issue 2: Local Multicast session

As discussed in [1], with SA2 and RAN3 agreements, for both local multicast service with location-dependent content and local multicast service available within a limited area, MBS service area related information will be provided from SMF to RAN via the SMF containers in the PDU SESSION RESOURCE SETUP REQUEST or PDU SESSION RESOURCE MODIFICATION REQUEST message. For details of TP of TS 38.413, according to the highlighted word above, the MBS service area related information could be included in PDU Session Resource Setup Request Transfer or PDU Session Resource Modify Request Transfer IE as follow:

#### 9.3.1.X	Service Area info

The Service Area info IE indicates that the Service Area information for local multicast service with location-dependent content or multicast service available within a limited area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Local MBS | M |  |  |  |
| > Area Limited info |  |  |  |  |
| >> MBS Service Area | M |  | FFS. Cell ID list or TAI list |  |
| > Location dependent info |  |  |  |  |
| >> MBS Area Session ID | M |  | FFS |  |
| >> MBS Service Area | M |  | FFS. Cell ID list or TAI list |  |
| >> Additional Service Area info list |  |  |  |  |
| >>> Additional Service Area info Item |  | 1..<maxnoofAreaSession> |  |  |
| >>>> MBS Area Session ID | M |  | FFS |  |
| >>>> MBS Service Area | M |  | FFS. Cell ID list or TAI list |  |

Question 4: Do companies agree that for local multicast sessions, service area related information should be provided from SMF to RAN via the PDU SESSION RESOURCE SETUP REQUEST or PDU SESSION RESOURCE MODIFICATION REQUEST messages.

| Company | Yes/No | Comment |
| --- | --- | --- |

For location dependent multicast service, the contents are different between different MBS service areas. For location-dependent multicast session, an MBS Session ID and an Area Session ID is map to a NG-U tunnel. So, RAN triggered per area session ID per MBS Session ID distribution establishment and release is needed. Thus, in Multicast Distribution related message(s), Area session ID besides MBS session ID should be included.

Question 5: Do companies agree that for a local dependent session, RAN can trigger the per area session ID per MBS session ID NG-U tunnel establishment and release. And one Area Session ID may be included in the Multicast Distribution related message(s) to setup or release the NG-U tunnel.

| Company | Yes/No | Comment |
| --- | --- | --- |

Issue 3: Support of mobility of local multicast

Issue 3-1: Mobility of multicast service available within a limited area

Basically, there are two alternatives of handover for multicast service available within a limited area:

-	Alternative 1: The Source gNB provides the MBS service area information (e.g. cell list or tracking area list) to target gNB in Handover Request message as a part of MBS session related information. or,

-	Alternative 2: the source gNB may deduce that the target not in the MBS service area anymore and not provide MBS session related information at all.

Usually, the admission control usually is made in the target gNB. It would be better to The Source gNB provides the MBS service area information (e.g. cell list or tracking area list) to target gNB in Handover Request message. And the alternative 1 is more aligned with SA2.

Question 6: Do companies agree that to support Xn handover of the multicast service available within a limited area, for each MBS multicast Session, the Service Area Information should be provided to the target gNB in HANDOVER REQUEST message.

| Company | Yes/No | Comment |
| --- | --- | --- |

For the NG based handover procedure, the Handover Request Transfer IE (containing the PDU Session Resource Setup Request Transfer IE) including MBS Session ID and Service Area information should be introduced to NGAP: HANDOVER REQUEST message.

Question 7: Do companies agree that to support NG handover of the multicast service available within a limited area, Service Area information should be provided per MBS Session in the NGAP: PDU Session Resource Setup Request Transfer IE in HANDOVER REQUEST message.

| Company | Yes/No | Comment |
| --- | --- | --- |

Issue 3-2: Mobility of multicast service with location-dependent content

According to the description of SA2, MBS Session ID, Area Session ID(s) and MBS service area(s) should be sent in XnAP: HANDOVER REQUEST message from the source NG-RAN to the target NG-RAN during Handover preparation phase. Upon the target NG-RAN received the message, the target NG-RAN can determine whether to establish the resources for multicast distribution or simply whether to perform data forwarding.

Question 8: Do companies agree that to support Xn handover of the multicast session with the location-dependent content, MBS Session Context Setup Request List IE including MBS Session ID and a list of (Area session ID+ Service Area information) should be included in the XnAP: HANDOVER REQUEST message.

| Company | Yes/No | Comment |
| --- | --- | --- |

For NG handover procedure, the Handover Request Transfer IE (containing the PDU Session Resource Setup Request Transfer IE) including MBS Session ID, Area session ID(s) and Service Area information(s) should be included in the NGAP: HANDOVER REQUEST message.

Question 9: Do companies agree that to support NG handover of the multicast session with the location-dependent content, the list of (Area Session ID + Service Area Information) should be included in the NGAP: PDU Session Resource Setup Request Transfer IE in the HANDOVER REQUEST message.

| Company | Yes/No | Comment |
| --- | --- | --- |

# References

- R3-215144 (TPs to TS 38.413 TS 38.423 BL CRs) Support of Local MBS (Huawei, CBN, China Unicom)

- R3-215275 Remaining Issues on MBS Service Area Management (Lenovo, Motorola Mobility)