3GPP TSG-RAN WG3 Meeting #114-e	R3-215892

E-meeting, 01-11 Nov 2021

Agenda Item:	22.4

Source:	CATT (moderator)

Title:	Summary of discussion on Broadcast Service

Document for:	Approval

# Introduction

CB: # MBS6_BroadcastService

- Whether to exchange following info between gNB via Xn: SAI, MBS service group ID, list of ongoing TMGIs?

- Provide the MBS SAIs belonging to a TAI supported by the NG-RAN node to AMF5442?

- Whether to reuse LTE MBS frequency layer prioritization in NR based on RAN2 agreements?

- Details on SAI? F1AP and E1AP impact?

- Capture agreements and open issues

(CATT - moderator)

Summary of offline disc R3-215892

# For the Chairman’s Notes

//to be added

# Discussion

## Introduction for SAI

In the last RAN3 meeting, the following two WAs were reached:

WA: For broadcast session, the LTE mechanism on MBS frequency layer prioritization shall be revisited for NR, if MBS frequency layer prioritization is supported by RAN2

WA: If SAI/ID is used to identify an MBS service area, it may be configured by OAM.

According to the progress of RAN2, frequency priorities have been captured in the 38.304 running CR [1], so the first WA should be changed into an agreement.  In addition, combined with LS [2] of RAN2 and WA of RAN3, the concept of SAI should be introduced for service continuity of Broadcast Service, and its mapping relationship with cells can be configured through OAM. Therefore, both WAs should be changed into agreements.

Question 1: Do you agree that the two WAs reached in last RAN3 meeting should be changed into agreements?

| Company | Answer | Comment |
| --- | --- | --- |

In addition, based on the company's contribution [3], it is proposed that size of SAI is at least minimum 2 bytes, but in another company's contribution [4], it is proposed to define MBS SAI with length of at least 3 octets.

Question 2: Companies are invited to provide your view on length definition for MBS SAI?

| Company | Answer | Comment |
| --- | --- | --- |

It is mentioned in the company's contribution [5] that more than one MBS SAI can be supported by a cell. So RAN3 should further discuss the maximum number of supported MBS SAIs.

Question 3: Companies are invited to provide your view on how many MBS SAIs to be supported by a cell?

| Company | Answer | Comment |
| --- | --- | --- |

## Impact of SAI on Xn Interface

According to some company contributions [3][5][6][7], to support the frequency priority mechanism, the serving cell needs to know the SAI information supported by neighbor cell so that the SAI information of the corresponding neighbor frequency can be broadcast in SIB. Similar to the mechanism of LTE eMBMS, the SAI information supported by neighbor cell can be obtained via XnAP. e.g, Xn Setup / NG-RAN node Configuration Update procedure.

Question 4: Do you agree to signal the supported MBS SAIs per served cell via XnAP?  e.g,  Xn Setup / NG-RAN node Configuration Update procedure

| Company | Answer | Comment |
| --- | --- | --- |

In addition, one company believes that for accommodating the scalability and frequent change of service area, it should be allowed that the list of ongoing TMGIs are exchanged across neighbor gNBs over Xn interface [3].

Question 5: Do you agree to signal the list of ongoing TMGIs per served cell via XnAP?  e.g, Xn Setup / NG-RAN node Configuration Update procedure

| Company | Answer | Comment |
| --- | --- | --- |

## Impact of SAI on NG Interface

According to the current SA2 conclusion, for a broadcast session, the information from NG interface only contains MBS session ID and cell list or TA list. But if more than one MBS SAIs can be supported by a cell, how does NG-RAN know the corresponding SAI value for a broadcast session? For example, SAI-1 corresponds to {cell 1, cell 2}, and SAI-2 corresponds to {cell 1,cell 2,cell 4, cell 5}. If the information from NG interface is TMGI X and a cell List (cell 1, cell 2). In this case, does the TMGI X respond to SAI-1 or SAI-2? [8]

A feasible solution, similar to the eMBMS solution, is to allow the 5GC to send a SAI List to the corresponding NG-RAN nodes via the NG interface.

Question 6: Do you agree to signal the supported MBS SAI list per MBS session via NGAP?

| Company | Answer | Comment |
| --- | --- | --- |

Since the SAI information has impact on 5GC, accordingly, an LS should be sent to SA2 to inform the conclusion of SAI information over NG interface.

Question 7: Do you agree to send an LS to SA2 to inform the conclusion of SAI information over NG interface？

| Company | Answer | Comment |
| --- | --- | --- |

In addition, according to the company contributions [3][6], it is mentioned whether the cell list over the NG interface contains the all cells within a MBS service area, or only the corresponding cells of the serving NG-RAN and  neighbour NG-RAN？

Question 8: Companies are invited to provide your view on which cells to be included in the MBS Service Area (i.e. Cell lists and TAI lists) over the NG interface？

| Company | Answer | Comment |
| --- | --- | --- |

## Impact of SAI on F1/E1 Interface

Although the relationship between SAI and cell can be configured through OAM, but in CU-DU split case, such configuration is mainly implemented in gNB-DU, while gNB-CU is responsible for SIB message generation, so the corresponding mapping relationship should be notified to the gNB-CU, So it is proposed that gNB-DU should communicate the supported MBS SAIs per served cell to gNB-CU [5].

Question 9: Do you agree that gNB-DU should communicate the supported MBS SAIs per served cell to gNB-CU via F1AP?

| Company | Answer | Comment |
| --- | --- | --- |

In addition, impact of the introduction of MBS SAIs for E1AP was mentioned, but it was assumed that such scenario is mainly for the mobility with shared UP resources case. Thus it needs further discussion in RAN3.

Question 10: Do you agree that the impact of the introduction of MBS SAIs for E1AP needs further discussion in RAN3？

| Company | Answer | Comment |
| --- | --- | --- |

## Others

If you have other issues to be discussed, please elaborate:

| Company | Answer | Comment |
| --- | --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R2-2108923  38.304 running CR for NR MBS, CATT

- R3-214683,  LS on the MBS broadcast service continuity and MBS session identification, Huawei

- R3-214794,  (TP for 38.300) Stage 2 for Broadcast Service Continuity, Nokia, Nokia Shanghai Bell

- R3-215194,  Discussion on RAN2 LS on the MBS broadcast service continuity and MBS session identification,  Ericsson

- R3-215209, 	On MBS SAI handling on NG-RAN internal interfaces, Ericsson

- R3-215150, 	 (TP to TS 38.300 BL CR) Broadcast service continuity, Huawei, CBN, China Unicom, China Telecom

- R3-214751, 	 (TP for 38.300 for Introduction of NR MBS) MBS broadcast service continuity, ZTE

- R3-215045, 	 MBS reception of Idle and In-active UEs, CATT