3GPP TSG-RAN WG3 #114-e							R3-215893

1-11 Nov 2021

Online

Agenda Item:	24.2

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on CB: # SDT1_RACHbased

Document for:	Approval

# Introduction

CB: # SDT1_RACHbased

- Whether to change previous WAs to agreements?

- The details of SDT assistance information?

- How to transfer SRB via SDT?

- Part/full of UE context shall be retrieved?

- How to retrieve part/full of UE context?

- Capture agreements and open issues

- Provide CR/TPs if agreeable, split work

(ZTE - moderator)

Summary of offline disc R3-215893

Please provide your views by 6:00 UTC Friday November 5rd so that they may be taken into account during the online session.

# For the Chairman’s Notes

The following is proposed (for Chair Minutes):

R3-215996 as baseline:   SDT BLCR to TS 38.300, ZTE

R3-215997 as baseline:   SDT BLCR to TS 38.401, HW

R3-215998 as baseline:   SDT BLCR to TS 38.423, Ericsson

R3-215999 as baseline:   SDT BLCR to TS 38.473, Samsung

R3-216000 as baseline:   SDT BLCR to TS 38.463, China Telecommunication Corp

Common for RA-SDT

Proposal 1: Change the following WA into agreement:

- WA: For CG based SDT, RAN3 will further discuss impacts and mainly consider split-gNB case.

- WA: Sequence UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported for SDT

- WA: The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements. Details will be discussed later.

- WA: UL data for SDT is buffered at the receiving node in the successful context retrieval procedure. For other cases, the common understanding is that UL data may need to be buffered as well, details are pending.

- WA: The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of assistance information are pending future discussion

Proposal 2: For RA-SDT, the new IE “SDT assistant information” is introduced in RETRIEVE UE CONTEXT REQUEST message, and it shall include at least IE “SDT indicator”. FFS: whether other IEs are needed.

WA1: For split CU/DU gNB, gNB-DU sends “SDT indicator” IE to gNB-CU in the INITIAL UL RRC MESSAGE TRANSFER message.

Proposal 3: During SDT procedure, SRB PDCP PDU shall be transferred between new gNB and anchor gNB, either via extending the XnAP: RRC TRANSFER message or via defining a new XnAP class-2 procedure. FFS: which XnAP message of them.

Option 1: extending the XnAP: RRC TRANSFER message

Option 2: defining a new XnAP class-2 procedure.

FFS: Extend the RETRIEVE UE CONTEXT REQUEST message, to forward the first SRB PDCP PDU from new gNB to anchor gNB

Specific for RA-SDT with anchor relocation

None for Stage3.

Specific for RA-SDT without anchor relocation

WA2: For RA-SDT without anchor relocation case, SDT related UE context shall be retrieved. FFS: the content of SDT related UE context

Option 1: Table 2 and Table 3 in this SoD can be as baseline/start point

Option 2:  R3-215001 “9.2.1.xx	SDT UE Context Information – Retrieve UE Context”

Option 3:  R3-215280 “9.2.1.xx	UE Context Information – Retrieve UE Context Response for SDT”

WA3: For RA-SDT without anchor relocation case, its stage 2 overall procedure shall enhance either legacy Periodic RNA update without UE context relocation procedure or legacy RNA update with UE context relocation procedure. FFS: which procedure of them.

Option 1: enhance legacy Periodic RNA update without UE context relocation procedure (8 companies)

Option 2: enhance legacy RNA update with UE context relocation procedure (3 companies)

FFS: If agreed to enhance Periodic RNA update without UE context relocation procedure, whether it is acceptable to use the Retrieve UE context failure message to send SDT related UE context information

FFS: If agreed to enhance RNA update with UE context relocation procedure, whether it is acceptable to include the following normative work.

- Enhancement 1:  Retrieve UE context Response message, referred to Table 5

- Enhancement 2:  Xn-U Address Indication message, referred to Table 6

- Enhancement 3:  UE context Release message, referred to Table 7

- Enhancement 4:  Path Switch procedure will not be used.

Proposal 4: The serving gNB forwards PDCP PDUs (RLC SDUs) per GTP-U tunnel to the anchor gNB

FFS: Whether to extend the RRC Resume Cause IE, adding SDT case (e.g., mo-data and mo-signalling).

FFS:  Whether 5GC can provide some assistance information to help gNB deciding to configure one DRB for SDT.

FFS: At least, the following impact on F1AP.

- Impact: RLC bearer configuration query at anchor gNB side. The intention is to help the anchor gNB-CU derive the RLC bearer configuration for SDT bearers before sending the UE to the INACTIVE status, since those configuration will be sent to the serving gNB when SDT session is started.

- Impact: RLC bearer configuration provision to gNB-DU at serving gNB side. The intention is to provide the configurations to serving gNB-DU for SDT data transmission

- Impact: UE inactivity notification to indicate the end of SDT at serving gNB side. The intention is to indicate the activity of SDT bearers so as to determine whether to end the SDT session or not.

FFS: Whether the anchor gNB-DU need to know which bearer is SDT bearer.

To be continued in 2nd phase:

Continue to discuss the content of SDT related UE context

Continue to discuss which legacy procedure shall be enhanced for RA-SDT without anchor relocation case

Option 1: enhance legacy Periodic RNA update without UE context relocation procedure (8 companies)

Option 2: enhance legacy RNA update with UE context relocation procedure (3 companies)

Continue to discuss through which XnAP class 2 procedure to transfer SRB PDCP PDU

Option 1: extending the XnAP: RRC TRANSFER message

Option 2: defining a new XnAP class-2 procedure.

Continue to discuss other FFS left from the 1st round.

Capture agreement into the TPs to be agreed.

# Discussion- Second round [if needed]

<TBD>

# Discussion-First round

## Common for both anchor relocation and no anchor relocation

### Change the WA to agreement

In the previous RAN3 e-meetings, we have achieved the following progress.

- WA: For CG based SDT, RAN3 will further discuss impacts and mainly consider split-gNB case.

- WA: Sequence UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported for SDT

- WA: The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements. Details will be discussed later.

- WA: UL data for SDT is buffered at the receiving node in the successful context retrieval procedure. For other cases, the common understanding is that UL data may need to be buffered as well, details are pending.

- WA: The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of assistance information are pending future discussion

- Keep the scope of without anchor relocation for SDT.

- Among the solutions proposed to support without anchor relocation, forwarding all the MAC PDUs directly to the anchor gNB, is excluded.

- Reply to RAN2 by saying that RAN3 would proceed with the alignment their assumption of RLC handling.

In [1], it proposes to change the above WAs into agreement.

Question1: Do companies agree to change the above WAs into agreements?

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE | Yes | After checking with the latest RAN2 progress, these WAs is stable and can be change to agreement. |
| Intel Corporation | Yes | We are also fine. |
| Lenovo, Motorola Mobility | Yes | We agree to turn the WAs as agreements. |
| CATT | Yes |  |
| Google | Yes |  |
| Nokia | Yes |  |
| Huawei | Yes |  |
| LGE | Yes |  |
| E/// | Yes |  |
| Qualcomm | Yes with comment | Note that #7 is misinterpreted by companies, and in fact some interpretations contradict an open issue recorded last time regarding handling of the first message. Our interpretation is that this does not prohibit either forwarding of RLC PDUs, or even forwarding of MAC PDUs in some cases (not all). So #7 is either off the table or needs clarification. In fact the first message handling is still being discussed by companies in contributions so we consider it still open. |
| Samsung | Yes |  |
| China telecom | Yes |  |
| NEC | Yes |  |

Summary: Majority companies agree with it.

Proposal 1: Change the following WA into agreement:

- WA: For CG based SDT, RAN3 will further discuss impacts and mainly consider split-gNB case.

- WA: Sequence UL/DL transmission following UL SDT without transitioning to RRC_CONNECTED is supported for SDT

- WA: The existing Retrieve UE Context procedure can be reused for both with and without anchor relocation scenarios with possible enhancements. Details will be discussed later.

- WA: UL data for SDT is buffered at the receiving node in the successful context retrieval procedure. For other cases, the common understanding is that UL data may need to be buffered as well, details are pending.

- WA: The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of assistance information are pending future discussion

### The format and content of SDT assistant information

We already have the following WA in past RAN3 meeting.

WA: The last serving gNB, i.e., anchor gNB, will be the decision maker on whether to relocate anchor or not. Assistance information provided by the receiving gNB may help on the decision. Details of assistance information are pending future discussion

As the above WA, the fundamental usage of the assistance information is to indicate SDT or non-SDT, then the gNB shall decide whether to handle SDT access based on this SDT indicator.

Another issue is whether the receiving gNB can generate the one-shot-SDT/multi-shot SDT indicator and send it to anchor gNB. Compared with the SDT indicator (which is essential and necessary), this indicator can be seen as the optimization, because even without the one shot/multi-shot indicator, the anchor can determinate anchor relocation based on network implementation.

Besides SDT indicator and one-shot/multi-shot, [5] additionally suggests to include “Buffered Data size”, because the new gNB may also provide the buffered data size to the last serving gNB in the assistance information, especially in case of multiple packets, it could be used by the last serving gNB to estimate the data volume of the consequent packets.

From moderator’s view, only the SDT indicator is essential and is proposed in majority company’s contributions, the other information (e.g., one-shot/multi-shot, buffered data size, data volume information) is suggested in part of contributions and it can be pending on to RAN2 or can be seen as optimization.

Table 1: SDT Assistant information

- 9.2.3.xxx SDT Assistant Information This IE indicates user data type is SDT. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description. SDT Indicator; M; ENUMERATED (true, ...); Indicates the assistant information for SDT. Other information; O; FFS; Note: Other information (e.g., one-shot/multi-shot, buffered data size, data volume information) is FFS and/or pending on RAN2.

It is the receiving gNB to generate the SDT indicator as the assistance information. For CU-DU split gNB, it is the gNB-DU to generate the SDT indicator. For anchor relocation case, the receiving gNB transfers the SDT indicator to the anchor node via Retrieve UE context request message.

Question 2: Do companies agree that

- In case of RA-SDT, the new IE “SDT assistant information” shall include at least SDT indicator IE, other information is FFS?  Refer to table 1.

- And, for split CU/DU gNB, it is gNB-DU sends it to gNB-CU included in the INITIAL UL RRC MESSAGE TRANSFER message?

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE | Yes for both | We have agreed to introduce a new IE “SDT assistance information”, in this IE, the SDT indicator is essential and shall be firstly agreed, others seems optimization and can be discussed latter. |
| Intel Corporation | 1) Yes; 2) Yes, but | For 1), we agree with the moderator that the majority supports SDT indication. But the semantic of this IE should not say "Indicates the assistance information for SDT". This IE should clearly tell to the last serving gNB whether the resume request is due to SDT or not, as described in R3-215747 [17]. We also agree with the moderator that assistance info in the form of whether SDT will be one shot or multiple, or based on volume of SDT data, are pending to RAN2. But there is one RAN3-specific assistance info that we can discuss without RAN2, which is about "relocation preference" from new gNB. We should not blindly assume that the decision from the last serving gNB of relocation or not is always followed by the new gNB at no problem. A gNB who is capable of supporting SDT may not support "no anchor relocation" scenario which requires quite different behaviors and data handling than the legacy NR INACTIVE. Or, the new gNB may not want to take the anchor role for the UE, if e.g. too many UEs are under its connection management. Such should be provided as the relocation preference of new gNB in the RTRV UE CTXT REQ message that can be taken into account by the last serving gNB's decision. For 2), we are generally fine but it may be useless if RAN2 progresses to include a dedicated resume cause for SDT into RRCResumeRequest message. Maybe it is better to put FFS and monitor RAN2, and if RAN2 doesn’t do such, then we consider adding SDT indicator into INITIAL UL RRC MESSAGE TRANSFER message. |
| Lenovo, Motorola Mobility | Yes for both | For the bullet 2, it would be better to clarify that it is only for RA-SDT. |
| CATT | Yes, but | We agree to introduce SDT Assistance Information, at least an indicator is included. “Other information” is pending to RAN2. |
| Google | Yes | Can agree on first bullet first |
| Nokia | Yes but | For 1) we need the SDT indicator as basics to differentiate between SDT, non-SDT. I agree with Intel’s comments that also as basics we need an indication from serving gNB to anchor gNB about relocation (required, preferred) because if the new gNB may not want to work as “without anchor relocation”, or for example the amount of data requires to turn on the connected mode. Serving gNB receives the BSR and can request this. We could also say that when non-SDT data comes, the serving gNB sends relocation (required) to anchor gNB as an alternative to the “SDT indicator”.. For 2) I assume that “it” means the SDT indicator, in the sense of SDT data, or non-DT data. |
| Huawei | Yes for both | It is ok to have the structure like table 1, and may consider another structure, i.e. SDT indicator outside of the SDT Assistance Information. |
| LGE | Yes for both | In Xn AP and F1AP, we are ok to introduce SDT assistance information IE which at least contains a new indicator. The details are FFS, and can be discussed at next meeting based on RAN2 progress. |
| E/// | Yes with comments | We may not have to preclude the IE name or content, a simple “SDT Indicator “will do, considering no detailed assistance information has been discussed yet. |
| Qualcomm | Yes with comments | For (1) we agree, but would like to point out that a decision between relocation and no relocation is taken at the anchor, and it would help if additional assistance is available. Having said that, this is also RAN2 matter. For (2), can be WA taking Intel comment into account (pending to RAN2 decisions etc) |
| Samsung | Yes for both | “SDT Indicator” is essential to indicate whether this resumption is for SDT or not (this is not the problem of anchor relocation). And this can be transferred within INITIAL UL RRC MESSAGE TRANSFER message. Other assistant information (e.g., buffer status, DRB ID used to transmit small data) would be considered. |
| China Telecom | Yes for both,but | For 1), during w/o anchor relocation procedure, “SDT assistant information” need to be informed to the anchor node. But in principle the “SDT indicator” may use “cause” value contained in corresponding procedure. Therefore, we support to define a “SDT assistant information”, but the detailed definition of assistant information is FFS. |
| NEC | 1) yes 2) FFS | For Xn, an assistance information with SDT indicator, this is OK for us. Whether to do the anchor relocation or not, so we are understanding that it is same as today resume case, up to the last serving gNB to decide, and Other information (e.g., one-shot/multi-shot) may help the last serving gNB to decide. For the indication in F1, may be ok but detail to be FFS. Since majority opinion for now is to have SDT indicator as one of assistance information in XnAP, may be for the progress in this meeting, this can be marked as agreement. |

Summary: All companies agree it in general.

Proposal 2: For RA-SDT, the new IE “SDT assistant information” shall include at least IE “SDT indicator”. FFS: whether other IEs are needed.

WA: For split CU/DU gNB, it is gNB-DU sends IE “SDT indicator” to gNB-CU included in the INITIAL UL RRC MESSAGE TRANSFER message.

### Data forwarding of SRB PDCP PDU

In RAN2#113e, working assumption on support of SRB transmission using SDT was agreed:

Working assumption

- Support configuring of SRB1 and SRB2 for small data transmission for carrying RRC and NAS messages.

- Upon initiating RRC Resume procedure for SDT initiation (i.e. for first SDT transmission), the UE shall also resume SRB2 that is configured for SDT, in addition to SDT DRBs that are configured for SDT

- RAN2 recommends to include SRB2 in WID

In [9], it provides two solutions for data forwarding of SRB PDCP PDUs:

-	Option 1: the SRB PDCP PDUs are forwarded via a GTP-U tunnel. In this option, a SRB specific or SRB type specific GTP-U tunnel should be established.

-	Option 2: the SRB PDCP PDUs are forwarded via signalling. One example is: the first SRB PDCP PDU can be carried in the RETRIEVE UE CONTEXT REQUEST message and the following SRB PDCP PDUs can be carried in the RRC TRANSFER message.

In [10], considering SDT latency, it suggests to support optional the transport of the first message in Xn-c towards the anchor; allow anchor to either process and send to CN or ignore and fallback to normal PDCP PDU forwarding.

In [5], it also suggests to extend XnAP: RRC TRANSFER message to forward the UL/DL PDCP-c PDU for NAS data between last serving gNB and current serving gNB.

The SRB PDCP PDU is PDCP PDU for RRC signalling. NAS PDU is included in the RRC message. Thus, receiving node cannot touch the NAS PDU, piggybacked in RRC message, which is encapsulated in the PDCP PDU.

Question 3: Do companies agree with one or more enhancements to transfer SRB PDCP PDU?

- Enhancement 1: Extend the XnAP: RRC TRANSFER message, to forward the UL/DL SRB PDCP PDU during SDT procedure between new gNB and anchor gNB?

- Enhancement 1A: Define new XnAP class-2 procedure to carry SRB PDCP-C PDU during SDT procedure between new gNB and anchor gNB.

- Enhancement 2: Additionally, Extend the RETRIEVE UE CONTEXT REQUEST message, to forward the first SRB PDCP PDU from new gNB to anchor gNB.

- Other enhancement, if any

| Company | Enhancement 1/2 | Comment |
| --- | --- | --- |
| ZTE | Both | For enhancement 2, it is optimization. We are fine to agree with both now, or agree with enhancement 1 now but agree with enhancement 2 latter. |
| Intel Corporation | Enhancement 1A | First, we think enhancement 2 is not needed. The initial UL SDT data from the UE (sent together with RRCResumeRequest) is not always an SRB1/2 message. Conventionally, SRB PDCP PDUs have been carried by control plane signaling. Enhancement 1 or 1A is in this direction, but we think enhancing RRC TRANSFER is not good because it has been defined and used only for DC. RRC TRANSFER is used not only to carry PDCP-C PDU of split SRB, but also to carry an RRC message (after PDCP decryption) as an OCTET STRING container. But here what we need for SDT is just to carry SRB PDCP-C PDUs between new gNB and old gNB in case of "no anchor relocation" scenario, since PDCP encryption/decryption is all done by the old gNB. It is better to define new dedicated class-2 XnAP procedure for this purpose, without making RRC TRANSFER too complicated. The example is proposed as "SRB Transfer" in R3-215747 [19]. |
| Lenovo, Motorola Mobility | Enhancement 1 or 1A | We are fine with 1 or 1A. For enhancement 2, it is not SRB SDT related issue. |
| CATT | Enhancement 1 or 1A |  |
| Google | Enhancement 1 or 1A |  |
| Nokia | Enhancement 1 or 1A. | TBD between the two. |
| Huawei | 1 or 1A |  |
| LGE | Enhancement 1 or 1A |  |
| E/// | 1 | Is it an enhancement or must-have? If we have to support this, then propose to change to Option 1 and so on. Reusing RRC Transfer is preferred since it is used to transfer PDCP-C PDU as being defined. The scope of procedure can be extended by revising the procedural text. For sure any further discussion on solution will depend on RAN2’s agreement. |
| Qualcomm | 2 and one of 1/1A | In our view, this question is jumping over an open issue, recorded at the last meeting, and discussed by various companies, of how to handle the first message. This is a general question and beyond the issue of whether the first message carries drb or srb. Enhancement 2 makes sense in that way. Of course afterwards option 1/1A are needed, and are not alternatives to 2. In other words this discussion is appropriate to make a decision on 1 vs 1A, but the decision on 2 and generally on handling of the first message should be considered separately. |
| Samsung | Enhancement 1 and 2 | Enhancement 2 can reduce the latency of the first RRC message. |
| China telecom | 1 or 1A |  |
| NEC | 2, 1 | to use the RETRIEVE UE CONTEXT REQUEST message to transfer only one-shot data. For the following SRB data, if any, can Extend the XnAP: RRC TRANSFER, for the introducing new procedure, probably should avoid if possible to avoid too many procedures in our specification, but this can be discussed. |

Summary: Majority companies agree with enhancement 1 or enhancement 1A.

Proposal 3: During SDT procedure, SRB PDCP PDU shall be transferred between new gNB and anchor gNB, either via extending the XnAP: RRC TRANSFER message or via defining a new XnAP class-2 procedure. FFS: which XnAP message of them.

FFS: Extend the RETRIEVE UE CONTEXT REQUEST message, to forward the first SRB PDCP PDU from new gNB to anchor gNB

## RA-SDT without anchor relocation

### Either part of or all of UE context shall be retrieved?

In [17], it proposes that after successful verification, the last serving gNB provides the partial context necessary for processing RLC PDUs as well as UL forwarding TNLs in response to the context retrieval request.  And the CU of new gNB use them to make DU do the job − processing RLC PDUs, forwarding UL SDT PDCP PDUs toward UL TNLs provided from the last serving gNB (as already supported in the current NG-RAN architecture), and creating DL TNL if requested (to be provided to the last serving gNB via Xn-U ADDRESS INDICATION).

RLC bearer configuration transfer

In order to support the SDT data reception at the new serving gNB side, the configuration information for RLC and logical channel is needed. Thus, the anchor node should provide this configuration to the new node. In this meeting, contributions [1][3][5][9][13][17] address this issue, and the following options can be considered for further discussion:

- Option 1: Partial UE context via RLC bearer configurations (RLCBearer-Config)

- Option 2: Full UE context via HandoverPreparationInformation

If agreed to retrieve partial UE context, in many contributions, the following new IEs can be discussed and then introduced (e.g., [1], [8], [11] and [16]).

Table 2: Partial UE context (direction: anchor node -> new node)

9.2.1.xx Retrieve UE Context for SDT

(Note: This Table is similar to split bearer, e.g., IE “PDU Session Resource Setup Response Info – SN terminated” in 9.2.1.6 in TS38.423)

This IE indicates that the SDT associated UE context shall be set up at the new NG-RAN node

| DRBs To Be Setup List |  | 0..1 |  |
| --- | --- | --- | --- |
| >DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |
| >>DRB ID | M |  | 9.2.3.33 |
| >>UL PDCP UP TNL Information | M |  | UP Transport Parameters 9.2.3. 76 |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |
| >>PDCP SN Length | O |  | 9.2.3.63 |
| >>RLC Mode | M |  | 9.2.3.28 |
| >>UL Configuration | O |  | 9.2.3.75 |
| >>QoS Flows Mapped To DRB List |  | 1 |  |
| >>>QoS Flows Mapped To DRB Item |  | 1 .. <maxnoofQoSFlows> |  |
| >>>>QoS Flow Identifier | M |  | 9.2.3.10 |
| >>>>QoS Flow Mapping Indication | O |  | 9.2.3.79 |
| >>>>Current QoS Parameters Set Index | O |  | Alternative QoS Parameters Set Index 9.2.3.103 |

Table 3: SDT DL address Transfer (direction: new node -> anchor node) (e.g., [1] and [16])

9.2.1.yy DL address transfer for SDT

This IE indicates that the SDT associated Xn-U DL address.

| DRBs To Be Setup List |  | 1 |  |
| --- | --- | --- | --- |
| >DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |
| >>DRB ID | M |  | 9.2.3.33 |
| >>DL Xn UP TNL Information | M |  | UP Transport Layer Information 9.2.3.30 |

However, in [3], it is suggested to retrieve full UE context. The advantage of retrieving full UE context is that we already have all the necessary IEs in place to provision necessary RLC configuration and UL forwarding TNLs to support "no anchor relocation" scenario of RA-SDT. But, as in [17] said, other UE context related IEs are also provided mandatorily, e.g. AS security context (new K*gNB, NCC, algorithms), NG-C signaling reference/TNL, UL NG-U UP TNL for each PDU session, etc., that are absolutely unnecessary.

Question 4: Which option do companies prefer, partial or full UE context?

- Option 1 : Retrieve partial UE context:

- The content within the above Table 2 and Table 3, as baseline into the XnAP specification?

- Please companies provide comment for the above mentioned “RLC bearer configuration transfer” (whether it is needed or not? Which option is selected?)

- Option 2: Retrieve full UE context:

- Reuse existing RETRIEVE UE CONTEXT RESPONSE message to retrieve full UE context

- Add an indication to indicate whether the anchor is kept or not for SDT

- Add SDT related UL address for UL SDT

| Company | Option 1/2 | Comment |
| --- | --- | --- |
| ZTE | Option 1 | We are fine to use Table 2/3 as baseline or start point into the XnAP spec. |
| Intel Corporation | Option 1 (partial context) | In case of "no anchor relocation", we think it is first necessary for RAN3 to decide whether to retrieve partial context or full context. Then, talk about which message to use (RTRV UE CTXT RESP vs FAILURE). We support retrieving only necessary context (partial) to make SDT work. |
| Lenovo, Motorola Mobility | See comments | It may be related to RRC container format. If the RRC container only includes RLC-config for SDT DRB related information, option 1 makes sense. If the RRC container includes RRC configuration for all DRBs, option 2 is preferred. We may need ask RAN2 to discuss/define the RRC container first. For option 1, the ‘partial’ should be further clarified. Whether the ‘partial’ means UE context for all configured SDT bearers or only for the SDT bearers which have ongoing data transmission. |
| CATT | Option 2 | Slightly prefer the option 2. The existing context retrieval procedure are greatly reused. For the option 1, how to define the “partial UE context” still need further discussion, maybe it should be done by RAN2. And transfer the “partial UE context” have more impact to Xn interface than the full UE Context relocation. |
| Google | Option 1 | Fine to use Table 2/3 as starting point |
| Nokia | Option 1 | Partial context with UL address as per tdoc R3-215356. |
| Huawei | Option 1 | We are fine to use Table 3. For table 2, we slight prefer it in [6] #### 9.2.1.xx SDT UE Context Information – Retrieve UE Context Response This IE contains the SDT UE context information within the RETRIEVE UE CONTEXT RESPONSE message. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RLC Bearer List; M. >RLC Bearer Item. >>RLC Bearer Configuration; 9.2.3.yy. Data Forwarding DRB List; 0..1. >Data Forwarding DRB Item; 1..<maxnoofDRBs>. >>DRB ID; M; 9.2.3.33. >>UL Forwarding UP TNL Information; O; UP Transport Layer Information 9.2.3.30. Before we decide the table, it’d better to determine the necessary content of the UE context. We think it is necessary to including the RLC and LCH channel configuration for SDT DRBs and SDT SRB, UL tunnels for SDT DRBs. Regarding the QoS and other context，it is FFS really needed for the SDT case. |
| LGE | Option 1 | We prefer Option 1, but if majority prefers Option 2, we are also fine. |
| E/// | Option 1 | Fetch necessary context as proposed in R3-215280. |
| Qualcomm | Option 1 | Can start with this as WA, and check any details depending on specific data distribution in the IEs |
| Samsung | Option 1 | We share the same view as HW. First, we need to determine the content of the UE context, which can be RLC-layer and logical channel configuration for SDT DRBs/SRBs. For the table shown by HW, RLC Bearer Configuration can be a container containing configuration of RLC-layer and logical channel for SDT DRBs/SRBs. So, it is unnecessary to use a list for RLC Bearer configuration. |
| China Telecom | Option 1 | We prefer Option 1” partial RLC configuration”. We are ok for Table 3. For table 2, we think RLC and LCH channel configuration are also needed. Because, the new node and anchor node may have different LCH channel configuration. In SDt procedure, UE sends the uplink data as per the LCH configuration of the anchor. When the new node receives the uplink data, it does not clear the mapping relationship between LCH and RLC/DRB. |
| NEC | Option 1 | Only necessary context will be enough. Table 2 and 3 can be a baseline for further checking. |

Summary: Majority companies (10:2) prefer to option 1.

WA: For RA-SDT without anchor relocation case, SDT related UE context shall be retrieved. FFS: the content of SDT related UE context

Option 1: Table 2 and Table 3 in this SoD can be as start point

Option 2:  R3-215001 “9.2.1.xx	SDT UE Context Information – Retrieve UE Context”

Option 3:  R3-215280 “9.2.1.xx	UE Context Information – Retrieve UE Context Response for SDT”

.

## RA-SDT without anchor relocation

### Whether prefer to enhance either Periodic RNA update without UE context relocation procedure or RNA update with UE context relocation procedure for SDT?

Majority contributions agree to transfer UE context and UL/DL address between new node and anchor node, but some contributions (e.g., [1], [3], [4], [9], [10] and [17]) prefer to enhance RETREIVE UE CONTEXT FAILURE message, while, some contributions (e.g., [5], [7], [12], and [15]) prefer to enhance RETREIVE UE CONTEXT RESPONSE message.

Moderator summaries the flow chart for RA-SDT as below (Figure 1).

Figure 1: General flow chart for RA-SDT

Step 2: The new gNB sends Retrieve UE context request message including SDT indicator.

Step 3: The anchor gNB decides to not relocate UE context, then sends partial UE context through XnAP message 1 (e.g., Retrieve UE context response/failure message, Xn-U address indication message, new message ).

Step 4: The new gNB sends SDT related DL address to anchor gNB, through XnAP message 2 (e.g., Xn-U address indication message, new message).

Step 5: The anchor gNB decides to terminate SDT (e.g., no SDT date expected), then sends RRCRelease message, included in XnAP message 3 (e.g., Retrieve UE context failure message, UE context release message).

In [17], it states that using REPONSE message anyway requires a new placeholder IE to provide necessary information and ignore those unnecessary mandatorily IEs. Moreover, using the RESPONSE message may complicate overall handling − two different scenarios by the same reply (normal context relocation including SDT, no anchor relocation for SDT.

Moderator’s view: If RAN3 agrees to enhance legacy Periodic RNA update without UE context relocation procedure for SDT, it will have less impact on both stage 2 and stage 3 protocols.

Question 5: For RA-SDT, which option do companies prefer?

Option 1: Enhance legacy Periodic RNA update without UE context relocation procedure (detail in 4.2.3)

Option 2: Enhance legacy RNA update with UE context relocation procedure (detail in 4.2.4)

Option 3: Other method to support SDT. If any, please input your suggestion

| Option 1: Enhance legacy Periodic RNA update without UE context relocation procedure Option 2: Enhance legacy RNA update with UE context relocation procedure Option 3: Other method to support SDT if any. Please input your suggestion |  |  |
| --- | --- | --- |
| Company | Option 1/2/3 | Comment |
| ZTE | Option 1 | If we agree to retrieve partial context, the legacy RETRIEVE UE CONTEXT FAILURE message seems better than RETREIVE UE CONTEXT RESPINSE message. |
| Intel Corporation | Seems a bit early | But we agree with the ZTE's comment, if RAN3 agrees to retrieve partial context in case of "no anchor relocation". |
| Lenovo, Motorola Mobility | Option 1 | We would prefer option 1. Currently the Retrieve UE Context Response message is only used for anchor relocation case. |
| CATT | Seems a bit early | The key point of the call flow is how to transfer the UE context, partial or full UE context will be transferred. |
| Google | Option 1 |  |
| Nokia | Option 1 | as per tdoc R3-215356 and R3-215368. |
| Huawei | Option 2 | For Option 1, in case of Retrieve UE Context Failure, the XnAP UE association is not established, the anchor gNB does not allocate corresponding UE AP ID, but after the Failure message, the new gNB still needs to provide DL address to the anchor gNB, therefore we should use the response message instead of failure message. For option 2, we can add a new partial UE context IE, and clarified that once this IE presence, the existing full UE context related IE shall be ignored. So it is not an issue to use Response message. |
| LGE | Option 2 | From the definition of the message point of view, we prefer Option 2 because the part of the UE context is provided to the new gNB from anchor gNB. |
| E/// | Option 2 | Considering the similarity of IEs, Response message is closer to what is required for SDT context fetch. |
| Qualcomm | Option 1 | Maybe some more analysis is needed, but our initial expectation was that the partial context relocation is a new thing that does not fit into the current messages very well. Hence better to reserve response to last message (failure with RRCRelease) as with RNAU, and do whatever needs to be done for traffic inside this. |
| Samsung | Option 1 | We think this case is for no context relocation. Then, from functionality point of view, the RETREIVE UE CONTEXT FAILURE is the one achieving this purpose. To avoid the removing of XnAP association for the UE, the specification can add some text to indicate the specific case for SDT. The problem of Option 2 is that the existing RETREIVE UE CONTEX RESPONSE is designed to transfer full UE context, which contains some mandatory IEs, e.g., GUAMI, UE Context Information – Retreive UE Context Response IE. Those IEs or some content in those IEs are unnecessary for the new serving gNB. Thus, Option 2 needs defining exception for those mandatory IEs, which is an unusual method. |
| China Telecom | Option 1 | We agree with ZTE’s view. RETREIVE FAILURE seems better than Response message. |
| NEC | Option 1 | Use of RETRIEVE UE CONTEXT FAILURE message. The RETRIEVE UE CONTEXT RESPONSE message is for relocation case. |

Summary: No consensus on this question. Option 1: 8 companies, Option 2: 3 companies, Too early to decide: 2 companies

WA: For RA-SDT without anchor relocation case, its stage 2 overall procedure shall enhance either legacy Periodic RNA update without UE context relocation procedure or legacy RNA update with UE context relocation procedure. FFS: which procedure of them.

### Enhance Periodic RNA update without UE context relocation procedure

In case of enhancing Periodic RNA update without UE context relocation procedure to retrieve UE context, there are two methods.

Method 1: Add two messages ([1] and [4])

Figure 2: Method 1

Both [1] and [4] suggest to enhance the existing Periodic RNA update without UE context relocation procedure.

Compared to the Figure 9.2.2.5-2: Periodic RNA update without UE context relocation in TS 38.300, in this method, two messages are added, i.e., the message 1 is to transfer SDT RLC config and UL address from anchor gNB to new gNB and the message 2 is to transfer DL address from new gNB to anchor gNB.

Observation 1: If using the Periodic RNA update without UE context relocation procedure with enhancement, for RA-SDT case, two messages (e.g., Xn-U Address Indication message) need to be added to transfer SDT RCL configuration and UL/DL address, no other change is needed.

Method 2: Modify three messages [17]

Figure 3: Method 2

In [17], it suggests another method, including the following enhancement based on the existing Periodic RNA update without UE context relocation procedure in TS 38.300.

- Enhance Retrieve UE context failure message, adding SDT RLC configure and UL address.

- Enhance Xn-U Address Indication message, adding SDT related DL address.

- Enhance UE context Release message, adding new feature to support SDT behavior

Compared to the method 1, the method 2 has more normative work, especially for the change of Retrieve UE context failure message and UE context Release message.

From moderator’s point of view, the method 1 can avoid the change of existing XnAP messages within the current Periodic RNA update without UE context relocation procedure. For SDT, the method 1 needs to insert two messages (e.g., Xn-U Address Indication message) to transfer SDT related information, it can also avoid the impact on the legacy procedure.

In conclusion, method 1 introduces less stage 2 and stage 3 work than method 2.

Question 6: If agreed to enhance Periodic RNA update without UE context relocation procedure, which method do companies prefer?

| Method 1: See figure 2 Method 2: See figure 3 |  |  |
| --- | --- | --- |
| Company | Method 1/2 | Comment |
| ZTE | Method 1 | Method 1 can have less effort than method 2, e.g., using legacy Stage2 procedure as much as possible, |
| Intel Corporation | Seems a bit early but Method 2 | We don't agree with the moderator that having extra two XnAP messages (2a, 2b in Figure 2 - Method 1) has less impacts than the Method 2 that re-uses the existing signalling as much as possible. Method 1 has more impacts. Xn-U ADDR IND has been defined to carry user-plane related information, and not even suitable to carry partial UE context. We should be open and discuss what is the best signalling flow to support SDT, rather than sticking to re-use the Periodic RNAU without UE context relocation, which is different to SDT in case of "no anchor relocation" scenario. |
| Lenovo, Motorola Mobility | Method 1 | We slightly prefer method 1 |
| CATT | Method 2 with comment | If partial UE context transfer is decided, the method 2 is preferred. |
| Google | Method 2 | For partial UE context transfer |
| Nokia | Method 2 | We also assume partial context transfer. Method 1 should be avoided due to for example nested procedures which is not good. |
| Huawei | Method 2, but | For these two method, method 2 is better, but For Method 1, it seems a little strange to exchange the Class2 procedure or another Class1 procedure between the Class1 Elementary Procedures. For the new serving gNB, it is not friendly to the implementation. And, Xn AP association has not been established. Whether or not the Xn-U Address Indication message can be used in step 2a in figure 1. For Method 2, Xn AP association has not been established. Whether or not the Xn-U Address Indication message can be used in step 5 in figure 1. |
| LGE | Method 2 | We have similar view with Intel. So, we slightly prefer Method 2. |
| E/// | None | Method 1 introduced complexity by having additional procedures, while at the same time the function can be realized by existing messages. |
| Qualcomm | Method 1 | It seems very strange to use a failure message to send context information. Also currently the failure message would be the last message for the association. Hence any other flow is actually breaking the basic principle of the procedure. We acknowledge that nested messages are not ideal, but otherwise there seems to be quite a lot of hacking. Another option would be to define a new context fetch procedure specifically for SDT access. Then it can be specifically optimized for the use case. |
| Samsung | Method 1 | Method 1 seems to have less impact to existing message, although two new messages for SDT is added. However, we are also open for Method 2. |
| China Telecom | slightly prefer method 1 | Both methods can work. We slightly prefer method 1 |
| NEC | ... | there are other ways e.g. adding DL TNL address in Retrieve UE Context Req message, adding Partial Context and UL TNL address in Retrieve UE Context Failure message |

Summary: Method 1: 5 companies, Method 2: 6 companies.

FFS: If agreed to enhance Periodic RNA update without UE context relocation procedure, whether it is acceptable to use the Retrieve UE context failure message to send SDT related UE context information

If agreed with method 1, both [1] and [5] suggest to add two messages to transfer SDT related RLC config and UL/DL address.

Method 1-1: Enhance Xn-U Address Indication message [4]

In [4], it suggests to enhance current Class 2 procedure, i.e. Xn-U Address Indication.

Table 4: Enhance Xn-U Address Indication message

9.1.1.11 XN-U ADDRESS INDICATION This message is either sent by the new NG-RAN node to transfer data forwarding information to the old NG-RAN node, or by the M-NG-RAN node to provide either data forwarding or Xn-U bearer address related information for SN terminated bearers to the S-NG-RAN node. For SDT, this message is either sent by the old NG-RAN node to transfer SDT related UE context information to the new NG-RAN node, or sent by the new NG-RAN node to transfer SDT related DL TNL address to the old NG-RAN node. Direction: new NG-RAN node  old NG-RAN node, M-NG-RAN node  S-NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Message Type; M; 9.2.3.1. New NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the new NG-RAN node. Old NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the old NG-RAN node. < Skip unchanged IEs >. Retrieve UE Context for SDT; O; 9.2.1.xx; Sent by the old NG-RAN node. DL address transfer for SDT; O; 9.2.1.yy; Sent by the new NG-RAN node.

Figure 4: Periodic RNA update/RA-SDT procedure without UE context relocation

Method 1-2: Introduce a new Class 1 procedure [1]

In [1], it suggests to introduce a new Class 1 procedure to transfer SDT RLC config and SDT tunnel. Compared to the solution in [4], the Class 1 procedure seems clearer than two Class 2 procedure. However, a new Class 1 procedure shall be specified.

Figure 5: Periodic RNA update/RA-SDT procedure without UE context relocation

Question 7: If agreed to enhance Periodic RNA update without UE context relocation procedure, which sub-method do companies prefer?

- Method 1-1: Enhance Xn-U Address Indication message [4]

  - Stage 3 work : The legacy Xn-U Address Indication message shall be enhanced as Table 4，the other XnAP messages can be reused without any enhancement.

  - Stage 2 work is based on the existing Periodic RNA update procedure without UE context relocation, as Figure 4.

- Method 1-2: Introduce a new Class 1 procedure [1]

  - Stage 3 work is to introduce a new Class1 procedure.

  - Stage 2 work is based on the existing Periodic RNA update procedure without UE context relocation, as Figure 5.

| Company | Method 1-1 or Method 1-2 | Comment |
| --- | --- | --- |
| ZTE | Both are fine, a slight prefer Method 1-2 | Both are fine, the current Stage 2 figure/procedure can be reused with a bit enhancement. Method 1-1 has a bit normative work. However, method 1-2 seems straightforward, which introduces a dedicate XnAP class 1 procedure for RLC context retrieve, similar to XnAP class 1 procedure UE context retrieve procedure. |
| Intel Corporation | Both are not fine | Method 1 is really not a good design. |
| Lenovo, Motorola Mobility | Method 1-2 | Class 1 procedure is more reliable. |
| Nokia | None | We prefer method 2. |
| Huawei | None | We do not support this solution, both of them are not good design. As commented in Q5 and Q6. |
| LGE | Method 1-2 if Method 1 is agreed in Q6. | If Method 1 is agreed in Q6, we prefer Method 1-2, i.e., a new Class 1 procedure needs to be defined. Anyway, we prefer Method 2. |
| E/// | None | Using Xn-U Address Indication procedure for context transferring is totally different with the design intention of the message, which means data forwarding purpose only. |
| Qualcomm | Method 1-1 slightly | Actually no strong preference, so fine to go with 1-2. Just wondering however if DL tunnels are always needed to be set up? |
| Samsung | Method 1-2 | Xn-U Address Indication is for dedicated purpose, i.e., providing Xn-U address for, e.g., forwarding. It is better to keep this rather than mix other functionalities in this message. A new class 2 message makes more sense to us. |
| China telecom | slight prefer Method 1-2 |  |
| NEC | ... | there are other ways e.g. adding DL TNL address in Retrieve UE Context Req message, adding Partial Context and UL TNL address in Retrieve UE Context Failure message |

Summary: No consensus

### Enhance RNA update with UE context relocation procedure

In [5], it suggests to extend XnAP: RETRIEVE UE CONTEXT RESPONSE message to carry the RLC configuration, and the DRB level data forwarding UL TNL Information, including the following change.

- Enhance Retrieve UE context Response message, adding SDT related UE context and UL addr

- Enhance Xn-U Address Indication message, adding SDT related DL address.

- Enhance UE context Release message, adding RRCRelease message to terminate SDT

As indicated in [17], much change within UE context Response message will be introduced.

It seems that new feature of SDT without anchor relocation is added, via some new IEs and via indicating some legacy mandatory IEs to be ignored.

Table 5: Enhance RETRIEVE UE CONTEXT RESPONSE message

- 9.1.1.9 RETRIEVE UE CONTEXT RESPONSE This message is sent by the old NG-RAN node to transfer the UE context to the new NG-RAN node. Direction: old NG-RAN node  new NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Message Type; M; 9.2.3.1. New NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the new NG-RAN node. Old NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the old NG-RAN node. GUAMI; M; 9.2.3.24. UE Context Information – Retrieve UE Context Response; M; 9.2.1.13; This IE shall be ignored if the IE” Retrieve UE Context for SDT” present.. < Skip unchanged IEs >. Retrieve UE Context for SDT; O; 9.2.1.xx; Sent by the old NG-RAN node.

Table 6: Enhance XN-U ADDRESS INDICATION message

9.1.1.11 XN-U ADDRESS INDICATION This message is either sent by the new NG-RAN node to transfer data forwarding information to the old NG-RAN node, or by the M-NG-RAN node to provide either data forwarding or Xn-U bearer address related information for SN terminated bearers to the S-NG-RAN node. For SDT, this message is sent by the new NG-RAN node to transfer SDT related DL TNL address to the old NG-RAN node. Direction: new NG-RAN node  old NG-RAN node, M-NG-RAN node  S-NG-RAN node. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Message Type; M; 9.2.3.1. New NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the new NG-RAN node. Old NG-RAN node UE XnAP ID reference; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated at the old NG-RAN node. < Skip unchanged IEs >. DL address transfer for SDT; O; 9.2.1.yy; Sent by the new NG-RAN node.

Table 7: Enhance UE CONTEXT RELEASE message

#### 9.1.1.5 UE CONTEXT RELEASE This message is sent by the target NG-RAN node to the source NG-RAN node to indicate that resources can be released. Direction: target NG-RAN node  source NG-RAN node, M-NG-RAN node  S-NG-RAN node, anchor NG-RAN new NG-RAN node IE/Group Name; Presence; Range; IE type and reference; Semantics description. Message Type; M; 9.2.3.1. Source NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated for handover at the source NG-RAN node or for dual connectivity at the S-NG-RAN node.. Target NG-RAN node UE XnAP ID; M; NG-RAN node UE XnAP ID 9.2.3.16; Allocated for handover at the target NG-RAN node or for dual connectivity at the M-NG-RAN node.. SDT without context relocation; 0..1. >RRC Container; O; OCTET STRING; Includes the RRCRelease message as defined in TS 38.331 [10], encapsulated in a PDCP-C PDU..

Figure 5: RA-SDT procedure without UE context relocation

Question 8: If agreed to enhance RNA update with UE context relocation procedure, do companies agree with the following enhancements?

- Enhancement 1:  Retrieve UE context Response message, referred to Table 5

- Enhancement 2:  Xn-U Address Indication message, referred to Table 6

- Enhancement 3:  UE context Release message, referred to Table 7

- Enhancement 4:  Path Switch procedure will not be used.

- Stage 2 work is as Figure 5

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE | Yes | If we agree to enhance RNA update with UE context relocation procedure, much enhancements on the legacy procedure and legacy message shall be introduced, so I think it is not a good way. |
| Intel Corporation | No idea | We cannot understand the moderator's understanding. We should not discuss based on following the existing RNAU with or without UE context relocation procedures.. Again, we should be open and discuss what is the best signalling flow to make SDT work flawlessly based on the required functionalities.. |
| Lenovo, Motorola Mobility | Yes | We agree with moderator’s summary. |
| CATT | Partially agree | Generally we are fine with the overall call flow as shown in Figure 5. - Enhancement 1: Retrieve UE context Response message, referred to Table 5 To the enhancement 1, we still prefer to transfer the full UE Context from anchor gNB to receiving node, with an indication to keep the anchor. |
| Google | Not sure | Agree with Intel |
| Nokia | Not sure | Same view as ZTE. If we agree to enhance RNA update with UE context relocation procedure, many enhancements on the legacy procedure and legacy message shall be introduced, so I think it is not a good way. |
| Huawei | Yes | It aligns with the current Class1 and class2 procedure design principle. And the design is pretty clear and just add some new IE in the legacy message. We don’t see much impact at all instead of embedding one procedure during another procedure as above method 1 in section 4.2.3. For Enhancement 1, it has less impact compared to Retrieve UE context failure message in Method 2 of section 4.2.3. For Enhancement 2, it is common impact for all the method above. For Enhancement 3 and 4. They are straight forward changes, and not complex. Generally speaking, the enhanced to RNA update with UE context relocation procedure is a better solution than others. |
| LGE | Yes | We also agree with moderator’s summary. |
| E/// | Yes | As raised in R18 discussion by some companies, there is potential risk that support of without anchor becomes too complicated. So reusing the existing procedures as much as possible is the best way forward. |
| Qualcomm | Yes | But need to agree first on the top flow |
| Samsung | Conditional yes | This is on the condition that RAN3 agrees to ues RNA update with UE context relocation procedure. In addition, we may also need to discuss how to deal with some mandatory IEs in RESPONSE message. |
| China Telecom | yes |  |
| NEC | Yes | Additional, think that it is not a good way to use of RNA update with UE Context Relocation procedure for the purpose of UE Context without UE Context Relocation purpose. |

Summary: No consensus

FFS: If agreed to enhance RNA update with UE context relocation procedure, whether it is acceptable to include the following normative work.

- Enhancement 1:  Retrieve UE context Response message, referred to Table 5

- Enhancement 2:  Xn-U Address Indication message, referred to Table 6

- Enhancement 3:  UE context Release message, referred to Table 7

- Enhancement 4:  Path Switch procedure will not be used.

## Others

Moderator’s view:

Due to maximum number of questions set to 8, the following questions are low priority and can be answered in the second round. However, all companies are kindly invited to answer them in the first round or second round as your wish.

### Extend RRC Resume Cause IE

In [5], the RRC Resume Cause IE is suggested to extend for SDT. In detail, currently the RRC Resume Cause IE is defined as ENUMERATED (rna-Update, ...), and limited to the case of RNA update. Considering that the RACH based SDT supports data delivery via DRB and via SRB (NAS PDU for Positioning), it is better to inform such difference to the last serving gNB, it may be needed to extend the RRC Resume Cause IE to include mo-data and mo-signalling.

- 9.2.3.61 RRC Resume Cause The purpose of the RRC Resume Cause IE is to indicate to the old NG-RAN node the reason for the RRC Connection Resume as received from the UE in the ResumeCause defined in TS 36.331 [14] and TS 38.331 [10]. In this version of the specification, this is limited to the case of RNA update, and SDT. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RRC Resume Cause; M; ENUMERATED (rna-Update, mo-data, mo-signalling, ...).

Question 9: Do companies agree to extend the RRC Resume Cause IE, adding SDT case (e.g., mo-data and mo-signalling)?

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE | Yes |  |
| Intel Corporation | No | If we are going to have SDT indicator, then why RRC Resume Cause needs to be extended? Did RAN2 already agree to do so? |
| Lenovo, Motorola Mobility | Too early to decide | We may need to wait RAN2 progress. If we have the SDT assistance information, the new causes seem unnecessary. |
| CATT | Seems No | It’s duplicated with the SDT Indicator as part of the SDT assistance information. |
| Google | Seems No | Seems duplicated with the SDT Indicator if agreed |
| Nokia | No | This seems related to how the PDCP-C PDU is transferred. If in UE Contxt Retrieve Request seems not needed. |
| Huawei | Yes | They can help the anchor gNB to distinguish SRB SDT and DRB SDT, which should be considered by the anchor gNB to make decision on with or without anchor relocation. |
| LGE | Yes |  |
| E/// | No |  |
| Qualcomm | Yes | We understand there is slight duplication, but seems reasonable to enhance cause. Once we have full details of the assistance we can decide how / whether there is duplication, and how to structure the IEs. |
| Samsung | No | Agree with Intel |
| NEC | Too early |  |

Summary: No consensus

FFS: Extend the RRC Resume Cause IE, adding SDT case (e.g., mo-data and mo-signalling).

### Data forwarding between serving gNB and anchor gNB

In [9], it states that when the serving gNB receives SDT data associated with RRCResumeRequest message from UE, the serving gNB buffers the SDT data. The serving gNB forwards the SDT data to the anchor gNB after Retrieve UE Context for SDT. Basically, there are three options on the data forwarding:

- Option 1: the serving gNB forwards PDCP PDUs (RLC SDUs) per DRB GTP-U tunnel to the anchor gNB.

- Option 2: the serving gNB forwards RLC PDUs per DRB GTP-U tunnel to the anchor gNB.

- Option 3: the serving gNB forwards MAC PDUs via a shared GTP-U tunnel to the anchor gNB.

In [5], it also suggest to forward PDCP PDU from the new gNB to the anchor gNB.

Question 10: Do companies agree that which option the new gNB forwards to the anchor gNB?

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE | Option 1 | Option 1 is aligned with RAN2 progress |
| Intel Corporation | Option 1 only |  |
| Lenovo, Motorola Mobility | Option 1 |  |
| CATT | Option 1 |  |
| Google | Option 1 |  |
| Nokia | Option 1 | As aligned with RAN2. |
| Huawei | Option1 |  |
| LGE | Option 1 |  |
| E/// | Option 1 |  |
| Qualcomm | Option 1 for tunnels | This excludes the first message, which keeps not being discussed. As noted many times, the “alignment with RAN2” does not correspond fully to their actual statements. |
| Samsung | Option 1 |  |
| China Telecom | Option 1 | If we consider CU/DU split architecture, RLC SDU is the most suitable option. |
| NEC | Option 1 |  |

Summary: all agree with option 1.

Proposal 4: The serving gNB forwards PDCP PDUs (RLC SDUs) per DRB GTP-U tunnel to the anchor gNB

### Whether 5GC needs to be involved for SDT

In [19], it states that, as agreed in RAN2, the SDT configuration is per DRB basis. The gNB decides which DRB is configured for SDT and includes the SDT related configuration in the RRCRelease message. One issue is how the gNB decides to configure a DRB for SDT. It could be beneficial that 5GC provides whether a PDU session or QoS flow is subjected to SDT for gNB deciding to configure one DRB for SDT.

Question 11: Do companies agree that 5GC is required to provide whether a PDU session or QoS flow is subjected to SDT for gNB deciding to configure one DRB for SDT？If yes, the LS to SA2 shall be sent as early as possible.

| Company | YES/NO | Comment |
| --- | --- | --- |
| ZTE | No | 5GC will not be involved for RRC inactive UE |
| Intel | Too early | We can discuss if requested by other WGs. BTW, the reference [19] is not correct. Intel didn't talk about 5GC involvement at all. |
| Lenovo, Motorola Mobility | Yes | Some implicit or explicit indication for 5GC is needed so that the gNB can decide which QoS flow or PDU session should be configured as a SDT DRB. |
| CATT | Pending | Pending to other WGs. |
| Google | Seems no |  |
| Nokia | Pending | Pending to other WGs. |
| Huawei | Yes | Maybe 5GC have a better idea of the Qos flow is suitable for the SDT. We think it can be used as an assistant information for the SDT configuration, it is beneficial to select the SDT bearer for gNB. |
| LGE | No | For now, we think that there is no requirement on this indication from 5GC. But we can further discuss this issue based on other WG’s input. |
| E/// | No |  |
| Samsung | Yes | The QoS information comes from 5GC, which can determine whether a QoS flow can support SDT or not. Thus, when configuring PDU sessions, the 5GC can indicate whether a QoS flow can be applied for SDT or not. |
| China telecom | No | Whether a bearer is SDT bearer is decided by OMC configuration…. |
| NEC | Prefer Not |  |

Summary: No consensus

FFS:  5GC is required to provide whether a PDU session or QoS flow is subjected to SDT for gNB deciding to configure one DRB for SDT.

### F1 related impacts

In this meeting, some contributions also mentioned some F1 impacts, which include:

- Impact 1: assist information provision in INITIAL UL RRC MESSAGE TRANSFER [5][10] (already covered by Q3&Q4)

- Impact 2: RLC bearer configuration query at anchor gNB side [3]. The intention is to help the anchor gNB-CU derive the RLC bearer configuration for SDT bearers before sending the UE to the INACTIVE status, since those configuration will be sent to the serving gNB when SDT session is started.

- Impact 3: RLC bearer configuration provision to gNB-DU at serving gNB side [3][10]. The intention is to provide the configurations to serving gNB-DU for SDT data transmission

- Impact 4: UE inactivity notification to indicate the end of SDT at serving gNB side [10]. The intention is to indicate the activity of SDT bearers so as to determine whether to end the SDT session or not.

Question 12: Please companies provide comments to the above impact 2~4 (impact 1 is already covered by Q3&4), e.g., whether the impact can be acknowledged?

| Company | Comment |
| --- | --- |
| ZTE | We agree with impact 1 and 3, are not sure of impact 2 and impact 4. |
| Intel Corporation | We think impacts 2 and 3 are essential for SDT to work. The impact 1 or impact 4 are not essential at this moment, and think can be discussed later. |
| Lenovo, Motorola Mobility | We also agree with impact 1 and impact 3 and we are not sure about impact 2&4. |
| CATT | We are acknowledged the impact 1 and 3 should be further considered. For Impact 2, it seems the intension is for anchor gNB-CU to get the stored UE context in the last serving DU. However, we understand anchor gNB-CU should have the full UE context for a UE in RRC Inactive. For impact 4, not sure how can the receiving gNB decide the SDT session end? We believe the anchor could also do such kind of judgement base on implementation, or refer to the SDT assistance info provided by the receiving node. |
| Google | Impact 2 and 3 should be further considered. |
| Nokia | Impacts 1, 3, 4 seems needed. Not sure about impact 2 (need to be checked about anchor CU knowledge of DU context). For impact 4 we think SDT end is better evaluated by serving gNB than anchor gNB because serving gNB is one that can have the BSR. |
| Huawei | Agree 1 and 3. |
| LGE | We also think that impacts 1 and 3 needs to be considered. But, we are not sure for impacts 2 and 4. |
| E/// | For Impact 1, we would say SDT indicator, assistant info is not decided yet. For Impact 2, CU anyway has the UE context. Impact 3 can be further considered. Impact 4 seems not required. |
| Samsung | We think impact 2 and 3 are essential for RA-SDT. For impact 2, gNB-CU only has the CellGroupConfig as a container for all RBs. However, if we agree to transmit partial context to the serving gNB, gNB-Cu cannot derive partial context from the CellGroupConfig since the information since Rel15 does not require CU to decode CellGroupConfig container. Impact 1 is already addressed by other question in this CB. |
| China Telecom | Agree 1 and 3 |

Summary: No consensus

FFS: At least, the following impact on F1AP.

- Impact: RLC bearer configuration query at anchor gNB side. The intention is to help the anchor gNB-CU derive the RLC bearer configuration for SDT bearers before sending the UE to the INACTIVE status, since those configuration will be sent to the serving gNB when SDT session is started.

- Impact: RLC bearer configuration provision to gNB-DU at serving gNB side. The intention is to provide the configurations to serving gNB-DU for SDT data transmission

- Impact: UE inactivity notification to indicate the end of SDT at serving gNB side. The intention is to indicate the activity of SDT bearers so as to determine whether to end the SDT session or not.

### E1 related impacts

It will be within another SDT CB.

### Left issues

Question 13: Do companies suggest to discuss other issue for RA-SDT with/without anchor relocation? If yes, please input your suggestion.

| Company | YES/NO | Comment |
| --- | --- | --- |
| Huawei | Yes | How to handle the case when DL non-SDT arrives during the SDT procedure in case of w.o anchor relocation? Moderator‘s view: It is included in CB #SDT3-others |
| Samsung | Yes | Whether the anchor gNB-DU need to know which bearer is SDT bearer. |

FFS: Whether the anchor gNB-DU need to know which bearer is SDT bearer.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-214845	Discussion on RACH based SDT (ZTE)	other

- R3-214847	BLCR 38.300 for support of SDT (ZTE)	draftCR

- R3-214883	Discussion on RA-based small data transmission (Samsung)	discussion

- R3-214904	Further discussion on RAN3 impacts of RACH based SDT (Qualcomm Incorporated)	discussion

- R3-215000	(TPs to TS 38.300 38.473 38.401BL CRs) Support of RACH based SDT (Huawei)	other

- R3-215001	Supporting of RACH based SDT (Huawei)	CR0679r, TS 38.423 v16.7.0, Rel-17, Cat. B

- R3-215279	Context fetch and data forwarding for NR SDT (Ericsson)	discussion

- R3-215280	Support of NR SDT over Xn (Ericsson)	CR0692r, TS 38.423 v16.7.0, Rel-17, Cat. B

- R3-215319	SDT Related Procedures (Lenovo, Motorola Mobility)	discussion

- R3-215356	Support of RACH-based SDT (Nokia, Nokia Shanghai Bell)	discussion

- R3-215368	Support of RACH-based SDT (Nokia, Nokia Shanghai Bell)	CR0697r, TS 38.423 v16.7.0, Rel-17, Cat. B

- R3-215599	Discussion on RA-based SDT with anchor relocation (CATT)	discussion

- R3-215600	Discussion on RA-based SDT without anchor relocation (CATT)	discussion

- R3-215601	Support of RA-based SDT (CATT)	draftCR

- R3-215709	Consideration on small data transmission without anchor relocation (LG Electronics)	discussion

- R3-215710	Support of RACH-based SDT over Xn (LG Electronics)	CR0712r, TS 38.423 v16.7.0, Rel-17, Cat. B

- R3-215745	Context retrieval and data forwarding for RACH based SDT (Intel Corporation)	discussion

- R3-215746	Assistance Information from New gNB for RACH based SDT (Intel Corporation)	discussion

- R3-215747	[Draft] SDT BL CR to 38.423 (Intel Corporation)	draftCR