3GPP TSG-RAN WG3 #114-e	R3-215853

Online, 1-11 Nov, 2021

Agenda Item:	10.2.1.6

Source:	Samsung (moderator)

Title:	Summary of Offline Discussion on MRO for SN Change Failure

Document for:	Approval

# Introduction

CB: # SONMDT4_SNChangeFailure

- MRO for SN Change failure for pre-R17 UEs. Whether and how to handle intra-SN PSCell change failure?

- continue to discuss the IEs included in the new XnAP message.

- scenarios for SN change failure in R17

- ambiguity in SCG failure case?

- Any other topic?

- Start with summary of offline, proceed to TPs if there are agreements

(Samsung - moderator)

Summary of offline disc R3-215853

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: xxx

# Discussion

## Available scenarios for MRO of SN change failure

In [1], it’s proposed to depend on RAN plenary.

In [3], it’s proposed to support EN-DC and NR-DC scenarios for SN change failure scheme.

In [4] and [8], it’s proposed to support all MR-DC cases for SN change failure scheme.

In [5], it’s proposed to support only NR-NR DC in R17.

Q1: What’s your viewpoint on the available scenarios for MRO of SN change failure?

| Company | Comment |
| --- | --- |
| Samsung | RAN3 has agreed “Prioritize NR-NR DC only” in the RAN3#110e meeting. Other cases are low priority. Considering only two meetings are left for Rel-17, we prefer to stick this agreement. |

## Whether source SN has the UE context when it receives SCGFailureInformtion

In [1], it’s proposed that keeping the UE context for the short while after completing SN change may be a mandatory part of the MRO for SN change.

In [2][3][10], the view is that the UE context may has been released in the source SN after successful SN change.

In [11], the view is that this is depending on which message is used for reporting the additional information.

Q2: Do you think the source SN always has the UE context when it receives SCG Failure Informtion?

| Company | Comment |
| --- | --- |
| Samsung | No. For the failure after successful SN change, the source SN may have no UE context. Because the source SN releases the UE context when receiving UE Context Release message. Pls see the following description in TS 37.340: 16. Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue |

## Intra-SN PSCell change after MN/SN initiated SN change for pre-R17 UEs

Let’s take an example for easy description:

- SN1 triggered SN change from SN1->SN2 successfully (from PSCell1 in SN1 to PSCell2 in SN2).

- SN2 triggered an intra-SN PScell change from PScell2 to PSCell3. Failure happens during or after intra-SN PSCell change in SN2.

There are different understandings on which node SCGFailureInformation should be sent to. It can be summarized to 2 options:

Option A: It is not necessarily mean the SN2 is the node which bring the problem if there is intra-SN PSCell change in SN2. So SCGFailureInformation should be forwarded to SN1 or SN2, which is decided by MN. [4][10]

Option B: SCGFailureInformation should be forwarded to SN1 if there is no intra-SN PSCell change in SN2, and to SN2 if there is intra-SN PSCell change in SN2. A class-1 procedure should be defined. [1][2][3][8]

Q3: Which option do you support?

| Company | Comment |
| --- | --- |
| Samsung | Option A MN has overall information, therefore can make a right decision on the node that caused the SCG failure. E.g. SCG failure just after successful SN1->SN2 change. According to UE measurement report, if the PSCell in SN2 is still a good cell, it could be intra-PSCell change issue. But if the PSCell in SN1 is a good cell while no good cell in SN2, it could be too early PSCell change no matter there is intra-PSCell change in SN2 or not. If the MN simply sends SCGFailureInformation to the SN2, SN2 reply that there is an intra-SN PSCell change, the issue of MRO for SN change could be excluded. It is not right. |

If Option B, [1] discussed 2 options. [8] raised an issue which SN should be responsible to analyze the PSCell change failure type, and provide 2 options. These discussions can be summarized to 2 solutions:

Solution B1: MN always forward SCG failure report to last serving SN. If no intra-SN PSCell change, last serving SN indicates it to MN, so it’s SN change failure. A class-1 procedure should be defined. [1][2][8]

Solution B2: MN asks first the last serving SN whether intra-SN PSCell change occurs. If yes, MN forward SCG failure report to last serving SN; Otherwise it’s SN change failure. A class-1 procedure should be defined. [3]

Q4: If Option B, which solution do you support?

| Company | Comment |
| --- | --- |

If Option B, a class-1 procedure will be defined. Some IEs are proposed in [8] to be included in the initiating/response message of the class-1 procedure:

In initiating message:

- SCG failure information from UE

- Whether needs to add SN after SCG failure? if needed, the next suitable PSCell CGI

- For Pre-Rel-17 UE case, without enhanced SCG failure information, MN may provide SN with source PSCell CGI, Failed PSCell CGI for SN detecting PSCell change failure type, if available in MN.

- UE history information

In response message:

- PSCell change failure type.

- Source PSCell CGI，Target PSCell CGI，Failed PSCell CGI

- SN selected the next PSCell CGI

Q5: Please provide your viewpoints about the proposed IEs

| Company | Comment |
| --- | --- |

## Information other than SCGFailureInformation in new XnAP message

In the last meeting, we agreed

b)	Source PSCell CGI, if available in MN

c)	Failed PSCell CGI, if available in MN

it’s still FFS:

a) PSCell failure type

d) Suitable PSCell CGI

e) Mobility Information

f) PSCell selection assistant information, e.g. UE history information

g) Initiating node type i.e. MN or SN

h) S-NG-RAN node UE X2AP ID

i) M-NG-RAN node UE XnAP ID

In addition, 2 new IEs are proposed in [8]:

j) Indicator for Whether to add SN (if no class 1 procedures ahead)

k) SN selected the next PSCell CGI (if class 1 procedures ahead)

Q6: which information should be included in the new XnAP message other than SCGfailureinformation?

| Company | Comment |
| --- | --- |
| Samsung | - d) e) For a), MN has to decide which node should SCG failure information be forwarded to by performing initial analysis for the SCG failure on the failure type. So the initial analysis on the failure type is unavoidable. It’s beneficial that MN can send the failure type to the source SN for information. For d), it’s for pre-R17 UEs. MN is the suitable node to select next PSCell. If source SN to select, it doesn’t know if there is Xn connectivity between the new SN and MN, as mentioned in [4]. For e) and h) i), in case of SCG failure just after successful SN1->SN2 change, it’s possible that SN has released the UE context. So the SN might not know UE IDs. Similar as HO case, mobility information is useful. So e) is helpful. But h) and i) is not necessary. For g), MN know which node initiated SN change. If MN initiated SN change, MN will not forward SCGFailureInformation to source SN. So if SN receives SCGFailureInformation, it’s supposed SN initiated SN change. g) is not necessary. |

For Suitable PSCell CGI, it’s proposed in [4]:

if MN decides not to continue the DC configuration for the UE, MN can send the additional information to source SN to indicate whether the cell(s) in the measurement results has direct Xn connectivity with the MN.

Q8: Is it OK for you?

| Company | Comment |
| --- | --- |
| Samsung | MN can select the suitable PSCell even it decide no DC for the UE. |

## Ambiguity in SCG failure cases

In the last meeting, a scenario is introduced: after RRC Reconfiguration message for SN change initiated by SN is received by a UE, a random access procedure occurs at the serving PSCell due to other reason, e.g. for timing synchronization, then it fails. So an SCG failure report is produced. It will make source SN confused whether it’s SN change failure or not.

Some companies think it’s an invalid scenario because NW can recognize the 2 failures based on the legacy specifications. [1][4][5]

Q9: Do you think there is ambiguity in SCG failure cases?

| Company | Comment |
| --- | --- |
| Samsung | We don’t find ambiguity in SCG failure cases. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-214814, [TP to SON BL CR to 38.423, TP to SON BL CR to 36.423, NR_ENDC_SON_MDT_enh]  MRO for PSCell change failure (Nokia, Nokia Shanghai Bell)

- R3-214841, Consideration on MRO for SN Change Failure (China Telecommunication)

- R3-214916, MRO for SN Change Failure (Qualcomm Incorporated)

- R3-214958, (TP for SON BLCR for 38.423, 38.300) MRO for SN Change Failure (Huawei)

- R3-214985, Further consideration on MRO SN change failure (ZTE)

- R3-214986, (TP for SON BL CR for TS 38.423) MRO SN change failure (ZTE)

- R3-214987, [draft] LS on MN awareness of Source PSCell CGI and Source PSCell CGI (ZTE)

- R3-215066, (TP for 38.423) consideration on support of SN change failure in case of MR-DC (CATT)

- R3-215067, Reply LS on information needed for MRO in SCG Failure Report (CATT)

- R3-215518, TP for SON BLCR for TS38.423: Support of SON for SN change failure (Samsung)

- R3-215442, MRO for SN change failure, Ericsson