3GPP TSG-RAN WG3 #114-e	R3-215944

1 – 11 Nov 2021

Agenda Item:	9.3.6.1

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on  UL PDU Session Information

Document for:	Approval

Introduction

CB: # 111_ULPDUSession

- Check group understanding on the directions

- Find the proper wording and update the figure

(HW - moderator)

Summary of offline disc R3-215944

For the Chairman’s Notes

Propose to agree the following proposals:

Proposal 1: In case of intra-system handover with direct data forwarding, for UL PDU session tunnel and DL PDU session tunnel, the DL PDU Session Information procedure (PDU Type 0) shall be used to send control information elements related to the PDU Session from NG-RAN to NG-RAN.

Proposal 2: The procedure texts are updated as “from NG-RAN node to UPF, or from UPF to NG-RAN node, or between NG-RAN nodes” (i.e. option 2).

Proposal 3: The Figure 5.4.1.1-1 is unchanged.

Proposal 4: R16 CR only is agreed.

To capture the above proposals:

Proposal A: Agree R3-216005 (revision of R3-215165 for R16, and Category is changed to “F”, the WI code is changed to “NR_newRAT-Core, TEI16”, adding Nokia, Nokia Shanghai Bell as co-signer).

Proposal B: withdraw R3-216004 (revision of R3-215164 for R15).

Discussion – Second round

The following proposals made.

Proposal 1: In case of intra-system handover with direct data forwarding, for UL PDU session tunnel and DL PDU session tunnel, the DL PDU Session Information procedure (PDU Type 0) shall be used to send control information elements related to the PDU Session from NG-RAN to NG-RAN.

Proposal 2: the proposal texts are updated as follows (i.e. option 2).

The purpose of the Transfer of DL PDU Session Information procedure is to send control information elements related to the PDU Session from UPF to NG-RAN. In the case of uplink and downlink data forwarding the DL PDU Session Information procedure shall be used to send control information elements related to the PDU Session from NG-RAN to UPF, or from UPF to NG-RAN, or between NG-RAN nodes.

Proposal 3: The Figure 5.4.1.1-1 is unchanged.

And there are comments to have the R16 CR only.

Proposal 4: R16 CR only is agreed.

If you have different view, please provide any view / comments on this topic and the question in bold below:

| Company | Comment |
| --- | --- |

Discussion – First round

Based on the online discussion, it seems that we can make consensus for the following proposal.

Proposal: In case of intra-system handover with direct data forwarding, for UL PDU session tunnel and DL PDU session tunnel, the DL PDU Session Information procedure (PDU Type 0) shall be used to send control information elements related to the PDU Session from NG-RAN to NG-RAN.

If you have different understanding, please provide any view / comments on this topic and the proposal in bold below:

| Company | Comment |
| --- | --- |

Moderator’s summary:

The proposal above can be agreed.

Update of procedure texts

At the online discussion, there are views to remove “from UPF/NG-RAN to NG-RAN/UPF” from the sentence.

Option 1:

The purpose of the Transfer of DL PDU Session Information procedure is to send control information elements related to the PDU Session from UPF to NG-RAN. In the case of uplink and downlink data forwarding the DL PDU Session Information procedure shall be used to send control information elements related to the PDU Session.

Also there are views to provide clear descriptions to cover all possible cases.

Option 2:

The purpose of the Transfer of DL PDU Session Information procedure is to send control information elements related to the PDU Session from UPF to NG-RAN. In the case of uplink and downlink data forwarding the DL PDU Session Information procedure shall be used to send control information elements related to the PDU Session from NG-RAN to UPF, or from UPF to NG-RAN, or from NG-RAN to NG-RAN.

Question: your preference of the above procedure text update?

Please provide any view / comments on this topic and the question in bold below:

| Company | Comment |
| --- | --- |
| Huawei | Option 2 is preferred, since the descriptions are much clear compared with option 1. Also option 2 has the very symmetric structure as the previous paragraph. - ……related to the PDU Session from UPF to NG-RAN In addition, it seems good to clearly specify the direct or indirect data forwarding as shown below. - “In the case of uplink and downlink (in)direct data forwarding……” |
| CATT | Both options are OK. If option 2 is adopted,maybe we could change “from NG-RAN node to NG-RAN node” to “between NG-RAN nodes” |
| Ericsson | Option 1 sounds clearer to us |
| Nokia | Both option are OK. Slight preference for option 2 if we make a change. |
| ZTE | Option 2 seems more clear. |

Moderator’s summary:

Majority companies are fine with option 2, so option 2 could be agreed.

Update of Figure 5.4.1.1-1

The original figure in TS 38.415 is copied as below.

Figure 5.4.1.1-1: Successful Transfer of DL PDU Session Information

Then three options are provided for discussion.

- Option A: No change

- Option B: Update as proposed in 5164 as follows.

- Option C: To have separate figure for each case

Question: Your preference of the Figure 5.4.1.1-1?

Please provide any view / comments on this topic and the question in bold below:

| Company | Comment |
| --- | --- |
| Huawei | Slightly prefer option B. If option 2 in section 3.1 is finally selected, then do nothing is also acceptable to us, since the procedure texts is somehow clear. |
| CATT | Option B or Option A with a note that the message could also be transferred between NG-RAN nodes. |
| Ericsson | Option A or C |
| Nokia | Option A or C. |
| ZTE | Option A. The figure can cover the three cases list in Option 2 above. Maybe a note is needed to mention that ‘UPF to UPF’ is excluded, in case that the figure itself might bring some confusion. |

Moderator’s summary:

All companies are fine with “no change”, while two companies think a note is needed. Two companies support B, and two companies support option C. So “no change” option is agreed. The moderator tends to think that the note is not needed since the procedure text is pretty clear.

The CR can be discussed at next phase.

Further aspects

Please add any further aspects that are in scope and were not included in the above:

| Company | Comment |
| --- | --- |
| Nokia | Release 16 CR is good enough. No R15 CR. |

Moderator’s summary:

The moderator is fine with R16 only CR.

Conclusion, Recommendations [if needed]

If needed

References

| R3-215164 | Correction for UL PDU Session Information (Huawei, China Telecom) | CR0029r1, TS 38.415 v15.3.0, Rel-15, Cat. F ** |
| --- | --- | --- |
| R3-215165 | Correction for UL PDU Session Information (Huawei, China Telecom) | CR0030r1, TS 38.415 v16.5.0, Rel-16, Cat. A ** |