3GPP TSG-RAN WG3 Meeting #114-e	R3-216221

E-meeting, 1 – 11 Nov 2021

Agenda item:	10.2.1.4

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	[NR_ENDC_SON_MDT_enh, TP SON BL CR to TS 36.413] Addition of SCG UHI information

Document for:	Endorsement

# 1	Introduction

At the RAN3 #114 meeting, RAN3 agreed to add PSCell information to the existing UE History Information. E-UTRAN cells are defined in S1AP, hence a TP to this specification is needed.

# 2	Text proposal for TS 36.413

The text proposal below is based in the most recently endorsed NGAP BL CR [R3-214609]. Please note: ASN.1 to be added once the TP is endorsed.

First change, ommited text not changed

#### 9.2.1.43a	Last Visited E-UTRAN Cell Information

The Last Visited E-UTRAN Cell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | E-UTRAN CGI 9.2.1.38 |  | - |  |
| Cell Type | M |  | 9.2.1.66 |  | - |  |
| Time UE stayed in Cell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the cell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. | YES | ignore |
| HO Cause Value | O |  | 9.2.1.3 | The cause for the handover from the E-UTRAN cell. | YES | ignore |
| Last Visited PS Cell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.2.1.X | The PSCell related information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PScell information records that can be reported in the IE. Value is FFS. |

Next change, ommited text not changed

#### 9.2.1.X	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Last Visited PS Cell Information | M |  | FFS | FFS |

Remaining text not changed

[ASN.1 to be added, if endorsed]

Remaining text not changed