3GPP TSG-RAN3 Meeting #114bis-e	R3-220102

Electronic meeting, 17- 26 January 2022

3GPP TSG-RAN WG2 Meeting #116 electronic		R2-2111415

Online, 1 - 12 November 2021, 2021

Title:	LS on paging subgrouping and PEI

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN2

To:	SA2, CT1, RAN3, RAN1

Cc:

Contact Person:

#### Name:	Yanhua Li

#### E-mail Address:	liyanhua1@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 discussed UE paging subgrouping as part of the Rel-17 work on UE power saving enhancement (see RP-200938). On paging subgrouping, the following provides the detailed agreements for the 2 approaches (CN-assigned subgrouping and UE ID-based subgrouping):

Assume that one subgroup indication refers to either CN-assigned subgroups or UE ID-based subgroup (no overlapping)

Both UE ID-based and CN-assigned  subgrouping can be supported simultaneously in a cell, it is allowed to just support one of them.

The total number of CN-assigned subgroups that is used is not fixed and can be configured up to 8 (e.g. by OAM). No impact on signalling is assumed.

RAN introduces a new parameter Nsg-UEID to indicate its support of UE ID-based subgrouping.

RAN does not support any type of subgrouping if its configuration for subgrouping is either absent or nullified (e.g. subgroupsNumPerPO is either absent or set to zero). FFS for the signalling details.

We assume separate indications for UE capability of CN-assigned subgrouping and UE ID-based subgrouping.

UE’s capability of supporting the UE ID-based subgrouping is reported to RAN by AS UE capability signalling while R2 assumes that UE’s capability of supporting the CN-assigned subgrouping is reported to CN by NAS signalling.

RAN2 also discussed the Paging Early Indication, and agreed to the following:

RAN2 assumes that if PEI is detected, and the PEI indicates that the UE has to monitor the associated PO, then the UE monitors paging DCI in the associated PO, including scheduling information for paging PDSCH (if included) as in legacy. This assumption may be updated based on RAN1 agreements.

As a baseline RAN2 has a preference to support PEI with both DRX and eDRX, but potential issues (e.g. PEI and PTW) are FFS.

For UE ID-based subgroups the UE identity is UE_ID = 5G-S-TMSI mod X, where X is 8192 (1024*8).

Introduce a UERadioPagingInfo IE in the UECapabilityInformation message in NR in Rel-17.

If the UE was not able to monitor the PEI occasion corresponding to its PO, the UE shall monitor the PO.

2. Actions:

To SA2 and CT1:

RAN2 respectfully asks SA2 and CT1 to take the above information into account for their future work, and provide further information on the following issue once concluded:

- NAS signalling between AMF and UE to convey the related information to the UE, e.g., UE’s capability reporting by NAS.

To RAN3:

RAN2 respectfully asks RAN3 to take the above information into account for their future work, and provide further information on the following issue once concluded:

- Signalling between AMF and gNB(s) to inform gNB(s) about the related subgroup information for paging a UE in RRC_IDLE/RRC_INACTIVE, e.g., to convey Subgroup ID, UE capability, etc.

To RAN1:

RAN2 respectfully asks RAN1 to take the above information into account for their future work.

3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #116bis   17 January – 25 January 2022	online meeting

TSG RAN WG2 Meeting #117        21 February – 3 March 2022	             online meeting