3GPP TSG-RAN WG3#114bis-e                                             	                     R3-220194

E-Meeting, 17 – 26 January 2022

Agenda item:	11.3

Source:	Nokia, Nokia Shanghai Bell

Title: 	(TP for TS 38.473) Support of eDRX for Redcap UEs

Document for:	Approval

# Introduction

The following WID was agreed with common parts between RAN2 and RAN3:

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]:

  - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle

  - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4.

  - RAN2 to decide which Node(s) configure eDRX in RRC_Idle and RRC_Inactive.

At the last RAN3 meeting the following was agreed:

- Agree to introduce Paging eDRX Information IE into NGAP Paging message with only one new idle eDRX Cycle IE.

- WA: this IE is for NR and not only Redcap (take NR and add editor’s note in BL CR).

- Agree to add the same Paging eDRX Information IE with the same one new idle eDRX Cycle IE in the NGAP Core Network Assistance Information IE.

- Agree to add a new Redcap Indicator IE into the NGAP Initial UE Message message.

- Agree to add a Paging eDRX Information IE with one new eDRX cycle into XnAP Paging message.

- Agree to add NR Redcap Indicator to the F1 Initial UL Message Transfer.

- Agree R3-216117, BL CR NGAP (Nokia, revision of 6051).

- Agree R3-216118, BL CR XnAP (Qualcomm, revision of 6108).

- Agree R3-216119, BL CR 38.300 (Ericsson, revision of 6080).

- Agree R3-216115, BL CR F1AP (Samsung).

This paper continues the discussion and proposes further agreements and TP for the baseline CR for TS38.473.

# Conclusion and Proposals

This paper has made some further analysis of the remaining open points concerning the Work Item of Reduced Capability for UEs taking into account recent decisions in other groups and makes following proposals.

Proposal 1: agree to turn the following working assumption into agreement:

The Paging eDRX Information IE in the NGAP Paging message with only one new idle eDRX Cycle IE is for NR and not only Redcap. Remove the editor’s note in BL CR).

Proposal 2: introduce a new IE over Xn for RRC_INACTIVE eDRX.

Proposal 3: add a new eDRX cycle for both idle and inactive over F1.

Proposal 4: need of assistance information to exchange between RAN and CN is not confirmed so far.

It is proposed to agree the baseline TP below for TS 38.473.

# TP for TS 38.473

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The NR Paging eDRX Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

Next change

#### 9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| Paging Origin | O |  | 9.3.1.79 |  | YES | ignore |  |
| NR Paging eDRX Information | O |  | 9.3.1.x |  | YES | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

Next change

#### 9.3.1.X	NR Paging eDRX Information

This IE indicates the NR Paging eDRX parameters for NR UE as defined in TS 38.304 [24].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR Paging eDRX Cycle | M |  | ENUMERATED (hfquarter, hfhalf, hf1, hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, hf512, hf1024, …) | TeDRX defined in TS 38.304 [24]. Unit: [number of hyperframes]. |
| NR Paging Time Window | O |  | ENUMERATED (s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, …) | Unit: [1.28 second]. |