3GPP TSG-RAN WG3 #114bis-e																	 R3-220219

17-26 Jan 2022

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1851 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-USIM BLCR to TS 36.413 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM |  |  |  |  |  | Date: |  |  | 2022-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Support of Multi-SIM |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Support of Multi-SIM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-SIM cannot be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.6, 9.2.1.3, 9.2.1.xx (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

9.1.6	PAGING

This message is sent by the MME and is used to page a UE in one or several tracking areas.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.2.3.10 |  | YES | ignore |
| UE Paging Identity | M |  | 9.2.3.13 |  | YES | ignore |
| Paging DRX | O |  | 9.2.1.16 | If the NB-IoT Paging DRX IE is included, the Paging DRX IE is ignored. | YES | ignore |
| CN Domain | M |  | 9.2.3.22 |  | YES | ignore |
| List of TAIs |  | 1 |  |  | YES | ignore |
| >TAI List Item |  | 1 .. <maxnoofTAIs> |  |  | EACH | ignore |
| >>TAI | M |  | 9.2.3.16 |  | - |  |
| CSG Id List |  | 0..1 |  |  | GLOBAL | ignore |
| >CSG Id |  | 1 .. <maxnoofCSGId> | 9.2.1.62 |  | - |  |
| Paging Priority | O |  | 9.2.1.78 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.2.1.98 |  | YES | ignore |
| Assistance Data for Paging | O |  | 9.2.1.103 |  | YES | ignore |
| Paging eDRX Information | O |  | 9.2.1.111 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.2.3.46 |  | YES | ignore |
| NB-IoT Paging eDRX Information | O |  | 9.2.1.115 |  | YES | ignore |
| NB-IoT UE Identity Index value | O |  | 9.2.3.47 |  | YES | ignore |
| Enhanced Coverage Restricted | O |  | 9.2.1.123 |  | YES | ignore |
| CE-Mode-B Restricted | O |  | 9.2.1.129 |  | YES | ignore |
| Data size | O |  | INTEGER (1..4095, …) | The unit is: bit | YES | ignore |
| WUS Assistance Information | O |  | 9.2.1.158 |  | YES | ignore |
| NB-IoT Paging DRX | O |  | 9.2.1.159x |  | YES | ignore |
| PagingRecord | O |  | 9.2.1.xx |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAIs | Maximum no. of TAIs. Value is 256. |
| maxnoofCSGIds | Maximum no. of CSG Ids within the CSG Id List. Value is 256. |

9.2.1.3	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the S1AP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, TX2RELOCOverall Expiry, Successful Handover, Release due to E-UTRAN Generated Reason, Handover Cancelled, Partial Handover, Handover Failure In Target EPC/eNB Or Target System, Handover Target not allowed, TS1RELOCoverall Expiry, TS1RELOCprep Expiry, Cell not available, Unknown Target ID, No Radio Resources Available in Target Cell, Unknown or already allocated MME UE S1AP ID, Unknown or already allocated eNB UE S1AP ID, Unknown or inconsistent pair of UE S1AP ID, Handover desirable for radio reasons, Time critical handover, Resource optimisation handover, Reduce load in serving cell, User inactivity, Radio Connection With UE Lost, Load Balancing TAU Required, CS Fallback Triggered, UE Not Available For PS Service, Radio resources not available, Failure in the Radio Interface Procedure, Invalid QoS combination, Inter-RAT redirection, Interaction with other procedure, Unknown E-RAB ID, Multiple E-RAB ID instances, Encryption and/or integrity protection algorithms not supported, S1 intra system Handover triggered, S1 inter system Handover triggered, X2 Handover triggered …, Redirection towards 1xRTT, Not supported QCI value, invalid CSG Id, Release due to Pre-Emption, N26 interface not available, Insufficient UE Capabilities, Maximum bearer pre-emption rate exceeded, Reject Paging) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, …) |  |
| >NAS |  |  |  |  |
| >>NAS Cause | M |  | ENUMERATED (Normal Release, Authentication failure, Detach, Unspecified, …, CSG Subscription Expiry) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Control Processing Overload, Not enough User Plane Processing Resources, Hardware Failure, O&M Intervention, Unspecified, Unknown PLMN, …) |  |

The meaning of the different cause values is described in the following table. In general, “not supported” cause values indicate that the related capability is missing. On the other hand, “not available” cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause |  | Meaning |
| --- | --- | --- |
| Unspecified |  | Sent for radio network layer cause when none of the specified cause values applies. |
| TX2RELOCOverall Expiry |  | The timer guarding the handover that takes place over X2 has abnormally expired. |
| Successful Handover |  | Successful handover. |
| Release due to E-UTRAN generated reason |  | Release is initiated due to E-UTRAN generated reason. |
| Handover Cancelled |  | The reason for the action is cancellation of Handover. |
| Partial Handover |  | Provides a reason for the handover cancellation. The HANDOVER COMMAND message from MME contained E-RABs to Release List IE and the source eNB estimated service continuity for the UE would be better by not proceeding with handover towards this particular target eNB. |
| Handover Failure In Target EPC/eNB Or Target System |  | The handover failed due to a failure in target EPC/eNB or target system. |
| Handover Target not allowed |  | Handover to the indicated target cell is not allowed for the UE in question. |
| TS1RELOCoverall Expiry |  | The reason for the action is expiry of timer TS1RELOCoverall. |
| TS1RELOCprep Expiry |  | Handover Preparation procedure is cancelled when timer TS1RELOCprep expires. |
| Cell not available |  | The concerned cell is not available. |
| Unknown Target ID |  | Handover rejected because the target ID is not known to the EPC. |
| No radio resources available in target cell |  | Load on target cell is too high. |
| Unknown or already allocated MME UE S1AP ID |  | The action failed because the MME UE S1AP ID is either unknown, or (for a first message received at the eNB) is known and already allocated to an existing context. |
| Unknown or already allocated eNB UE S1AP ID |  | The action failed because the eNB UE S1AP ID is either unknown, or (for a first message received at the MME) is known and already allocated to an existing context. |
| Unknown or inconsistent pair of UE S1AP ID |  | The action failed because both UE S1AP IDs are unknown, or are known but do not define a single UE context. |
| Handover Desirable for Radio Reasons |  | The reason for requesting handover is radio related. |
| Time Critical Handover |  | Handover is requested for time critical reason i.e., this cause value is reserved to represent all critical cases where the connection is likely to be dropped if handover is not performed. |
| Resource Optimisation Handover |  | The reason for requesting handover is to improve the load distribution with the neighbour cells. |
| Reduce Load in Serving Cell |  | Load on serving cell needs to be reduced. When applied to handover preparation, it indicates the handover is triggered due to load balancing. |
| User Inactivity |  | The action is requested due to user inactivity on all E-RABs, e.g., S1 is requested to be released in order to optimise the radio resources. |
| Radio Connection With UE Lost |  | The action is requested due to losing the radio connection to the UE. |
| Load Balancing TAU Required |  | The action is requested for all load balancing and offload cases in the MME. |
| CS Fallback triggered |  | The action is due to a CS fallback that has been triggered. When it is included in UE CONTEXT RELEASE REQUEST message, it indicates the PS service suspension is not required in the EPC. |
| UE Not Available for PS Service |  | The action is requested due to a CS fallback to GERAN that has been triggered. When it is included in the UE CONTEXT RELEASE REQUEST message, it indicates that the PS service suspension is required in the EPC due to the target GERAN cell or the UE has no DTM capability. |
| Radio resources not available |  | No requested radio resources are available. |
| Invalid QoS combination |  | The action was failed because of invalid QoS combination. |
| Inter-RAT Redirection |  | The release is requested due to inter-RAT redirection or intra-LTE redirection. When it is included in UE CONTEXT RELEASE REQUEST message, the behaviour of the EPC is specified in TS 23.401 [11]. |
| Failure in the Radio Interface Procedure |  | Radio interface procedure has failed. |
| Interaction with other procedure |  | The action is due to an ongoing interaction with another procedure. |
| Unknown E-RAB ID |  | The action failed because the E-RAB ID is unknown in the eNB. |
| Multiple E-RAB ID Instances |  | The action failed because multiple instance of the same E-RAB had been provided to the eNB. |
| Encryption and/or integrity protection algorithms not supported |  | The eNB is unable to support any of the encryption and/or integrity protection algorithms supported by the UE. |
| S1 Intra system Handover triggered | The action is due to a S1 intra system handover that has been triggered. |  |
| S1 Inter system Handover triggered | The action is due to a S1 inter system handover that has been triggered. |  |
| X2 Handover triggered | The action is due to an X2 handover that has been triggered. |  |
| Redirection towards 1xRTT | The release of the UE-associated logical S1 connection is requested due to redirection towards a 1xRTT system e.g., CS fallback to 1xRTT, or SRVCC to 1xRTT, when the PS service suspension is required in the EPC. During this procedure, the radio interface message might but need not include redirection information. |  |
| Not supported QCI Value | The E-RAB setup failed because the requested QCI is not supported. |  |
| Invalid CSG Id | The CSG ID provided to the target eNB was found invalid. |  |
| Release due to Pre-Emption | Release is initiated due to pre-emption. |  |
| N26 interface not available | The action failed due to a temporary failure of the N26 interface. |  |
| Insufficient UE Capabilities | The procedure can’t proceed due to insufficient UE capabilities. |  |
| Maximum bearer pre-emption rate exceeded | The procedure can’t proceed because the number of requests exceed the maximum bearer pre-emption rate. |  |
| Reject Paging | The action failed because the UE is an MultiSIM device and its response in the Service Request includes a Reject Paging Indication |  |

| Transport Layer cause | Meaning |
| --- | --- |
| Transport Resource Unavailable | The required transport resources are not available. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

9.2.1.xx	PagingRecord

This information element indicates the paging cause for paging a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Paging Cause | O |  | ENUMERATED (voice, …) | Indicates whether the Paging message is originated due to IMS voice. If this field is present, it implies that the corresponding paging entry is for IMS voice. If this field is not present but PagingRecord is presen, it implies that the corresponding paging entry is for a service other than IMS voice. |

ANS.1 Part

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

Cause,

<Skip unchanged part>

ENB-EarlyStatusTransfer-TransparentContainer,

WUS-Assistance-Information,

NB-IoT-PagingDRX,

PagingRecord

FROM S1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

S1AP-PRIVATE-IES,

S1AP-PROTOCOL-EXTENSION,

S1AP-PROTOCOL-IES,

S1AP-PROTOCOL-IES-PAIR

FROM S1AP-Containers

id-AssistanceDataForPaging,

id-AerialUEsubscriptionInformation,

id-uEaggregateMaximumBitrate,

id-BearerType,

<Skip unchanged part>

id-WUS-Assistance-Information,

id-NB-IoT-PagingDRX,

id-UERadioCapabilityForPaging-NR-Format,

id-PagingRecord

<Skip unchanged part>

-- **************************************************************

--

##### -- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{PagingIEs}},

...

}

PagingIEs S1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue			CRITICALITY ignore	TYPE UEIdentityIndexValue				PRESENCE mandatory}|

{ ID id-UEPagingID						CRITICALITY ignore	TYPE UEPagingID							PRESENCE mandatory}|

{ ID id-pagingDRX						CRITICALITY ignore	TYPE PagingDRX							PRESENCE optional}|

{ ID id-CNDomain						CRITICALITY ignore	TYPE CNDomain							PRESENCE mandatory}|

{ ID id-TAIList							CRITICALITY ignore	TYPE TAIList							PRESENCE mandatory}|

{ ID id-CSG-IdList						CRITICALITY ignore	TYPE CSG-IdList							PRESENCE optional}|

{ ID id-PagingPriority					CRITICALITY ignore	TYPE PagingPriority						PRESENCE optional}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging			PRESENCE optional}|

-- Extension for Release 13 to support Paging Optimisation and Coverage Enhancement paging –-

{ ID id-AssistanceDataForPaging			CRITICALITY ignore	TYPE AssistanceDataForPaging			PRESENCE optional}|

{ ID id-Paging-eDRXInformation			CRITICALITY ignore	TYPE Paging-eDRXInformation				PRESENCE optional}|

{ ID id-extended-UEIdentityIndexValue	CRITICALITY ignore	TYPE Extended-UEIdentityIndexValue	PRESENCE optional}|

{ ID id-NB-IoT-Paging-eDRXInformation	CRITICALITY ignore	TYPE NB-IoT-Paging-eDRXInformation	PRESENCE optional}|

{ ID id-NB-IoT-UEIdentityIndexValue		CRITICALITY ignore	TYPE NB-IoT-UEIdentityIndexValue		PRESENCE optional}|

{ ID id-EnhancedCoverageRestricted		CRITICALITY ignore	TYPE EnhancedCoverageRestricted			PRESENCE optional}|

{ ID id-CE-ModeBRestricted				CRITICALITY ignore	TYPE CE-ModeBRestricted					PRESENCE optional}|

{ ID id-DataSize						CRITICALITY ignore	TYPE DataSize							PRESENCE optional}|

{ ID id-WUS-Assistance-Information		CRITICALITY ignore	TYPE WUS-Assistance-Information			PRESENCE optional}|

{ ID id-NB-IoT-PagingDRX				CRITICALITY ignore	TYPE NB-IoT-PagingDRX					PRESENCE optional}|

{ ID id-PagingRecord					CRITICALITY ignore	TYPE PagingRecord						PRESENCE optional},

...

}

<Skip unchanged part>

9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Skip unchanged part>

#### -- C

<Skip unchanged part>

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

transport			CauseTransport,

nas					CauseNas,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseMisc ::= ENUMERATED {

control-processing-overload,

not-enough-user-plane-processing-resources,

hardware-failure,

om-intervention,

unspecified,

unknown-PLMN,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

abstract-syntax-error-falsely-constructed-message,

unspecified,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

tx2relocoverall-expiry,

successful-handover,

release-due-to-eutran-generated-reason,

handover-cancelled,

partial-handover,

ho-failure-in-target-EPC-eNB-or-target-system,

ho-target-not-allowed,

tS1relocoverall-expiry,

tS1relocprep-expiry,

cell-not-available,

unknown-targetID,

no-radio-resources-available-in-target-cell,

unknown-mme-ue-s1ap-id,

unknown-enb-ue-s1ap-id,

unknown-pair-ue-s1ap-id,

handover-desirable-for-radio-reason,

time-critical-handover,

resource-optimisation-handover,

reduce-load-in-serving-cell,

user-inactivity,

radio-connection-with-ue-lost,

load-balancing-tau-required,

cs-fallback-triggered,

ue-not-available-for-ps-service,

radio-resources-not-available,

failure-in-radio-interface-procedure,

invalid-qos-combination,

interrat-redirection,

interaction-with-other-procedure,

unknown-E-RAB-ID,

multiple-E-RAB-ID-instances,

encryption-and-or-integrity-protection-algorithms-not-supported,

s1-intra-system-handover-triggered,

s1-inter-system-handover-triggered,

x2-handover-triggered,

...,

redirection-towards-1xRTT,

not-supported-QCI-value,

invalid-CSG-Id,

release-due-to-pre-emption,

n26-interface-not-available,

insufficient-ue-capabilities,

maximum-bearer-pre-emption-rate-exceeded,

rejectPaging

}

<Skip unchanged part>

#### -- P

Packet-LossRate	::= INTEGER(0..1000)

PagingAttemptInformation ::= SEQUENCE {

pagingAttemptCount					PagingAttemptCount,

intendedNumberOfPagingAttempts		IntendedNumberOfPagingAttempts,

nextPagingAreaScope					NextPagingAreaScope		OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { PagingAttemptInformation-ExtIEs} } OPTIONAL,

...

}

PagingAttemptInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

PagingAttemptCount ::= INTEGER (1..16, ...)

Paging-eDRXInformation ::= SEQUENCE {

paging-eDRX-Cycle			Paging-eDRX-Cycle,

pagingTimeWindow			PagingTimeWindow			OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { Paging-eDRXInformation-ExtIEs} } OPTIONAL,

...

}

Paging-eDRXInformation-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

Paging-eDRX-Cycle ::= ENUMERATED{hfhalf, hf1, hf2, hf4, hf6, hf8, hf10, hf12, hf14, hf16, hf32, hf64, hf128, hf256, ...}

PagingTimeWindow ::= ENUMERATED{s1, s2, s3, s4, s5, s6, s7, s8, s9, s10, s11, s12, s13, s14, s15, s16, ...}

PagingDRX ::= ENUMERATED {

v32,

v64,

v128,

v256,

...

}

PagingPriority ::= ENUMERATED {

priolevel1,

priolevel2,

priolevel3,

priolevel4,

priolevel5,

priolevel6,

priolevel7,

priolevel8,

...

}

PagingProbabilityInformation ::= ENUMERATED {p00, p05, p10, p15, p20, p25, p30, p35, p40, p45, p50, p55, p60, p65, p70, p75, p80, p85, p90, p95, p100, ...}

PagingRecord::= SEQUENCE {

pagingCause-Multi-SIM ::= 	ENUMERATED {voice, ...} OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { PagingRecord-ExtIEs} }	OPTIONAL,

...

}

PagingRecord-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

<Skip unchanged part>

9.3.6	Constant Definitions

<Skip unchanged part>

--

#### -- IEs

--

-- **************************************************************

id-MME-UE-S1AP-ID									ProtocolIE-ID ::= 0

id-HandoverType										ProtocolIE-ID ::= 1

id-Cause											ProtocolIE-ID ::= 2

<Skip unchanged part>

id-WUS-Assistance-Information						ProtocolIE-ID ::= 323

id-NB-IoT-PagingDRX									ProtocolIE-ID ::= 324

id-TraceCollectionEntityURI							ProtocolIE-ID ::= 325

id-EmergencyIndicator								ProtocolIE-ID ::= 326

id-UERadioCapabilityForPaging-NR-Format 			ProtocolIE-ID ::= 327

id-PagingRecord										ProtocolIE-ID ::= xxx

END