3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220238

# E-meeting, 17th – 26th January, 2022

Online

Agenda Item:	23.2

Source:	CATT

Title:	Discussion on NR ProSe Authorization

Document for:	Discussion

Introduction

This contribution discusses NR ProSe Authorization based on the LSin from RAN2 [1] and agreements in RAN3#114-e [2].

Discussion

An LS was sent from RAN2 to RAN3 [1] which is about whether authorisation information for L3 remote UE is needed for NG-RAN.

Q4) Per TS 23.304 clause 6.6.2, NG-RAN is provided with 5G ProSe authorised information indicating whether a UE is authorized to use 5G ProSe Direct Discovery, 5G ProSe Direct Communication, to act as a 5G ProSe Layer-2 UE-to-Network Relay, a 5G ProSe Layer-3 UE-to-Network Relay and a 5G ProSe Layer-2 Remote UE. NG-RAN is not provided with authorisation information for whether a UE is authorised to act as a 5G ProSe Layer-3 Remote UE.

Is the authorisation information for whether a UE can act as a 5G ProSe Layer-3 Remote UE needed by NG-RAN to enable configuring the UE with correct discovery configuration information via dedicated signalling?

[Answer]:

Whether authorization information for L3 remote UE is needed for NG-RAN can be decided by RAN3. The full set of relay discovery agreements is listed in Appendix for reference.

RAN2 has the following agreements shown in [1]:

Relay UE and remote UE (IC) in RRC CONNECTED can use the discovery configuration provided via dedicated signalling if available.

WA: L3 relay UE uses pre-configuration for discovery, only if the discovery SIB configuration is not provided by gNB, in case its serving carrier is not shared with carrier for sidelink operation. Otherwise, L3 relay UE uses the configuration for discovery provided by gNB.

For RRC CONNECTED L3 remote UE, if mode 1 is configured by network, it requests mode 1 resource for relay discovery from network. As describe in [3], network is able to receive NR ProSe authorized information from CN. Hence network can allocate resource for L3 remote UE based on NR ProSe Direct Discovery IE included in NR ProSe authorized information.

Currently, we do not identify any requirement from RAN2 and SA2 to introduce NR ProSe Layer-3 Remote UE in NR ProSe authorized information. Also, authorization information for L3 remote UE was not contained in LTE. Therefore, we consider that 5G ProSe Layer-3 Remote UE does no need to be introduced to NR ProSe authorized information in R17 unless RAN2 and SA2 have further requirement.

Proposal 1: Do not introduce NR ProSe Layer-3 Remote UE in NR ProSe authorized information unless RAN2 and SA2 have further requirement.

Based on the above discussion, a draft LS is provided in [4].

Proposal 2: RAN3 agrees the reply LS R3-220239 to RAN2 and SA2.

According to the baseline CR from SA2 [3] and agreements achieved in last RAN3 meeting [2], NR ProSe authorized information includes NR ProSe Direct Discovery, NR ProSe Direct Communication, NR ProSe Layer-2 UE-to-Network Relay, NR ProSe Layer-3 UE-to-Network Relay and NR ProSe Layer-2 Remote UE shall be added as optional IEs in the following messages.

- NGAP messages: INITIAL CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUES, HANDOVER REQUEST, PATH SWITCH REQUEST ACKNOWLEDGE;

- XnAP messages: HANDOVER REQUEST, RETRIEVE UE CONTEXT RESPONSE;

- F1AP messages: UE CONTEXT SETUP REQUEST, UE CONTEXT MODIFICATION REQUEST.

The relevant CRs for F1AP and XnAP are in [5] [6].

Proposal 3: RAN3 agrees CR for F1AP (R3-220240) and XnAP (R3-220241).

Conclusion

Proposal 1: Do not introduce NR ProSe Layer-3 Remote UE in NR ProSe authorized information unless RAN2 and SA2 have further requirement.

Proposal 2: RAN3 agrees the reply LS R3-220239 to RAN2 and SA2.

Proposal 3: RAN3 agrees CR for F1AP (R3-220240) and XnAP (R3-220241).

References

[1] R3-220137, Reply LS on discovery and relay (re)selection, CATT.

[2] RAN3#114-e, Chair notes.

[3] TS 23.304 Technical Specification Group Services and System Aspects; Proximity based Services (ProSe) in the 5G System (5GS).

[4] R3-220239, Draft reply LS for authorisation information for 5G ProSe Layer-3 Remote UE is needed by NG-RAN, CATT.

[5] R3-220240, Support of 5G ProSe Authorization for XnAP, CATT.

[6] R3-220241, Support of 5G ProSe Authorization for F1AP, CATT.