3GPP TSG-RAN WG3 #114bis-e						R3-220244

E-meeting, 17– 26 January 2022

Agenda Item:  14.2

Source:       NEC

Title:		SCG Activation / Deactivation open issues discussion

Document for:	dicision

- Introduction

For the SCG activation/deactivation aspect, this contribution discussed the remaining issues and give some proposal to progress the work, namely

FFS whether SN can reject the SCG activation when accepting SN addition request during SN addition.

FFS on the exact value(s) for the new cause value(s).

FFS whether/how to enhance the E1 and F1 inactivity notification procedure.

FFS on the IE name in the SN Modification Required message.

- 2. Discussion

Discussion on “FFS whether SN can reject the SCG activation when accepting SN addition request during SN addition.”

This issue has been discussed several time, both camps stay the same position. We are in the understanding that there is no point for the SN to accept the SN Addition procedure while reject the SCG activation. This is because the MN when decide to add SN, is to offload its traffic, if SN cannot serve it can simply just reject the whole procedure, as the currently legacy way has already done.

Therefore, again;

- For X2/Xn: SN Addition Request, if the activation or deactivation request of SCG is not accepted, the SN shall reject the whole procedure by sending back the SN Addition Request Reject message.

- For F1 UE Context Setup Request, if the activation or deactivation request of SCG is not accepted, the gNB-DU shall reject the whole procedure by sending back the UE Context Setup Failure message.

There will be no protocol impact on the SN addition procedure, except to add a new cause value to indicate the SN addition rejection.

Proposal 1: Partial rejection for SCG (de)activation during SN addition is NOT supported.

Discussion on “FFS on the exact value(s) for the new cause value(s).”

This issue is, when the SN reject the SCG activation/deactivation request, what cause value (general cause value or specific cause value) will be needed.

- General cause value e.g. “Failure due to SCG activation/deactivation”

- Specific cause value e.g. “Requested SCG Activation Status not available due to UE overheating”, “Requested SCG Activation Status not available due to expected new data”, “Failure SCG Activation due to xxx”, “Failure SCG Deactivation due to data on-going”, etc.

It can be seen that if to add specific cause value then there may be as many as possible as many reason can be assumed. Therefore a general cause value should be enough.

Nevertheless, one probably obvious or need to be known by the MN is for the failure due to data still on-going in SN-Terminated SCG bearer, then MN may not try again to ask for SCG deactivation. Then if need to, only one specific cause value, will be the “Failure SCG deactivation due to data on-going”.

Proposal 2: RAN3 to specify a general cause value “Failure due to SCG activation/deactivation”, and one specific cause value “Failure SCG Deactivation due to data on-going”.

Discussion on “FFS whether/how to enhance the E1 and F1 inactivity notification procedure.”

E1: assistant information from UP to CP for inactivity notification

Use of E1AP: BEARER CONTEXT INACTIVITY NOTIFICATION message.

As the BEARER CONTEXT INACTIVITY NOTIFICATION message can have a choice to choose UE level activity, this can be reused. Only one condition is that, it has to be indicated by the CU-CP in the BEARER CONTEXT SETUP REQUEST message as “UE” in the Activity Notification Level IE.

This is no further issue, because if the CU-CP wish to have assistant information from UP for the SCG deactivation purpose, then it should ask for UE level activity notification.

There is another way to use e.g. BEARER CONTEXT MODIFICATION REQUIRED message from UP to CP, however using of this message may not be called as “assistance information”, as it is rather “required” for SCG deactivation.

It is general understood that the SCG deactivation is controlled in the gNB-CU-CP, with collecting assistant information from gNB-CU-UP and gNB-DU, rather than required by them. Therefore we will propose:

Proposal 3: For the assistant information purpose to collect the activity status, it is proposed to use the E1AP: BEARER CONTEXT INACTIVITY NOTIFICATION message as it is without further enhancement.

F1: assistant information from DU to CU for inactivity notification

Use of F1AP:UE INACTIVITY NOTIFICATION message.

It is general understood that the SCG deactivation is controlled in the gNB-DU, with collecting assistant information from gNB-CU-UP and gNB-DU, there it will be simple way to reuse of UE INACTIVITY NOTIFICATION rather than e.g. UE CONTEXT MODIFICATION REQUIRED message.

One possible worry of using F1AP: UE INACTIVITY NOTIFICATION message is that, the UE Inactivity Notification procedure is used by the gNB-CU for decision of sending UE to INACTIVE state, this is not essential as a procedure can be used by multiple purposes if it can fit for the purposes. For this case, the gNB-CU can use this for both, for decision of sending UE to INACTIVE state or for decision to ask for SCG Deactivation, depending on the application. For example, for low power consumption UE it can be used for decision of sending UE to INACTIVE state, for eMBB UE it can be used for asking SCG Deactivation e.g. if UE claim for over-heated.

One more possible concern of using F1AP: UE INACTIVITY NOTIFICATION message is that, the existing DRB Activity List IE is mandatory present, this is also no essential problem as the gNB-DU can set all DRB to “Not active”.

Proposal 4: For the assistant information purpose to collect the activity status, it is proposed to use the F1AP: UE INACTIVITY NOTIFICATION message as it is without further enhancement.

Discussion on “FFS on the IE name in the SN Modification Required message”

This issue is how to name the IE in SN Modification Required message when the SN ask for SCG Activation, whether it should be SCG Activation Request, or it should be SCG Activation Status.

Since the SN is using the SN Modification Required message for requesting the SCG Activation, then why don’t just follow the “Required” in massage name so to have the name of SCG Activation Required IE.

Proposal 5: when the SN ask for SCG Activation, to use the name of SCG Activation Required IE in the SN MODIFICATION REQUIRED message.

Further more

Regarding the “Wait for RAN2 progress before discussing whether gNB-DU can request SCG (de)activation via UE Context Modification Required message.”, since in RAN2#116 has agreed that: (refer to RP-213213 status report)

- SCG activation - RAN2 will not ask RAN3 to allow SCG activation by the SN without MN's response. - Support the following solutions for UL data arrival while the SCG is deactivated: - for split bearers, send the data via the MCG leg. FFS how this can be implemented in Stage-3. - for SCG bearers, the UE indicates via the MCG that it has UL data to send for an SCG bearer. - FFS indication contents and format (e.g. MN RRC message, embedded SN RRC message) - FFS whether this indication can be used for split bearers

At least, there will be no need to specify in F1from the DU of the deactivated SCG to indicate the UE has tried to activate the deactivated SCG.

Since RAN2 is still working on stage 3 detail, for this issue likely no RAN3 impact but can still wait RAN2 to progress.

Proposal 6: for the issue that whether gNB-DU can request SCG activation/deactivation, still wait for RAN2 progress on further stage 3 detail.

- 3. proposal

Proposal 1: Partial rejection for SCG (de)activation during SN addition is NOT supported.

Proposal 2: RAN3 to specify a general cause value “Failure due to SCG activation/deactivation”, and one specific cause value “Failure SCG Deactivation due to data on-going”.

Proposal 3: For the assistant information purpose to collect the activity status, it is proposed to use the E1AP: BEARER CONTEXT INACTIVITY NOTIFICATION message as it is without further enhancement.

Proposal 4: For the assistant information purpose to collect the activity status, it is proposed to use the F1AP: UE INACTIVITY NOTIFICATION message as it is without further enhancement.

Proposal 5: when the SN ask for SCG Activation, to use the name of SCG Activation Required IE in the SN MODIFICATION REQUIRED message.

Proposal 6: for the issue that whether gNB-DU can request SCG activation/deactivation, still wait for RAN2 progress on further stage 3 detail.

References:

R3-215951 Summary of Offline Discussion on MRDC2 - SCG (de)activation

RP-213213 Status Report of Further Multi-RAT Dual-Connectivity enhancements