3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220368

Online, 17 – 26 January 2022

Agenda item:	21.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.423) Time synchronization and handover

Document for:	Discussion and Decision

# 1	Introduction

RAN3 has discussed assistance information that may be useful for the target gNB to maintain timing accuracy required by the UE following handover, and at RAN3#113e the following agreement was captured in the Chair’s minutes:

It is FFS on whether assistance information (e.g., UE TSN timing reference, referenceTimeInfo delivery periodicity, timestamp) should be delivered during HO.

At the last meeting, RAN3 agreed to introduce the Time Synchronisation Assistance Information IE (Time Distribution Indication and Uu Time Synchronisation Error Budget) in the HANDOVER REQUEST message and discussed whether any additional information should be passed to the target gNB during handover as captured in the Summary of Offline [1].

In this paper, we further analyse the RAN3 impact of handover for a UE receiving a time synchronization service and propose a way forward.

# 2	Discussion

When an inter-gNB handover occurs, the target gNB can use the Time Synchronization Assistance Information received from the source gNB via XnAP to determine the delivery configuration of RTI to the UE in the new cell. However, to ensure the time synchronization service is properly maintained when changing the serving gNB, meaning that the Uu time synchronization error budget is not violated following a handover and that radio resources are used efficiently, the source gNB could additionally assist the target gNB in determining the level of urgency/reliability to deliver the first RTI following the handover. That is, the target gNB needs to assess if the current configuration of SIB9 in the cell can fulfil the UE’s time synchronization requirements and its current timing accuracy status, or a potential disruption of the time synchronization service may occur.

During the handover, the holdover capability of the UE’s clock is exploited to maintain the required timing accuracy. Therefore, in addition to the Uu time synchronization error budget provided by the source gNB, the determination of the urgency for the first RTI to be delivered by the target gNB requires the source gNB to provide a timestamp when, at the latest, the next RTI needs to be delivered at the target gNB. That is, from the source gNB perspective, at which time instant the next timing information update was planned to be delivered to the source gNB-DU from the source gNB-CU for the UE (via SIB9 or RRC signalling). To do so, the source gNB-CU implementation takes into account the RTI delivery periodicity it is using for the UE as well as the knowledge of the time instant where the latest RTI was delivered to the source gNB-DU, to compute the timestamp for the next RTI delivery. This timestamp tells the target gNB when, at the latest, the next RTI needs to be delivered to the UE. If the target gNB does not receive this parameter, it cannot determine how urgent the next RTI update is for the UE and the target gNB must always deliver the RTI as soon as possible to avoid time synchronization service disruption.

Apart from ensuring that RTI is delivered to the UE without having violated the Uu time synchronization error budget, the timestamp from the source gNB-CU for the expected next RTI delivery can also be used at the target gNB to decide on the handover configuration parameters e.g. which type of handover and/or RACH procedures to enable/use, in order to appropriately reduce the handover interruption time depending on the current timing accuracy at the UE and its holdover capability.

Proposal:	To support target gNB in determining the level of urgency/reliability to deliver the first RTI following handover, source gNB provides the timestamp when, at the latest, the next RTI needs to be delivered for the UE in the XnAP HANDOVER REQUEST message.

The above proposal can be realized by introducing a Latest Time for Next RTI IE in the Time Synchronisation Assistance Information passed over XnAP, with encoding similar to the Burst Arrival Time IE (i.e. OCTET STRING encoded in the same format as ReferenceTime IE defined in TS 38.331 which has 10ns granularity).

# 3	Conclusions

In this paper, we further analysed RAN3 impacts of handover for a UE receiving a time synchronization service and proposed the following:

Proposal:	To support target gNB in determining the level of urgency/reliability to deliver the first RTI following handover, source gNB provides the timestamp when, at the latest, the next RTI needs to be delivered for the UE in the XnAP HANDOVER REQUEST message.

A TP for the XnAP BL CR is provided in Annex A, where the changes reflecting the above proposal are highlighted in yellow.

# References

- R3-215956 Summary of offline: time synchronization enhancements, Nokia (moderator)

# Annex A: Text Proposal for TS 38.423 BL CR

Start of Text Proposal for TS 38.423 BL CR

#### 9.2.3.aaa	Time Synchronisation Assistance Information

This IE indicates the 5G access stratum time distribution parameters as specified in TS 23.501 [7].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Time Distribution indication | M |  | ENUMERATED (enabled, disabled, …) |  |
| Uu Time Synchronization Error Budget | C-ifEnabled |  | INTEGER (0..1000000, …) | Expressed in units of 1 ns. |
| Latest Time for Next RTI | O |  | OCTET STRING | Latest time for the next ReferenceTimeInfo to be sent toward the UE. Encoded in the same format as the ReferenceTime IE as defined in TS 38.331 [10]. |

| Condition | Explanation |
| --- | --- |
| C-ifEnabled | This IE shall be present if the Time Distribution Indication IE is set to “enabled”. |

Next Change

### 9.3.5	Information Element Definitions

** SKIPPING UNCHANGED TEXT **

TimeSynchronizationAssistanceInformation ::= SEQUENCE {

timeDistributionIndication					ENUMERATED {enabled, disabled, ...},

uuTimeSynchronizationErrorBudget			INTEGER (0..1000000, ...)				OPTIONAL,

-- The above IE shall be present if the Time Distribution Indication IE is set to the value “enabled”

latestTimeForNextRTI						OCTET STRING							OPTIONAL,

ie-Extension								ProtocolExtensionContainer { { TimeSynchronizationAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

TimeSynchronizationAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

End of Text Proposal for TS 38.423 BL CR