3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220427

E-meeting, 17-26 Jan 2022

Title: 	(TP to RA-SDT BL CR of TS 38.300) CCCH solution for UL non-SDT arrival and ROHC continuity aspects

Source: 	Huawei

Agenda item:	24.4

Document Type:	other

# 1. Introduction

In RAN3#114bis meeting, the following working assumptions were achieved on the switch from SDT to non-SDT:

WA: for the switch from SDT to non SDT,

  - Way 1 (FFS for Way 2) is applied for SDT with anchor relocation

  - Way 2 (FFS for Way 1) is applied for SDT without anchor relocation

where

- Way 1: sending RRCResume message during SDT session

- Way 2: sending RRCRelease message to end SDT session first, and then another RRC Resume procedure is triggered for anchor relocation

WA: when applying Way 1 for SDT with anchor relocation, no RAN3 stage 3 impact is identified

WA: when applying Way 2 for SDT without anchor relocation, RAN3 assumes the anchor could move the UE back to RRC Inactive by using RRCRelease message.

In RAN2#116 meeting, RAN2 discussed DCCH vs CCCH signalling triggered for the switch from SDT to non-SDT and no consensus has been reached.

In RAN2#116 meeting, RAN2 discussed ROHC continuity function for SDT DRB  the following agreement on the ROHC continuity were achieved on SDT and send an LS to RAN3 in [4]:

Agreement: - For SDT, ROHC continuity functionality can be configurable between the cell and RNA. Send LS to RAN3

In this document, we further analyses how to support this CCCH solution in case of UL non-SDT data arrival, the ROCH continuity aspects, and provide the corresponding TPs to the BL CRs to support the CCCH solution.

# 2. Discussion

## 2.1 CCCH solution in case of UL non-SDT data arrival

Upon non-SDT data arrival during the SDT procedure, UE triggers the second RRC connection Resume procedure along with the key updating and key derivation behaviour, which (i.e. option 6.e/6.a) is captured in the draft RRC CR in [1], as simply described as following.

#### TS 38.331/5.3.13.2 Initiation Upon initiation of the procedure, the UE shall: 2> else: 3> if Txxx(NewSDTTimer) is running: 4> suspend all SRB(s) and DRB(s), except SRB0; 4> indicate PDCP suspend to lower layers of all DRBs; 4> replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys in the stored UE Inactive AS context;

During the last RAN2 meeting discussion, seeing no critical issue is foreseen on the CCCH based solution.

- The CCCH based solution i.e. option 6.e/6.a captured in draft CR part in [1] is feasible from RAN2 point of view. The CCCH based solution, i.e. Upon non-SDT data arrives during the SDT procedure, UE triggers the second RRC Resume Request procedure along with suspending all SRBs and DRBs execpt the SRB0 and updating the current KgNB and KRRCint to the UE Inactive AS context.

Next, we give RAN3 impact analysis based on the above CCCH solution.

Figure 1 Non-SDT data indication using second RRC Resume Request

According to RAN2 majority understanding, the period RNA update procedure is not triggered during the SDT procedure. Therefore, here we assume no period RNA Update is triggered during the SDT session. This detailed procedure is depicted in Figure 1:

Step 1~10. The normal SDT procedure.

Step 11. Non-SDT data arrives.

Step 12. UE initiates the 2nd RRC Resume procedure, including updating the current KgNB*_1 and KRRCint_1 to the UE context, and calculates the short MAC-I with the latest updated KRRCint_1 and derive new KgNB*2 and the corresponding new RRC and new UP key.

Step 13. UE send 2nd RRC Resume Request message to the new serving gNB.

Step 14~15. Upon reception of the 2nd legacy XnAP: RETRIEVE UE CONTEXT REQUEST message of the same UE before ending of the SDT, the anchor gNB could:

- determine the 2nd RRC connection Resume procedure is triggered for the non-SDT arrival by the same UE based on the same I-RNTI received;

- updates the current KgNB*_1 and KRRCint_1 to the local UE context,

- calculates the short MAC-I with the latest updated KRRCint_1 in the local UE context to verify the short MAC-I from the UE;

- derive new KgNB*2 and the corresponding new RRC and new UP key like UE.

Step 16. If the verification is successfully performed, the anchor gNB generates the RRC RELEASE message to indicate UE into the RRC_INACTVE. The RRC RELEASE message is protected by the new RRC key and included in the legacy XnAP: RETRIEVE UE CONEXT FAILURE message.

- Assuming the the period RNA update procedure is not triggered during the SDT procedure, no impcat on XnAP is foressen about the CCCH based solution:

- The new serving gNB send the 2nd XnAP: RETRIEVE UE CONTEXT REQUEST to the anchor gNB in case of the 2nd RRC Resume Request is received from the UE.

- The anchor gNB updates the current KgNB and KRRCint to the local UE context and verify the short MAC-I  from UE based on the latest updated KRRCint, and dervive the new key for the RRC RRC RELEASE message protection.

In the CU-DU split case, new serving gNB-DU can send the RRC Resume Request message in the legacy F1AP: UL RRC MESSAGE TRANSFER message.

- No impcat on F1AP is foressen in CU-DU split case to support the CCCH based solution:

The new serving gNB-DU can send the RRC Resume Request message from UE via the legacy F1AP: INITIAL UL RRC MESSAGE TRANSFER message.

- Send an LS to RAN2 that the CCCH solution (i.e. option 6.e/6.a in [1]) for the switch from non-SDT to SDT is feasible from RAN3 point of view, and no RAN3 stage 3 impact is foreseen.

## 2.2 Cell based ROHC continuity

If the cell based ROHC continuity is configured for the SDT DRB via the RRC Release message, it means that the UE can continue using the ROHC context to compress/decompress the IP header of the data for SDT DRB in the same cell where the RRC connection is suspended. If the UE moves to another cell and initiates SDT, the UE should reset the ROHC context. And the anchor gNB finds that the UE is accessing from another cell, the anchor gNB also reset the ROHC context. Therefore, UE and the anchor gNB can synchronize the ROHC context both with anchor relocation and without anchor relocation cases and No RAN3 impact is foreseen in the cell based ROHC continuity.

- No RAN3 impact is foreseen to support the cell based ROHC continuity for SDT DRB.

## 2.2 RNA based ROHC continuity

If the RNA based ROHC continuity is configured for the SDT DRB via the RRC Release message, it means that the UE can continue using the ROHC context to compress/decompress the IP header of the data for SDT DRB in the cells within the same RNA. One RAN Notification Area (RNA) can cover cells within one or multiple gNBs, which is up to the network deployment. Here we assume the one RNA covers more than one gNB as an example.

- Case 1: UE initiates the SDT in the new cell under the anchor gNB within the RNA

The UE access to the new cell under the anchor gNB in the same RNA and initiates SDT using the ROHC continuity function. Since the anchor gNB has the ROHC context for the SDT DRB, naturally the anchor gNB can use the ROHC context to decompress the UL data and vice versa, and No RAN3 impact is foreseen.

- No RAN3 impact is foreseen to support the RNA based ROHC continuity for SDT DRB in case the UE initiates SDT in the cell under the anchor gNB within the RNA.

- Case 2: UE initiates the SDT in the new cell out of the RNA

If the UE moves to another cell out of the RNA and initiates SDT, the UE should reset the ROHC context. And once the anchor gNB notices that the UE is accessing from a cell out of the RNA, the anchor gNB also reset the ROHC context. There is no RAN3 impact is foreseen.

- No RAN3 impact is foreseen to support the RNA based ROHC continuity for SDT DRB in case the UE initiates SDT in the cell out of RNA.

- Case 3: UE initiates the SDT in the new cell under the new serving gNB within the RNA

To align with the principle for NR R15 inter-gNB handover for the RRC_CONNECTED UE, we assume that the ROHC context is not expected to be retrieved from the anchor gNB to the new serving NB, then in case the UE initiates the SDT in a cell under a new serving gNB within the RNA, there are several solutions:

Solution 1: anchor gNB determines to use without anchor relocation in case RNA based ROHC continuity is configured.

As the anchor gNB is responsible for the ROHC handling based on the ROHC context for the SDT DRB. Therefore, this case is similar as above case 1, the anchor gNB can use the ROHC context to decompress the UL data and vice versa and, No RAN3 impact is foreseen.

Solution 2: in case of with anchor relocation, the new serving gNB sends RRCRelease message to UE to de-configure the ROHC continuity.

In this solution, the anchor gNB needs to inform the new serving gNB about whether the RNA based ROHC continuity is configured or not via RETRIEVE UE CONTEXT RESPONSE message.

In case RNA based ROHC continuity is configured, the new serving gNB sends RRCRelease message to the UE to de-configure the ROHC continuity.

Then the UE initiates another SDT procedure by re re-establishing the PDCP entity for the SDT DRB which includes the ROHC context reset, and re-transmit the PDCP SDU.

Solution 3: in case of with anchor relocation, the new serving gNB sends RRCResume message including the reestablishPDCP IE for the SDT DRB, which triggers the ROHC context reset.

In this solution, the anchor gNB needs to inform the new serving gNB about whether the RNA based ROHC continuity is configured or not via RETRIEVE UE CONTEXT RESPONSE message.

In case RNA based ROHC continuity is configured, the new serving gNB sends RRCResume message including the reestablishPDCP IE for the SDT DRB, which triggers the ROHC context reset, and after that, the UE re-transmits the PDCP SDU.

Note that in such case the security key used for this radio bearer is not changed, which is not aligned with current RAN2 specification:

reestablishPDCP Indicates that PDCP should be re-established. Network sets this to true whenever the security key used for this radio bearer changes. Key change could for example be due to termination point change for the bearer, reconfiguration with sync, resuming an RRC connection, or the first reconfiguration after reestablishment. It is also applicable for LTE procedures when NR PDCP is configured. Network doesn't include this field for DRB if the bearer is configured as DAPS bearer.

Comparing of these three solutions, for solution 1, there is no stage3 impact for both RAN2 and RAN3. For solution 2, a new IE e.g. RNA based ROHC Continuity Indication IE (ENUMERATED {true, ...}) is needed to be introduced in XnAP RETRIEVE UE CONTEXT RESPONSE message, and the UE needs to initiate another SDT procedure which is not as efficient as solution 1. Solution 3 is even worse, besides the need of the new IE e.g. RNA based ROHC Continuity Indication IE, it also impacts the RAN2 principle of setting reestablishPDCP to true.

- No RAN3 impact is foreseen in the RNA based ROHC continuity for SDT DRB in case of SDT without anchor relocation.

- There is RAN3 impact and Uu impact in the RNA based ROHC continuity for SDT DRB in case of SDT with anchor relocation, and therefore it is not preferred.

- The anchor gNB determines to use SDT without anchor relocation if the RNA based ROHC continuity is configured for SDT DRB for the UE.

# 3. Proposals

- In this contribution, we analyse how to support the switch from SDT to non-SDT based on CCCH solution and ROHC continuity aspects, and get the following observations and proposals:

- The CCCH based solution i.e. option 6.e/6.a captured in draft CR part in [1] is feasible from RAN2 point of view. The CCCH based solution, i.e. Upon non-SDT data arrives during the SDT procedure, UE triggers the second RRC Resume Request procedure along with suspending all SRBs and DRBs execpt the SRB0 and updating the current KgNB and KRRCint to the UE Inactive AS context.

- Assuming the the period RNA update procedure is not triggered during the SDT procedure, no impcat on XnAP is foressen about the CCCH based solution:

- The new serving gNB send the 2nd XnAP: RETRIEVE UE CONTEXT REQUEST to the anchor gNB in case of the 2nd RRC Resume Request is received from the UE.

- The anchor gNB updates the current KgNB and KRRCint to the local UE context and verify the short MAC-I  from UE based on the latest updated KRRCint, and dervive the new key for the RRC RRC RELEASE message protection.

- No impcat on F1AP is foressen in CU-DU split case to support the CCCH based solution:

The new serving gNB-DU can send the RRC Resume Request message from UE via the legacy F1AP: INITIAL UL RRC MESSAGE TRANSFER message.

- Send an LS to RAN2 that the CCCH solution (i.e. option 6.e/6.a in [1]) for the switch from non-SDT to SDT is feasible from RAN3 point of view, and no RAN3 stage 3 impact is foreseen.

- No RAN3 impact is foreseen to support the cell based ROHC continuity for SDT DRB.

- No RAN3 impact is foreseen to support the RNA based ROHC continuity for SDT DRB in case the UE initiates SDT in the cell under the anchor gNB within the RNA.

- No RAN3 impact is foreseen to support the RNA based ROHC continuity for SDT DRB in case the UE initiates SDT in the cell out of RNA.

- No RAN3 impact is foreseen in the RNA based ROHC continuity for SDT DRB in case of SDT without anchor relocation.

- There is RAN3 impact and Uu impact in the RNA based ROHC continuity for SDT DRB in case of SDT with anchor relocation, and therefore it is not preferred.

- The anchor gNB determines to use SDT without anchor relocation if the RNA based ROHC continuity is configured for SDT DRB for the UE.

- To support proposal1, the corresponding TP to TS 38.300 is provided in section 5.

- And in section 6, we provide a draft Reply LS to RAN2 on the ROHC continuity aspect.

# 4. References

- [1] R2-2110596, Non-SDT data arrival, Huawei, HiSilicon, InterDigital, LGE, Ericsson, ASUSTeK, Nokia, Nokia Shanghai Bell, Google, Rakuten Mobile, Fujitsu, NEC

- [2] R2-2111509, Further clarifications on non-SDT arrival solutions, Huawei, HiSilicon, InterDigital, LGE, Ericsson, ASUSTeK, Nokia, Nokia Shanghai Bell, Google, NEC, Fujitsu, Rakuten Mobile

- [3] R2-2109065, Reply LS on Small data transmissions, Source: SA3

- [4]	R2-2111446, LS on the ROHC continuity for SDT, RAN2

# 5. TP to RA-SDT BL CR of TS 38.300

### XX.3	RA-based SDT without UE context relocation

Editor’s Note: Further discussion is needed for the overall procedure for RACH based SDT without anchor relocation.

Non-SDT arrival during the RACH based SDT without anchor relocation

The switch from SDT to non-SDT procedure for RACH based small data transmission without anchor relocation is illustrated in Figure xx.b-1.

Figure xx.3-1: switch from SDT to non-SDT without anchor relocation.

1.  Upon the SDT data arrival at UE, UE triggers RA-based SDT, RA-based SDT without UE context relocation is performed as specified in xx.3.1 step 1 to aa.

2. Upon the non-SDT data arrival at UE, UE triggers the second RRC Resumes Request procedure by sending an RRCResumeRequest to the receiving gNB with the same I-RNTI used in step 1.

3. The new serving gNB locates the last serving gNB using the received I-RNTI, and sending the RETRIEVE UE CONTEXT REQUEST message towards the last serving gNB.

4. Upon receiving the same I-RNTI received in step1, the last serving gNB makes decision of ending ongoing SDT procedure.

5. The last serving gNB sends the RETRIEVE UE CONTEXT FAILURE message to the new serving gNB with RRCRelease message contains an indication to re-access for non-SDT.

6. The new serving gNB forwards the RRCRelease message to the UE.

7. The UE sends another RRCResumeRequest to the receiving gNB for non-SDT access, as specified in section 9.2.2.4.1.

# 6. [Draft] reply LS to RAN2

Title:	[Draft] Reply LS on the ROHC continuity for SDT

Response to:	LS R2-2111446 on the ROHC continuity for SDT

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	Huawei [to be RAN3]

To:	RAN 2

Cc:

Contact person:

#### Name:	Yan Wang

#### E-mail Address:	wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 would like to thank RAN2 for the LS on ROHC continuity for SDT.

RAN3 discussed the Cell-based ROHC continuity and RAN-based ROHC continuity, and would like to provide the following feedback:

- No RAN3 impact is foreseen to support the cell based ROHC continuity for SDT DRB.

- No RAN3 impact is foreseen to support the RNA based ROHC continuity for SDT DRB, in case the UE initiates SDT in the cell under the anchor gNB, or the UE initiates SDT in the cell out of RNA.

- In case in case RNA based ROHC continuity is configured, if the UE initiates the SDT in the new cell under the new serving gNB within the RNA, the last serving gNB should make decision to use without anchor relocation.

# 2	Actions

To RAN2

ACTION: RAN3 respectfully asks RAN2 to take the above information into account.

# 3	Dates of next RAN3 meetings

RAN3#115-e 	21 February – 3 March 2022			Online