3GPP TSG-RAN WG3 Meeting #114-bis-e	R3-220439

17th January – 26th January 2022

Title:	LS on S-MBR and Target NSSAI

Response to:

Release:	Rel-17

Work Item:	NR_Slice

Source:	RAN3

To:	SA2

Cc:	-

Contact person:	Angelo Centonza

Angelo.Centonza@Ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachment: R3-220438

# Overall description

RAN3 has discussed solutions for handling of the S-MBR and Target NSSAI.

For the handling of the S-MBR, RAN3 has agreed to the following principles:

- It is agreed to signal the S-MBR over the NG interface in a way similar to the UE AMBR, namely as part of the following messages:

  - NG: PDU Session Resource Setup Request

  - NG: Initial UE Context Setup Request

  - NG: Handover Request

  - NG: UE Context Modification Request

  - NG: Path Switch Request Ack

  - NG: Downlink NAS Transport

- It is proposed that, at reception of the S-MBR, the RAN is able to reply to the AMF with information indicating that the S-MBR enforcement is not-supported or not-feasible or supported-and-feasible

For the handling of the Target NSSAI, RAN3 has agreed to the following principles:

- It is agreed that the gNB should save the Target NSSAI and the Target RFSP in the UE context.

- It is agreed that signalling from AMF to NG-RAN is introduced to indicate removal of the Target NSSAI and Target RFSP from the UE context

RAN3 also discussed that signalling from AMF to NG-RAN of the Target NSSAI and Target RFSP tasks the NG-RAN to perform UE mobility towards target cells able to serve the Target NSSAI. If, after such mobility, the Target NSSAI is signalled to the new serving RAN, there may be sub-optimal mobility cases, such as ping pongs.

Therefore, RAN3 assumes that the AMF provides the Target NSSAI and Target RFSP to the NG-RAN in a consistent manner, ensuring to minimise sub-optimal mobility such as ping pong events.

An agreed CR to TS38.413 is attached, reflecting the principles above.

# 2	Actions

To SA2:

ACTION: 	RAN3 respectfully asks SA2 to take the above agreements and considerations into account.

# 3	Dates of next RAN3 meetings

The upcoming RAN3 meetings can be found in the RAN3 Meetings calendar