3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220495

E-meeting, 17 - 26 January 2022

Agenda item:	  22.3.1

Source:		Lenovo, Motorola Mobility

Title:			(TP to TS 38.415 BL CR) Support of MBS Sequence number in PDU Session User Plane Protocol

Document for:		Approval

1	Introduction

This paper provides the text proposal to TS 38.415 for the following proposal in [1]:

Define a new 32 bits MBS Sequence number in PDU Session User Plane Protocol (TS 38.415) for PDCP count value synchronization between source and target gNBs.

[1] R3-220494	On handover between MBS supporting nodes - Alt.1	Lenovo, Motorola Mobility

2	Text Proposal

--------------------------------------------------Change Start -----------------------------------------------------------------

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MBS	Multicast Broadcast Service

PPI	Paging Policy Indicator

PPP	Paging Policy Presence

QFI	QoS Flow Identifier

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

UP	User Plane

UPF	User Plane Function

--------------------------------------------------Next Change -----------------------------------------------------------------

4.1	General aspects

The PDU Session User Plane protocol is located in the User Plane of the Radio Network Layer above the Transport Network Layer of the interface.

Each PDU session User Plane protocol instance is associated to one PDU Session.

In this version of the present document, the PDU session user plane protocol data is conveyed by GTP-U protocol means, more specifically, by means of the "GTP-U Container" GTP-U Extension Header as defined in TS 29.281 [3].

In this version of the present document, the term of PDU Session also represents MBS Session if applicable.

--------------------------------------------------Next Change -----------------------------------------------------------------

5.4.1	Transfer of DL PDU Session Information

5.4.1.1	Successful operation

The purpose of the Transfer of DL PDU Session Information procedure is to send control information elements related to the PDU Session from UPF to NG-RAN.

In the case of uplink and downlink data forwarding the DL PDU Session Information procedure shall be used to send control information elements related to the PDU Session from NG-RAN node to UPF, or from UPF to NG-RAN node, or between NG-RAN nodes.

A PDU Session user plane instance making use of the Transfer of DL PDU Session Information procedure is associated to a single PDU Session. The Transfer of DL PDU Session Information procedure may be invoked whenever packets for that particular PDU Session need to be transferred across the related interface instance.

The DL PDU SESSION INFORMATION frame includes a QoS Flow Identifier (QFI) field associated with the transferred packet. The NG-RAN shall use the received QFI to determine the QoS flow and QoS profile which are associated with the received packet.

The DL PDU SESSION INFORMATION frame shall include the Reflective QoS Indicator (RQI) field to indicate that user plane Reflective QoS shall be activated or not. The NG-RAN shall, if RQA has been configured for the involved QoS flow, take the RQI into account as specified in TS 37.324 [4].

The DL PDU SESSION INFORMATION frame may also include a Paging Policy Indicator (PPI) field associated with the transferred packet. The NG-RAN shall use the received PPI to determine the paging policy differentiation which is associated with the received packet as described in [5].

The DL PDU SESSION INFORMATION frame may also include a QoS Monitoring Packet (QMP) field and a DL sending time stamp field. The NG-RAN shall, if QoS monitoring has been configured for the included QFI field, perform delay measurement and QoS monitoring, as specified in TS 23.501 [5].

The DL PDU SESSION INFORMATION frame may also include a DL QFI Sequence Number field associated with the transferred packet. The NG-RAN shall, if the QoS flow has been configured eligible for redundant transport bearer in TS 38.413 [6], use the received DL QFI Sequence Number field to determine and eliminate duplicated packets for a given QoS flow as specified in TS 23.501 [5].

The DL PDU SESSION INFORMATION frame may also include a DL MBS Sequence Number field associated with the transferred packet. The NG-RAN shall use the received DL MBS Sequence Number field to determine the PDCP count value of the packet as specified in TS 38.300 [2].

When needed, the NG-RAN shall propagate the DL PDU Session Information to a peer NG-RAN.

Figure 5.4.1.1-1: Successful Transfer of DL PDU Session Information

--------------------------------------------------Next Change -----------------------------------------------------------------

5.5.2.1	DL PDU SESSION INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective DL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| PDU Type (=0) |  |  |  | QMP | SNP | MSNP |  | Spare | 1 |
| PPP | RQI | QoS Flow Identifier |  |  |  |  |  |  | 1 |
| PPI |  |  | Spare |  |  |  |  |  | 0 or 1 |
| DL Sending Time Stamp |  |  |  |  |  |  |  |  | 0 or 8 |
| DL QFI Sequence Number |  |  |  |  |  |  |  |  | 0 or 3 |
| DL MBS Sequence Number |  |  |  |  |  |  |  |  | 0 or 4 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL PDU SESSION INFORMATION (PDU Type 0) Format

--------------------------------------------------Next Change -----------------------------------------------------------------

#### 5.5.3.x1	MBS Sequence Number Presence (MSNP)

Description: This parameter indicates the presence of the DL MBS  Sequence Number in the DL PDU Session Information frame.

Value range: {0= DL MBS Sequence Number not present, 1= DL MBS Sequence Number present}.

Field length: 1 bit.

#### 5.5.3.x2	DL MBS Sequence Number

Description: This parameter indicates the sequence number for an MBS Session or an MBS QoS flow (FFS).

Value range: {0..232-1}.

Field length: 4 octets.