3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220525

Online, 17 – 26 January 2022

Agenda item:	19.2.5

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR_pos_enh BL CR for TS 38.455) Multipath reporting enhancements

Document for:	Discussion and Decision

# 1	Introduction

The Rel-17 WID on NR Positioning Enhancements [1] includes the following objective:

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

RAN1 has discussed multipath reporting enhancements and reached several agreements which are captured in the latest Rel-17 NR higher-layers parameter list [2]. In this paper, we evaluate the RAN3 impacts of the RAN1 agreements and provide a text proposal for the NRPPa BL CR.

# 2	Discussion

## 2.1	Additional path reporting (N>2) for time measurements

RAN1 has discussed support for reporting relative timing of additional paths (N>2) relative to the timing of the first detected path, and reached the following agreement:

Agreement (see [2] row 133) - For enhanced multipath reporting support N=8 for the value of maximum number of additional paths.

In Rel-16, it was already possible to report up to 2 additional paths. Therefore, this RAN1 agreement simply extends the maximum number of additional paths from 2 to 8. In RAN3 specifications, this can be supported by introducing an Extended Additional Path List IE which is included by the gNB when more than 2 additional paths are being reported.

Proposal 1:	Introduce an Extended Additional Path List IE within the UL RTOA Measurement IE and gNB Rx-Tx Time Difference IE.

RAN1 has also agreed that the (N>2) relative timing (to the first detected path) is reported by the gNB at the request of the LMF (see [2] row 139). Therefore, a new Extended Additional Path List Request IE should be introduced in the MEASUREMENT REQUEST message to enable the LMF to trigger the extended (N>2) additional path reporting.

Proposal 2:	Introduce an Extended Additional Path List Request IE in the MEASUREMENT REQUEST message.

## 2.2	UL AoA per SRS

RAN1 has discussed UL AoA enhancements for both the first arrival path and the additional arrival paths. For the first arrival path, RAN1 reached the following agreements:

Agreement (see [2] row 65) Reporting of one UL-RTOA and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning and per SRS resource for MIMO in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF. Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported - The above measurements are associated with SRS resource ID which is also reported to LMF. Agreement (see [2] row 68) The maximum number of UL-AOAs values (pair of AOA & ZOA values) to be reported per SRS resource for the first arrival path corresponding to the same timestamp is 8.

This means that for the first arrival path, up to 8 UL-AoA values per SRS resource can be reported when included with UL RTOA or gNB Rx-Tx time difference measurements (i.e. for hybrid positioning). Therefore, the following is proposed:

Proposal 3:	Introduce a new UL AoA per SRS IE which enables reporting of up to 8 UL-AoA measurements per SRS resource (or positioning SRS resource).

Proposal 4:	The new UL AoA per SRS IE can be included in the UL RTOA Measurement IE and gNB Rx-Tx Time Difference IE.

For the additional paths, RAN1 reached the following agreements:

Agreement (see [2] row 134) Reporting multiple UL-AoA values per SRS resource for the additional path is supported for at least UL TDOA and multi-RTT. - FFS: maximum number of UL-AoA values per additional path. For hybrid positioning methods where UL TDOA and multi-RTT are used in addition to UL AoA, support reporting of up to M=8 UL-AoA values per additional path

This means that up to 8 UL-AoA values per SRS resource can be included per additional path. Therefore, the following is proposed:

Proposal 5:	The new UL AoA per SRS IE can be included in the Additional Path List IE and Extended Additional Path List IE.

Finally, RAN1 has also agreed that the UL-AoA values per SRS resource for the additional path is reported by the gNB at the request of the LMF (see [2] row 140). Therefore, a new UL AoA of Additional Path Request IE should be introduced in the MEASUREMENT REQUEST message to enable the LMF to request that the gNB include the UL AoA per SRS IE in the Additional Path List IE and Extended Additional Path List IE.

Proposal 6:	Introduce an UL AoA of Additional Path Request IE in the MEASUREMENT REQUEST message.

# 3	Conclusions

In this paper, we evaluated the RAN3 impacts of multipath reporting enhancements. The following is proposed:

Proposal 1:	Introduce an Extended Additional Path List IE within the UL RTOA Measurement IE and gNB Rx-Tx Time Difference IE.

Proposal 2:	Introduce an Extended Additional Path List Request IE in the MEASUREMENT REQUEST message.

Proposal 3:	Introduce a new UL AoA per SRS IE which enables reporting of up to 8 UL-AoA measurements per SRS resource (or positioning SRS resource).

Proposal 4:	The new UL AoA per SRS IE can be included in the UL RTOA Measurement IE and gNB Rx-Tx Time Difference IE.

Proposal 5:	The new UL AoA per SRS IE can be included in the Additional Path List IE and Extended Additional Path List IE.

Proposal 6:	Introduce an UL AoA of Additional Path Request IE in the MEASUREMENT REQUEST message.

A text proposal for the NRPPa BL CR [3] is provided in Annex A, where the changes reflecting the above proposals are highlighted in yellow.

# References

- RP-210903 Revised WID on NR Positioning Enhancements, Intel Corporation, CATT

- R3-220094 LS on updated Rel-17 LTE and NR higher-layers parameter list, RAN1

- R3-216270 Introduction of NR Positioning enhancements to NRPPa, Ericsson

# Annex A:	TP for TS 38.455 BL CR

Start of Text Proposal for TS 38.455 BL CR

#### 8.5.1.2	Successful Operation

Figure 8.5.1.2.1: Measurement procedure. Successful operation.

The LMF initiates the procedure by sending a MEASUREMENT REQUEST message to the NG-RAN node, indicating in the TRP Measurement Request List IE the TRP(s) from which measurements are requested. The NG-RAN node shall use the included information to configure positioning measurements by the indicated TRP(s). If at least one of the requested measurements has been successful for at least one of the TRPs, the NG-RAN node shall reply with a MEASUREMENT RESPONSE message including the TRP Measurement Response List IE.

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

If the Measurement Beam Information Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall include the Measurement Beam Information IE in the Measurement Result IE of the MEASUREMENT RESPONSE message.

If the Measurement Quality IE is included in the Measurement Result IE in the MEASUREMENT RESPONSE message, the LMF may take it into account as the TRP estimate of the measurement quality. If the Measurement Quality IE includes the Zenith Quality IE, the LMF may take it into account within the angle measurement quality.

If the Timing Reporting Granularity Factor IE is included in the TRP Measurement Quantities IE in the MEASUREMENT REQUEST message, the NG-RAN node may take it into account when configuring measurements including UL RTOA and gNB Rx-Tx Time Difference.

If the System Frame Number IE and/or the Slot Number IE are included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, consider that the respective information indicates the activation time of SRS transmission.

If the Report Characteristics IE is set to "OnDemand" and the Response Time IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, return the corresponding measurement results in the MEASUREMENT RESPONSE message within the indicated time.

If the Extended Additional Path List Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the Extended Additional Path List IE in the Measurement Result IE of the MEASUREMENT RESPONSE message if extended (i.e. greater than 2) additional path results of time measurement are available.

If the UL AoA of Additional Path Request IE is included in the MEASUREMENT REQUEST message, the NG-RAN node shall, if supported, include the UL AoA per SRS IE in the Additional Path IE of the MEASUREMENT RESPONSE message , and in the Extended Additional Path IE of the MEASUREMENT RESPONSE message if extended (i.e. greater than 2) additional path results of time measurement are available.

Next Change

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| TRP Measurement Request List |  | 1 |  |  | YES | reject |
| >TRP Measurement Request Item |  | 1..<maxnoofMeasTRPs> |  |  | EACH | reject |
| >>TRP ID | M |  | 9.2.24 |  | - |  |
| >>Search Window Information | O |  | 9.2.26 |  | - |  |
| >>Cell ID | O |  | NR CGI 9.2.9 | The Cell ID of the TRP identified by the TRP ID IE. | YES | ignore |
| >> AoA Search Window Information | O |  | UL-AoA Assistance Information 9.2.x4 |  | YES | ignore |
| Report Characteristics | M |  | ENUMERATED (OnDemand, Periodic, ...) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED (120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…, 20480ms, 40960ms) | The codepoint 60min is not applicable | YES | reject |
| TRP Measurement Quantities |  | 1 |  |  | YES | reject |
| >TRP Measurement Quantities Item |  | 1 .. <maxnoPosMeas> |  |  | EACH | reject |
| >TRP Measurement Type | M |  | ENUMERATED (gNB-RxTxTimeDiff, UL-SRS-RSRP, UL-AoA, UL-RTOA,…) |  | - |  |
| >Timing Reporting Granularity Factor | O |  | INTEGER (0..5) | Value (0..5) corresponds to (k0..k5) TS 38.133 [16] | - |  |
| SFN initialisation Time | O |  | Relative Time 1900 9.2.36 | If this IE is not present, the TRP may assume that the value is same as its own SFN initialisation time. | YES | ignore |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| Measurement Beam Information Request | O |  | ENUMERATED (true,...) |  | YES | ignore |
| System Frame Number | O |  | INTEGER(0..1023) |  | YES | ignore |
| Slot Number | O |  | INTEGER(0..79) |  | YES | ignore |
| Response Time | O |  | 9.2.x6 |  | YES | ignore |
| Extended Additional Path List Request | O |  | ENUMERATED (true, …) |  | YES | ignore |
| UL AoA of Additional Path Request | O |  | ENUMERATED (true, …) |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maxmum no. of TRPs that can be included within one message. Value is 64. |

Next Change

### 9.2.37	TRP Measurement Result

This information element contains the measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Measured Result Item |  | 1 .. <maxnoPosMeas> |  |  |  |  |
| >CHOICE Measured Results Value | M |  |  |  |  |  |
| >>UL Angle of Arrival | M |  | 9.2.38 |  | - |  |
| >>UL SRS-RSRP | M |  | INTEGER (0..126) |  | - |  |
| >>UL RTOA | M |  | 9.2.39 |  | - |  |
| >>gNB Rx-Tx Time Difference | M |  | 9.2.40 |  | - |  |
| >>Zenith Angle of Arrival | M |  | 9.2.x5 |  | YES | reject |
| >Time Stamp | M |  | 9.2.42 |  | - |  |
| >Measurement Quality | O |  | 9.2.43 |  | - |  |
| >Measurement Beam Information | O |  | 9.2.57 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |

### 9.2.38	UL Angle of Arrival

This information element contains the uplink Angle of Arrival measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [16] |
| Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [16] |
| LCS to GCS Translation | O |  | 9.2.x7 | If absent, the azimuth and zenith are provided in GCS. |

### 9.2.39	UL RTOA Measurement

This information element contains the uplink RTOA measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE UL RTOA Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| Additional Path List | O |  | 9.2.41 |  | - |  |
| Extended Additional Path List | O |  | 9.2.b2 |  | YES | ignore |
| UL AoA per SRS | O |  | 9.2.b3 |  | YES | ignore |

### 9.2.40	gNB Rx-Tx Time Difference

This information element contains the gNB Rx-Tx Time Difference measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE gNB Rx-Tx Time Difference Measurement | M |  |  |  | - |  |
| >k0 | M |  | INTEGER (0.. 1970049) | TS 38.133 [16] | - |  |
| >k1 | M |  | INTEGER (0.. 985025) | TS 38.133 [16] | - |  |
| >k2 | M |  | INTEGER (0.. 492513) | TS 38.133 [16] | - |  |
| >k3 | M |  | INTEGER (0.. 246257) | TS 38.133 [16] | - |  |
| >k4 | M |  | INTEGER (0.. 123129) | TS 38.133 [16] | - |  |
| >k5 | M |  | INTEGER (0.. 61565) | TS 38.133 [16] | - |  |
| Additional Path List | O |  | 9.2.41 |  | - |  |
| Extended Additional Path List | O |  | 9.2.b2 |  | YES | ignore |
| UL AoA per SRS | O |  | 9.2.b3 |  | YES | ignore |

### 9.2.41	Additional Path List

This information element contains the additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnopath> |  |  | - |  |
| >CHOICE Relative Path Delay | M |  |  |  | - |  |
| >>k0 | M |  | INTEGER(0..16351) |  | - |  |
| >>k1 | M |  | INTEGER(0..8176) |  | - |  |
| >>k2 | M |  | INTEGER(0..4088) |  | - |  |
| >>k3 | M |  | INTEGER(0..2044) |  | - |  |
| >>k4 | M |  | INTEGER(0..1022) |  | - |  |
| >>k5 | M |  | INTEGER(0..511) |  | - |  |
| >Path Quality | O |  | Measurement Quality 9.2.43 |  | - |  |
| >UL AoA per SRS | O |  | 9.2.b3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnopath | Maximum no. of additional path measurement. Value is 2. |

Next Change

### 9.2.b2	Extended Additional Path List

This IE contains the extended (i.e. greater than 2) additional path results of time measurement.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Additional Path Item |  | 1..<maxnoExtPath> |  |  |
| >CHOICE Relative Path Delay | M |  |  |  |
| >>k0 | M |  | INTEGER(0..16351) |  |
| >>k1 | M |  | INTEGER(0..8176) |  |
| >>k2 | M |  | INTEGER(0..4088) |  |
| >>k3 | M |  | INTEGER(0..2044) |  |
| >>k4 | M |  | INTEGER(0..1022) |  |
| >>k5 | M |  | INTEGER(0..511) |  |
| >Path Quality | O |  | Measurement Quality 9.2.43 |  |
| >UL AoA per SRS | O |  | 9.2.b3 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoExtPath | Maximum no. of additional path measurement. Value is 6. |

### 9.2.b3	UL AoA per SRS

This IE contains UL AoA measurement values per SRS.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL AoA per SRS Item |  | 1..<maxnoSRS> |  |  |
| >CHOICE Reference Signal | M |  |  |  |
| >>SRS |  |  |  |  |
| >>>SRS Resource ID | M |  | INTEGER(0..63) |  |
| >>Positioning SRS |  |  |  |  |
| >>>Positioning SRS Resource ID | M |  | INTEGER(0..63) |  |
| >UL Angle of Arrival List | M |  | 9.2.b4 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoSRS | Maximum no. of SRS. Value is 64. |

### 9.2.b4	UL Angle of Arrival List

This IE contains a list of UL AoA measurements.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| UL Angle of Arrival Item |  | 1..<maxnoAoA> |  |  |
| >Azimuth Angle of Arrival | M |  | INTEGER(0..3599) | TS 38.133 [16] |
| >Zenith Angle of Arrival | O |  | INTEGER(0..1799) | TS 38.133 [16] |

| Range bound | Explanation |
| --- | --- |
| maxnoAoA | Maximum no. of UL AoA measurements. Value is 8. |

Next Change

### 9.3.4	PDU Definitions

** SKIPPING UNCHANGED TEXT **

PRSTRPList,

PRSTransmissionTRPList,

ResponseTime,

ExtAddPathListRequest,

ULAOAofAddPathRequest

** SKIPPING UNCHANGED TEXT **

id-PRSTRPList,

id-PRSTransmissionTRPList,

id-ResponseTime,

id-ExtAddPathListRequest,

id-ULAOAofAddPathRequest

** SKIPPING UNCHANGED TEXT **

MeasurementRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{MeasurementRequest-IEs}},

...

}

MeasurementRequest-IEs NRPPA-PROTOCOL-IES ::= {

{ ID id-LMF-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-TRP-MeasurementRequestList		CRITICALITY reject	TYPE TRP-MeasurementRequestList PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics		PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity		PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to “periodic” –

{ ID id-TRPMeasurementQuantities			CRITICALITY reject	TYPE TRPMeasurementQuantities		PRESENCE mandatory}|

{ ID id-SFNInitialisationTime			CRITICALITY ignore	TYPE RelativeTime1900		PRESENCE optional}|

{ ID id-SRSConfiguration				CRITICALITY ignore	TYPE SRSConfiguration			PRESENCE optional}|

{ ID id-MeasurementBeamInfoRequest		CRITICALITY ignore	TYPE MeasurementBeamInfoRequest	PRESENCE optional}|

{ ID id-SystemFrameNumber				CRITICALITY ignore	TYPE SystemFrameNumber			PRESENCE optional}|

{ ID id-SlotNumber						CRITICALITY ignore	TYPE SlotNumber					PRESENCE optional}|

{ ID id-ResponseTime					CRITICALITY ignore	TYPE ResponseTime				PRESENCE optional}|

{ ID id-ExtAddPathListRequest			CRITICALITY ignore	TYPE ExtAddPathListRequest		PRESENCE optional}|

{ ID id-ULAOAofAddPathRequest			CRITICALITY ignore	TYPE ULAOAofAddPathRequest		PRESENCE optional},

...

}

Next Change

### 9.3.5	Information Element definitions

** SKIPPING UNCHANGED TEXT **

id-OnDemandTRPPRS,

id-AoA-SearchWindow,

id-ZoA,

id-ExtAdditionalPathList,

id-ULAoAperSRS,

maxnoExtPath,

maxnoSRS,

maxnoAoA

** SKIPPING UNCHANGED TEXT **

AdditionalPathList ::= SEQUENCE (SIZE (1.. maxNoPath)) OF AdditionalPathListItem

AdditionalPathListItem ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TrpMeasurementQuality	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AdditionalPathListItem-ExtIEs} } OPTIONAL,

...

}

AdditionalPathListItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ULAoAperSRS			CRITICALITY ignore EXTENSION ULAoAperSRS		PRESENCE optional },

...

}

** SKIPPING UNCHANGED TEXT **

ExtAdditionalPathList ::= SEQUENCE (SIZE (1.. maxnoExtPath)) OF ExtAdditionalPathListItem

ExtAdditionalPathListItem ::= SEQUENCE {

relativeTimeOfPath	RelativePathDelay,

pathQuality			TrpMeasurementQuality	OPTIONAL,

uLAoAperSRS			ULAoAperSRS				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ExtAdditionalPathListItem-ExtIEs} } OPTIONAL,

...

}

ExtAdditionalPathListItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ExtAddPathListRequest ::= ENUMERATED (true, ...)

** SKIPPING UNCHANGED TEXT **

GNB-RxTxTimeDiff ::= SEQUENCE {

rxTxTimeDiff		GNBRxTxTimeDiffMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { GNB-RxTxTimeDiff-ExtIEs} }	OPTIONAL,

...

}

GNB-RxTxTimeDiff-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ExtAdditionalPathList		CRITICALITY ignore EXTENSION ExtAdditionalPathList		PRESENCE optional },

{ ID id-ULAoAperSRS					CRITICALITY ignore EXTENSION ULAoAperSRS				PRESENCE optional },

...

}

** SKIPPING UNCHANGED TEXT **

ULAoAList ::= SEQUENCE (SIZE (1.. maxnoAoA)) OF ULAoAItem

ULAoAItem ::= SEQUENCE {

azimuthAoA			INTEGER (0..3599),

zenithAoA			INTEGER (0..1799)				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ULAoAItem-ExtIEs} } OPTIONAL,

...

}

ULAoAItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ULAOAofAddPathRequest ::= ENUMERATED (true, ...)

ULAoAperSRS ::= SEQUENCE (SIZE (1.. maxnoSRS)) OF ULAoAperSRSItem

ULAoAperSRSItem ::= SEQUENCE {

uLAoAReferenceSignal	ULAoAReferenceSignal,

uLAoAList				ULAoAList,

iE-Extensions		ProtocolExtensionContainer { { ULAoAperSRSItem-ExtIEs} } OPTIONAL,

...

}

ULAoAperSRSItem-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

...

}

ULAoAReferenceSignal ::= CHOICE {

sRS						SRSResourceID,

positioningSRS			SRSPosResourceID,

choice-Extension		ProtocolIE-Single-Container {{ULAoAReferenceSignal-ExtensionIE }}

}

ULAoAReferenceSignal-ExtensionIE NRPPA-PROTOCOL-IES ::= {

...

}

** SKIPPING UNCHANGED TEXT **

UL-RTOAMeasurement ::= SEQUENCE {

uLRTOAmeas			ULRTOAMeas,

additionalPathList	AdditionalPathList	OPTIONAL,

iE-extensions			ProtocolExtensionContainer { { UL-RTOAMeasurement-ExtIEs } }	OPTIONAL,	...

}

UL-RTOAMeasurement-ExtIEs NRPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ExtAdditionalPathList		CRITICALITY ignore EXTENSION ExtAdditionalPathList		PRESENCE optional },

{ ID id-ULAoAperSRS					CRITICALITY ignore EXTENSION ULAoAperSRS				PRESENCE optional },

...

}

** SKIPPING UNCHANGED TEXT **

Next Change

### 9.3.7	Constant definitions

** SKIPPING UNCHANGED TEXT **

maxnoofPRSresource							INTEGER ::= 64

maxnoExtPath								INTEGER ::= 6

maxnoSRS									INTEGER ::= 64

maxnoAoA									INTEGER ::= 8

** SKIPPING UNCHANGED TEXT **

id-ResponseTime													ProtocolIE-ID ::= x7

id-ExtAddPathListRequest										ProtocolIE-ID ::= xx

id-ULAOAofAddPathRequest										ProtocolIE-ID ::= xx

id-ExtAdditionalPathList										ProtocolIE-ID ::= xx

id-ULAoAperSRS													ProtocolIE-ID ::= xx

End of Text Proposal for TS 38.455 BL CR