3GPP TSG-RAN WG3 #114bis-e		                                   R3-220547

17th – 26th Jan, 2022

Online

Source:	CATT

Title:	TP for 38.413 on session management for multicast service

Agenda Item:	22.2.2

Document for:	Discussion and Decision

Introduction

According to the discussion paper in R3-220546,we provide TP for 38.413 on session management for multicast service.

Text Proposal to 38.413

- 8	NGAP Procedures

8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| UE Radio Capability Check | UE RADIO CAPABILITY CHECK REQUEST | UE RADIO CAPABILITY CHECK RESPONSE |  |
| UE Context Suspend | UE CONTEXT SUSPEND REQUEST | UE CONTEXT SUSPEND RESPONSE | UE CONTEXT SUSPEND FAILURE |
| UE Context Resume | UE CONTEXT RESUME REQUEST | UE CONTEXT RESUME RESPONSE | UE CONTEXT RESUME FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| Multicast Distribution Setup | MULTICAST DISTRIBUTION SETUP REQUEST | MULTICAST DISTRIBUTION SETUP RESPONSE | BROADCAST SESSION RESOURCE SETUP FAILURE |
| Multicast Session Resource Update | MULTICAST SESSION RESOURCE UPDATE REQUEST | MULTICAST SESSION RESOURCE UPDATE RESPONSE | MULTICAST SESSION RESOURCE UPDATE FAILURE |
| Multicast Distribution Release | MULTICAST DISTRIBUTION RELEASE REQUEST | MULTICAST DISTRIBUTION RELEASE RESPONSE |  |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |

8.x	Multicast Session Management Procedure

8.x.1	Multicast Distribution Setup

- 8.x.1.1	General

The purpose of the Multicast Distribution Setup procedure is to request the MB-SMF to establish the shared delivery for the multicast session.

The procedure uses non-UE associated signalling.

- 8.x.1.2	Successful Operation

Figure 8.x.1.2-1. Multicast Distribution Setup procedure. Successful operation.

The gNB initiates the procedure by sending an MULTICAST DISTRIBTUION SETUP REQUEST message to the AMF.

- 8.x.1.3	Unsuccessful Operation

Figure 8.x.1.3-1. Multicast Distribtution Setup procedure. Unsuccessful operation.

If the shared delivery tunnel could not be established, the AMF shall send MULTICAST DISTRIBUTION SETUP FAILURE message.

- 8.x.1.4	Abnormal Conditions

Void.

8.x.2	Multicast Session Resource Update

- 8.x.2.1	General

The purpose of the Multicast Session Resource Update procedure is to request gNB to update the information related to the previously established Multicast session.

The procedure uses non-UE associated signalling.

- 8.x.2.2	Successful Operation

Figure 8.x.2.2-1. Multicast Session Resource Update procedure. Successful operation.

The AMF initiates the procedure by sending a MULTICAST SESSION RESOURCE UPDATE REQUEST message to the gNB.

- 8.x.2.3	Unsuccessful Operation

Figure 8.x.2.3-1. Multicast Session Resource Update procedure. Unsuccessful operation.

If the gNB is not capable of correctly processing the request, the gNB shall send MULTICAST SESSION RESOURCE UPDATE FAILURE message.

- 8.x.2.4	Abnormal Conditions

Void.

- 8.x.3		Multicast Distribution Release

- 8.x.3.1	General

The purpose of the Multicast Distribution Release procedure is to request the release of shared delivery of Multicast session.

The procedure uses non-UE assocated signalling.

- 8.x.3.2	Successful Operation

Figure 8.x.3.2-1. Multicast Distribution Release procedure. Successful operation.

The gNB initiates the procedure by sending a MULTICAST DISTRIBTUION RELEASE REQUEST message to the AMF.

Upon receiption of the MULTICAST DISTRIBTUION RELEASE REQUEST message, the AMF shall respond with the MULTICAST DISTRIBTUION RELEASE RESPONSE message.

- 8.x.3.3	Abnormal Conditions

Void.

============ Next change ==============

- 9.2.x		Multicast Session Management Messages

- 9.2.x.1	MULTICAST DIRTRIBUTION SETUP REQUEST

This message is sent by the gNB to request the share delivery of multicast session.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.x |  |  |  |
| Area Session ID | O |  | FFS |  |  |  |
| Multicast Distribution Setup Request Transfer | O |  | OCTET STRING | Containing the UP Transport Layer Information IE specified in subclause 9.3.2.2 |  |  |

- 9.2.x.2 MULTICAST DIRTRIBUTION SETUP RESPONSE

This message is sent by the AMF to infrom NG-RAN node of the the successful setup of shared delivery of multicast session as the response of MULTICAST DISTRIBUTION SETUP REQUEST message.

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.x |  |  |  |
| Area Session ID | M |  | FFS |  |  |  |
| Multicast Distribution Setup Response Transfer | O |  | OCTET STRING | Containing the Multicast Distribution Setup Response Transfer IE specified in subclause 9.3.4.x |  |  |

- 9.2.x.3	MULTICAST DIRTRIBUTION SETUP FAILURE

This message is sent by the AMF to infrom NG-RAN node of the failure of shared delivery of multicast session as the response of MULTICAST DISTRIBUTION SETUP REQUEST message.

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.x |  |  |  |
| Area Session ID | O |  | FFS |  |  |  |
| Multicast Distribution Setup Failure Transfer |  |  | FFS |  |  |  |

- 9.2.x.4	MULTICAST SESSION RESOURCE UPDATE REQUEST

This message is sent by the AMF to modify a MBS context of multicast service.

Direction: AMF  gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.x |  |  |  |
| Multicast Session Resource Update Request Transfer | O |  | OCTET STRING | Containing the MBS Session Resource Updata Request Transfer IE specified in subclause 9.3.4.xx |  |  |

- 9.2.x.5	MULTICAST SESSION RESOURCE UPDATE RESPONSE

This message is sent by the gNB to report the successful outcome of the request from the MULTICAST SESSION RESOURCE UPDATE REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.x |  |  |  |
| Multicast Session Resource Update ResponseTransfer | O |  | OCTET STRING | Containing the Multicast Session Resource Update Response Transfer IE specified in subclause 9.3.4.yy |  |  |

- 9.2.x.6	MULTICAST SESSION RESOURCE UPDATE FAILURE

This message is sent by the gNB to report the unsuccessful outcome of the request from the MULTICAST SESSION RESOURCE UPDATE REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.x |  |  |  |
| Multicast Session Resource Update Failure Transfer | M |  | OCTET STRING | Containing the MBS Session Resource Modify Failure Transfer IE specified in subclause 9.3.4.zz |  |  |

- 9.2.x.7	MULTICAST DIRTRIBUTION RELEASE REQUEST

This message is sent by the gNB to release the shared delivery for multicast session.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.X |  |  |  |
| Area Session ID | O |  | FFS |  |  |  |
| Multicast Distribution Release Request transfer |  |  | OCTET STRING | Containing the UP Transport Layer Information IE specified in subclause 9.3.2.2 |  |  |

- 9.2.x.8	MULTICAST DIRTRIBUTION RELEASE RESPONSE

This message is sent by the AMF to acknowledge the MULTICAST DISTRIBTUION RELEASE REQUEST message.

Direction: gNB  AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  |  |  |
| MBS Session ID | M |  | 9.3.4.X |  |  |  |
| Area Session ID | O |  | FFS |  |  |  |

- 9.3.4	SMF Related IEs

- 9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | 9.3.1.102 | This IE shall be present when at least one Non-GBR QoS flow is being setup and is ignored otherwise. | YES | reject |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. | YES | reject |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of UL PDUs for split PDU session. | YES | reject |
| Data Forwarding Not Possible | O |  | 9.3.1.63 | This IE may be present in case of HANDOVER REQUEST message and is ignored otherwise. | YES | reject |
| PDU Session Type | M |  | 9.3.1.52 |  | YES | reject |
| Security Indication | O |  | 9.3.1.27 |  | YES | reject |
| Network Instance | O |  | 9.3.1.113 | This IE is ignored if the Common Network Instance IE is included. | YES | reject |
| QoS Flow Setup Request List |  | 1 |  |  | YES | reject |
| >QoS Flow Setup Request Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  | - |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  | - |  |
| >>TSC Traffic Characteristics | O |  | 9.3.1.130 | This IE may be present in case of GBR QoS flows and is ignored otherwise. | YES | ignore |
| >>Assocaited MBS session ID | O |  | 9.3.4.x |  |  |  |
| >>Assocated Multicast QoS flow ID | O |  | 9.3.1.51 |  |  |  |
| >>Redundant QoS Flow Indicator | O |  | 9.3.1.134 | This IE indicates whether this QoS flow is requested for the redundant transmission. | YES | ignore |
| Common Network Instance | O |  | 9.3.1.120 |  | YES | ignore |
| Direct Forwarding Path Availability | O |  | 9.3.1.64 | This IE may be present in case of inter-system handover and is ignored otherwise. | YES | ignore |
| Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs for the redundant transmission. | YES | ignore |
| Additional Redundant UL NG-U UP TNL Information | O |  | UP Transport Layer Information List 9.3.2.12 | UPF endpoint of the additional NG-U transport bearer(s), for delivery of redundant UL PDUs for split PDU session. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.120 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.3.1.136 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

- 9.3.4.X MBS session ID

The MBS session ID uniquely identifies the MBMS Bearer Service.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMGI |  |  |  |  |
| >PLMN identity | M |  | 9.3.3.5 |  |
| >Service ID | M |  | OCTET STRING (SIZE (3)) |  |

- 9.3.5	MB-SMF Related IEs

- 9.3.5.X	Multicast Distribution Setup Response Transfer

This IE is used to provide the IP multicast user plane transport layer information associated with a multicast session for an NG-RAN node – MB-UPF pair.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UP Transport Layer Information | M |  |  |  |
| >GTP tunnel |  |  |  |  |
| >>IP Endpoint Address | M |  | Transport Layer Address 9.3.2.4 |  |
| >>IP Source Address | M |  | 9.3.2.4 |  |
| >>GTP-TEID | M |  | 9.3.2.5 |  |

- 9.3.5.XX	Multicast Session Resource Update Request Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| TNL Information list | O |  |  |  |  |  |
| >TNL Information Item |  | 1 .. <maxnoofAreasessionpergNB> |  |  |  |  |
| >>Area sessio ID |  | FFS |  |  |  |  |
| >>Multicast session area |  | FFS |  |  |  |  |
| >>TNL Information | O |  |  |  |  |  |
| >>>IP Multicast Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >>>IP Source Address | M |  | Transport Layer Address 9.3.2.4 |  |  |  |
| >>>GTP DL TEID | M |  | 9.3.2.5 |  |  |  |
| QoS Flows To Be Setup List | O |  |  |  |  |  |
| >QoS Flows To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |  |  |
| QoS Flows To Be Modify List | O |  |  |  |  |  |
| >QoS Flows To Be Modify Item |  | 1 .. <maxnoofQoSFlows> |  |  |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |  |  |
| QoS Flows To Be Release List | O |  |  |  |  |  |
| >QoS Flows To Be Release Item |  | 1 .. <maxnoofQoSFlows> |  |  |  |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  |  |  |
| MBS session status | O |  | ENUMERATED (Active,Inactive…) |  |  |  |

- 9.3.5.YY	Multicast Session Resource Update Response Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UP TNL Information list | O |  |  |  |  |  |
| >UP TNL Information Item |  | 1 .. <maxnoofAreasessionpergNB> |  |  |  |  |
| >>Area sessio ID |  | FFS |  |  |  |  |
| >>UP Transport Layer Information | O |  | 9.3.2.2 |  |  |  |
| QoS Flow Add or Modify Response List |  | 0..1 |  |  | - |  |
| >QoS Flow Add or Modify Response Item |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Identifier | M |  | 9.3.1.51 |  | - |  |
| QoS Flow Failed to Add or Modify List | O |  | QoS Flow List with Cause 9.3.1.13 |  | - |  |

- 9.3.5.ZZ	MBS Session Resource update Failure Transfer

This IE is transparent to AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Cause | M |  | 9.3.1.2 |  |  |  |

- //ASN.1 would be added after there is conclusion on the tabular and procdures.