3GPP TSG-RAN WG3 Meeting # 114bis-e	R3-220554

# Online,  17 - 26 Jan, 2022                                                                            was R3-215623

Agenda item:	14.3

Source:	Google

Title:	(TP to CPAC BLCR to 37.340) Coordination Information and configuration parameters application in CPAC

Document for:	Discussion & Decision

- Introduction

In this paper, the Resource Coordination Information and lower layer parameters coordination issue is found for CPA or inter-SN CPC and a text proposal is made to resolve it.

- Discussion

RAN3#111e agreed the following working assumption.

- WA: Prepare multiple PSCells in one CPAC procedure. Do not provide Location Information and Resource Coordination information in CPAC, use same parameters for other IEs in the response message for different PSCells, FFS for single RRC container or multiple RRC containers which is pending to RAN2.

Since there could be multiple candidate PSCells prepared the same target SN, during the CPAC prepaprtion phase, it was argued that the Resource Coordination Information IE and other parameters or IEs as defined in X2AP or XnAP may not be necessary.

- Quoted from TS 38.473, 8.3.4 UE Context Modification (gNB-CU initiated) For DC operation, if the gNB-CU includes the CG-Config IE in the CU to DU RRC Information IE that is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU may initiate low layer parameters coordination taking this information into account.

In addition to the Resource Coordination Information IE, as quoted above, there are other parameters in the CG-Config IE that could be used by the MN (or MN-DU if the MN is a disaggregated base station) to perform lower layer parameters coordination. For example, the drx-InfoSCG, measConfigSN, selectedBandCombination, and ph-InfoSCG are parameters configured by the SN and MN needs to take them into account when performing its own configuration. Following the similar logic as the WA, when receiving the multiple CG-Config IEs during the CPAC preparation phase, the MN should suspend applying the CG-Config IEs for the purpose of low layer parameter coordination.

Proposal 1: RAN3 to merge the text proposal for the MN to suspend applying the lower layer parameters coordination into the Section 10.2 and 10.5 of CPAC BLCR for TS 37.340

If the MN is a disaggregated base station, it may be necessary to discuss how and when the Resource Coordination Information and/or cell-specific configuration paraemeters requiring MN and SN coordination is applied by the MN-DU. As there could be multiple choices of CG-Config IE(s) to be sent to the MN-DU during the CPAC preparation, following the proposal 1, the MN-CU should suspend forwarding the received CG-Config IE(s) to the MN-DU until CPAC execution.

As the cell-specific physical resource or configuration coordination may be still required eventually, it is proposed to capture in the specification how the coordination information and configuration parameters are provided by the target SN upon the successful execution of a prepared CPAC (e.g., after the target SN receives the SN Reconfiguration Complete message from the MN). To achieve so, the SN-initiated SN Modification procedure by the target SN can be utilized upon the successful execution of a prepared CPAC.

Upon CPAC execution, depending on whether the CG-Config IE(s) have cell specific parameters, there could be two alternatives as below. If the principle of the WA is followed for the CG-Config IE(s) during the CPAC preparation as well, it is proposed to use the Alt 1.

- Alt1: (the CG-Config IE(s) in the CG-CandidateList does not have cell specific parameters) target SN provides the "CG-Config" with cell specific parameters (together with a cell-specific Resource Coordination Information if necessary) in a SN Modification Required message after the successful execution of a prepared CPAC (e.g., after the target SN receives the SN Reconfiguration Complete message from the MN). The MN-CU transmits this received CG-Config (along with the cell-specific Resource Coordination Transfer Information IE) to the MN-DU in a UE Context Modification Request message.

  - – in short, an SN Modification Required message is sent by the target SN to provide CG-Config IE and the Resource Coordination Information

- Alt2: (the CG-Config IE(s) in the CG-CandidateList have cell specific parameters) MN selects the CG-Config to be transmitted out of the CG-Config IE(s) in the CG-CandidateList to the DU based on the configuration ID received in the RRC reconfiguration complete message. If necessary the cell-specific Resource Coordination Information is provided in a SN Modification Required message from the target SN after the successful execution of a prepared CPAC (e.g., after the target SN receives the SN Reconfiguration Complete message from the MN). The MN-CU transmits the selected CG-Config (along with the cell-specific Resource Coordination Transfer Information IE if received) to the MN-DU in a UE Context Modification Request message.

  - – in short, an SN Modification Required message is sent by the target SN to provide the Resource Coordination Information

Proposal 2: RAN3 to merge the text proposal for the Resource Coordination Information and configuration parameter exchange upon CPAC execution into the Section 10.3 of CPAC BLCR for TS 37.340

Proposal 3: RAN3 to confirm that the Resource Coordination Information and configuration parameters are signalled to the MN-DU upon CPAC execution

- Text Proposal to CPAC BL CR to TS 37.340

## ----------Start of the First Change--------------

## 10.2	Secondary Node Addition

### 10.2.1	EN-DC

## ----------Skip the unchanged parts--------------

Conditional PSCell Addition (CPA)

Figure 10.2.1-2 shows the Conditional Secondary Node Addition procedure.

Figure 10.2.1-2: Conditional Secondary Node Addition procedure

1.	The MN decides to configure CPA for the UE, the MN requests the candidate SN to allocate resources for a specific E-RAB, indicating E-RAB characteristics (E-RAB parameters, TNL address information corresponding to bearer type), provides the upper limit for the number of PSCells to the candidate SN and indicates the list of PSCells that may be prepared by the MN. In addition, for the bearers requiring SCG radio resources, the MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the latest measurement results for the SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. The MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision. In case of bearer options that require X2-U resources between the MN and the SN, the MN provides X2-U TNL address information for the respective E-RAB, X2-U DL TNL address information for SN terminated bearers, X2-U UL TNL address information for MN terminated bearers. In case of SN terminated split bearers the MN provides the maximum QoS level that it can support. The SN may reject the request.

NOTE 6:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective E-RAB is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MNs decision is reflected in step 1 by the E-RAB parameters signalled to the SN, which may differ from E-RAB parameters received over S1.

NOTE 7:	For a specific E-RAB, the MN may request the direct establishment of an SCG or a split bearer, i.e., without first having to establish an MCG bearer. It is also allowed that all E-RABs can be configured as SN terminated bearers, i.e. there is no E-RAB established as an MN terminated bearer.

2.	If the RRM entity in the SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer option, respective transport network resources, and provides the prepared PSCell ID(s) to the MN. For bearers requiring SCG radio resources, the SN triggers Random Access so that synchronisation of the SN radio resource configuration can be performed. Within the list of PSCells indicated by the MN, the SN decides the list of  n PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***) contained in the SgNB Addition Request Acknowledge message. In case of bearer options that require X2-U resources between the MN and the SN, the SN provides X2-U TNL address information for the respective E-RAB, X2-U UL TNL address information for SN terminated bearers, X2-U DL TNL address information for MN terminated bearers. For SN terminated bearers, the SN provides the S1-U DL TNL address information for the respective E-RAB and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided. The MN suspends applying the lower layer parameters coordination.

NOTE 8:	For the SN terminated split bearer option, the SN may either decide to request resources from the MN of such an amount, that the QoS for the respective E-RAB is guaranteed by the exact sum of resources provided by the MN and the SN together, or even more. The SNs decision is reflected in step 2 by the E-RAB parameters signalled to the MN, which may differ from E-RAB parameters received in step 1. The QoS level requested from the MN shall not exceed the level that the MN offered when setting up the split bearer in step 1.

NOTE 9:	In case of SN terminated bearers and MN terminated split/SCG bearers, early data forwarding may take place after step 2. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN. For the early data forwarding of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN. The MN sends the Early Status Transfer message to the SN.

3.	The MN sends to the UE an RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) including the CPA configuration, (i.e. a list of RRCConnectionReconfiguration messages RRCConnectionReconfiguration**) and associated execution conditions, in which RRCConnectionReconfiguration** contains RRCReconfiguration*** received from the candidate SN. Besides, the RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements.

4.	The UE applies the RRC configuration (in RRCConnectionReconfiguration*) excluded the CPA configuration, stores the CPA configuration and replies to the MN with an RRCConnectionReconfigurationComplete message (RCConnectionReconfigurationComplete*) without any NR RRC response messag. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration* message, it performs the reconfiguration failure procedure.

4a. The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration message (RRCConnectionReconfiguration**) corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete message (RRCConnectionReconfigurationComplete**), including an NR RRC message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN

5.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully via SgNB ReconfigurationComplete message, including the RRCReconfigurationComplete*** message.The MN sends the SgNB Release Request message(s) to cancels CPA in the other target candidate SN(s), if configured.

6.	The UE performs synchronisation towards the selected PSCell indicated in RRCConnectionReconfiguration**. The order the UE sends the RRCConnectionReconfigurationComplete** message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

7.	If PDCP termination point is changed to the SN for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status Transfer.

8.	For SN terminated bearers moved from the MN, dependent on the bearer characteristics of the respective E-RAB, the MN may take actions to minimise service interruption due to activation of EN-DC (Data forwarding).

9-12.	If applicable, the update of the UP path towards the EPC is performed.

### 10.2.2	MR-DC with 5GC

## ----------Skip the unchanged parts--------------

MN initiated Conditional PSCell Addition (CPA)

Figure 10.2.2-2 shows the Conditional SN Addition procedure.

Figure 10.2.2-2: Conditional Secondary Node Addition procedure

1.	The MN decides to configure CPA for the UE. The MN requests the target candidate SN to allocate resources for one or more specific PDU Sessions/QoS Flows, indicating QoS Flows characteristics (QoS Flow Level QoS parameters, PDU session level TNL address information, and PDU session level Network Slice info), provides the upper limit for the number of PSCells to the candidate SN and indicates the list of PSCells that may be prepared by the MN. In addition, for bearers requiring SCG radio resources, the MN indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. In this case, the MN also provides the latest measurement results for the SN to choose and configure the SCG cell(s). The MN may request the SN to allocate radio resources for split SRB operation. In NGEN-DC and NR-DC, the MN always provides all the needed security information to the SN (even if no SN terminated bearers are setup) to enable SRB3 to be setup based on SN decision.

For MN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides Xn-U UL TNL address information. For SN terminated bearers, the MN provides a list of available DRB IDs. The S-NG-RAN node shall store this information and use it when establishing SN terminated bearers. The SN may reject the request.

For SN terminated bearer options that require Xn-U resources between the MN and the SN, the MN provides in step 1 a list of QoS flows per PDU Sessions for which SCG resources are requested to be setup upon which the SN decides how to map QoS flows to DRB.

NOTE 6:	For split bearers, MCG and SCG resources may be requested of such an amount, that the QoS for the respective QoS Flow is guaranteed by the exact sum of resources provided by the MCG and the SCG together, or even more. For MN terminated split bearers, the MN decision is reflected in step 1 by the QoS Flow parameters signalled to the SN, which may differ from QoS Flow parameters received over NG.

NOTE 7:	For a specific QoS flow, the MN may request the direct establishment of SCG and/or split bearers, i.e. without first having to establish MCG bearers. It is also allowed that all QoS flows can be mapped to SN terminated bearers, i.e. there is no QoS flow mapped to an MN terminated bearer.

2.	If the RRM entity in the SN is able to admit the resource request, it allocates respective radio resources and, dependent on the bearer type options, respective transport network resources, and provides the prepared PSCell ID(s) to the MN. For bearers requiring SCG radio resources the SN triggers UE Random Access so that synchronisation of the SN radio resource configuration can be performed. Within the list of PSCells indicated by the MN, the SN decides the list of  n PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***), contained in the SN Addition Request Acknowledge message. In case of bearer options that require Xn-U resources between the MN and the SN, the SN provides Xn-U TNL address information for the respective DRB, Xn-U UL TNL address information for SN terminated bearers, Xn-U DL TNL address information for MN terminated bearers. For SN terminated bearers, the SN provides the NG-U DL TNL address information for the respective PDU Session and security algorithm. If SCG radio resources have been requested, the SCG radio resource configuration is provided. The MN suspends applying the lower layer parameters coordination.

NOTE 8:	For MN terminated bearers for which PDCP duplication with CA is configured in NR SCG side, the MN allocates up to 4 separate Xn-U bearers and the SN provides a logical channel ID for primary or split secondary path to the MN.

For SN terminated bearers for which PDCP duplication with CA is configured in NR MCG side, the SN allocates up to 4 separate Xn-U bearers and the MN provides a logical channel ID for primary or split secondary path to the SN via an additional MN-initiated SN modification procedure.

NOTE 9:	In case of SN terminated bearers and MN terminated split/SCG bearers, early data forwarding may take place after step 2. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN. For the early data forwarding of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN.

2a.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message. In case of early data forwarding in CPA, the MN sends the Early Status Transfer message to the SN.

3.	The MN sends to the UE an RRC reconfiguration message (RRC reconfiguration*) including the CPA configuration, (i.e. a list of RRC reconfiguration messages RRC reconfiguration**) and associated execution conditions, in which RRC reconfiguration** contains RRCReconfiguration*** received from the candidate SN. Besides, the RRC reconfiguration message (RRC reconfiguration*) can also include the current MCG updated configuration. e.g. to configure the required conditional measurements.

4.	The UE applies the RRC configuration (in RRC reconfiguration*) excluded the CPA configuration, stores the CPA configuration and replies to the MN with an RRC reconfiguration complete message (RRC reconfiguration complete*) without any NR SN RRC response messag). In case the UE is unable to comply with (part of) the configuration included in the RRC reconfiguration* message, it performs the reconfiguration failure procedure.

4a. The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRC reconfiguration message (RRC reconfiguration**) corresponding to the selected candidate PSCell, and sends an MN RRC reconfiguration complete message (RRC reconfiguration complete**), including an NR RRC reconfiguration complete message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN.

5.	The MN informs the SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete*** response message. The MN sends the SN Release Request message(s) to cancels CPA in the other target candidate SN(s), if configured.

6.	The UE performs synchronisation towards the selected PSCell indicated in RRC reconfiguration**. The order the UE sends the MN RRC reconfiguration complete** message and performs the Random Access procedure towards the SCG is not defined. The successful RA procedure towards the SCG is not required for a successful completion of the RRC Connection Reconfiguration procedure.

7.	If PDCP termination point is changed to the SN for bearers using RLC AM, and when RRC full configuration is not used, the MN sends the SN Status Transfer.

8.	For SN terminated bearers or QoS flows moved from the MN, dependent on the characteristics of the respective bearer or QoS flow, the MN may take actions to minimise service interruption due to activation of MR-DC (Data forwarding).

9-12.	If applicable, the update of the UP path towards the 5GC is performed via a PDU Session Path Update procedure.

## ----------Start of the Next Change--------------

## 10.3	Secondary Node Modification (MN/SN initiated)

### 10.3.1	EN-DC

## ----------Skip the unchanged parts--------------

SN initiated SN Modification with MN involvement

Figure 10.3.1-2: SN Modification procedure - SN initiated with MN involvement

The SN uses the procedure to perform configuration changes of the SCG within the same SN, e.g. to trigger the release of SCG bearer(s) and the SCG RLC bearer of split bearer(s) (upon which the MN may release the bearer or maintain current bearer type or reconfigure it to an MCG bearer, either MN terminated or SN terminated), and to trigger PSCell change (e.g. when a new security key is required or when the MN needs to perform PDCP data recovery). In case of CPAC, the target SN uses the procedure to provide Resource Coordination Information and configuration parameters for the accessed PSCell to the MN upon a successful execution of CPAC preparation. The MN cannot reject the release request of SCG bearer and the SCG RLC bearer of a split bearer. The MN shall either accept modification of all of the requested SCG bearer(s) and the SCG RLC bearer of split bearer(s), or fail the procedure. Figure 10.3.1-2 shows an example signalling flow for an SN initiated SgNB Modification procedure, with MN involvement.

1.	The SN sends the SgNB Modification Required message including a NR RRC configuration message, which may contain bearer context related, other UE context related information and the new SCG radio resource configuration. For bearer release or modification, a corresponding E-RAB list is included in the SgNB Modification Required message. In case of change of security key, the PDCP Change Indication indicates that a S-KgNB update is required. In case the MN needs to perform PDCP data recovery, the PDCP Change Indication indicates that PDCP data recovery is required.

The SN can decide whether the change of security key is required.

NOTE 1a:	In case SN includes the indication of full RRC configuration in SgNB Modification Required message to MN e.g. comprehension failure upon intra-CU inter-DU change, MN performs release and add of the NR SCG part of the configuration but does not release SN terminated radio bearers towards the UE.

2/3.	The MN initiated SN Modification procedure may be triggered by the SN Modification Required message (e.g. to provide information such as data forwarding addresses, new SN security key, measurement gap, etc...)

NOTE 2:	If only SN security key is provided in step 2, the MN does not need to wait for the reception of step 3 to initiate the RRC connection reconfiguration procedure.

4.	The MN sends the RRCConnectionReconfiguration message including a NR RRC configuration message to the UE including the new SCG radio resource configuration.

5.	The UE applies the new configuration and sends the RRCConnectionReconfigurationComplete message, including an encoded NR RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SgNB Modification Confirm message containing the encoded NR RRC response message, if received from the UE.

7.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN Status Transfer takes place between the MN and the SN (Figure 10.3.1-2 depicts the case where a bearer context is transferred from the SN to the MN).

NOTE 2a:	The SN may not be aware that a SN terminated bearer requesting to release is reconfigured to a MN terminated bearer. The SN Status for the released SN terminated bearers with RLC AM may also be transferred to the MN.

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.1-2 depicts the case where a bearer context is transferred from the SN to the MN).

10.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the E-RABs to be released.

NOTE 3:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped.

11.	If applicable, a path update is performed.

## ----------Start of the Next Change--------------

### 10.3.2	MR-DC with 5GC

## ----------Skip the unchanged parts--------------

SN initiated SN Modification with MN involvement

Figure 10.3.2-2: SN Modification procedure - SN initiated with MN involvement

The SN uses the procedure to perform configuration changes of the SCG within the same SN, e.g. to trigger the modification/release of the user plane resource configuration, to trigger the release of SCG resources (e.g., release SCG lower layer resources but keep SN), and to trigger PSCell changes (e.g. when a new security key is required or when the MN needs to perform PDCP data recovery). In case of CPAC, the target SN uses the procedure to provide Resource Coordination Information and configuration parameters for the accessed PSCell to the MN upon a successful execution of CPAC preparation. The MN cannot reject the release request of PDU session/QoS flows and the release request of SCG resources. The SN also uses the procedure to request the MN to provide more DRB IDs to be used for SN terminated bearers or to return DRB IDs used for SN terminated bearers that are not needed any longer. Figure 10.3.2-2 shows an example signalling flow for SN initiated SN Modification procedure.

1.	The SN sends the SN Modification Required message including an SN RRC reconfiguration message, which may contain user plane resource configuration related context, other UE context related information and the new radio resource configuration of SCG. In case of change of security key, the PDCP Change Indication indicates that an SN security key update is required. In case the MN needs to perform PDCP data recovery, the PDCP Change Indication indicates that PDCP data recovery is required.

The SN can decide whether the change of security key is required.

2/3.	The MN initiated SN Modification procedure may be triggered by SN Modification Required message, e.g. when an SN security key change needs to be applied.

NOTE 3:	For SN terminated bearers to be setup for which PDCP duplication with CA is configured in NR MCG side, the SN allocates up to 4 separate Xn-U bearers and the MN provides a logical channel ID for primary or split secondary path to the SN via the nested MN-initiated SN modification procedure.

4.	The MN sends the MN RRC reconfiguration message to the UE including the SN RRC reconfiguration message with the new SCG radio resource configuration.

5.	The UE applies the new configuration and sends the MN RRC reconfiguration complete message, including an SN RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SN Modification Confirm message including the SN RRC response message, if received from the UE.

7.	If instructed, the UE performs synchronisation towards the PSCell configured by the SN as described in SN Addition procedure. Otherwise, the UE may perform UL transmission directly after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN Status Transfer takes place between the MN and the SN (Figure 10.3.2-2 depicts the case where a bearer context is transferred from the SN to the MN).

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.2-2 depicts the case where a user plane resource configuration related context is transferred from the SN to the MN).

10.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 4:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

11.	If applicable, a PDU Session path update procedure is performed.

## ----------Start of the Next Change--------------

## 10.5	Secondary Node Change (MN/SN initiated)

### 10.5.1	EN-DC

## ----------Skip the unchanged parts--------------

MN initiated conditional SN Change

The MN initiated conditional inter-SN change procedure is used to configure CPC configuration.

Figure 10.5.1-3: Conditional SN Change – MN initiated

Figure 10.5.1-3 shows an example signalling flow for the MN initiated Conditional Secondary Node Change:

1/2.	The MN initiates the conditional SN change by requesting the target candidate SN to allocate resources for the UE by means of the SgNB Addition procedure. The MN may include measurement results related to the target SN, provides the upper limit for the number of PSCells and indicates the list of PSCells that may be prepared by the MN. Within the list of PSCells indicated by the MN, the SN decides the list of  n PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***) contained in the SgNB Addition Request Acknowledge message with the prepared PSCell ID(s). If forwarding is needed, the target SN provides forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration. The MN suspends applying the lower layer parameters coordination.

NOTE 5:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration before step 1.

4.	The MN sends to the UE an RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) including the CPC configuration, (i.e. a list of RRCConnectionReconfiguration messages RRCConnectionReconfiguration**) and associated execution conditions, in which RRCConnectionReconfiguration** contains RRCReconfiguration*** received from the candidate SN. Besides, the RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements.

5.  The UE applies the RRC configuration (in RRCConnectionReconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message (RCConnectionReconfigurationComplete*) without any NR RRC response messag. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration* message, it performs the reconfiguration failure procedure.

Upon receiving the RRCConnectionReconfigurationComplete message from the UE, the MN triggers the Data Forwarding Address Indication procedure to the source SN to inform that the CPC has been triggered, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding.

5a.	The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration message (RRCConnectionReconfiguration**) corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete message (RRCConnectionReconfigurationComplete**), including an NR RRC message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN.

6.  The MN sends the Data Forwarding Address Indication message to inform the source SN to stop providing user data to the UE, and the address of the selected SN and if applicable, to start late data forwarding.

7.	If the RRC connection reconfiguration procedure was successful, the MN informs the selected target SN via SgNBReconfigurationComplete message, including the RRCReconfigurationComplete*** message for the target SN. The MN sends the SgNB Release Request messages to cancel CPC in the other target candidate SNs, if configured.

8.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the target SN indicated in RRCConnectionReconfiguration**.

9.	For SN terminated bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends to the target selected SN, if needed.

10.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding address in step 3.

11.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 6:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped.

12-16.	If applicable, a path update is triggered by the MN.

17.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

SN initiated conditional inter-SN Change

The SN initiated conditional inter-SN change procedure is used to configure CPC configuration.

The SN initiated conditional inter-SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the target SN by cancellation of all the prepared PSCells at the target SN and releasing the CPC related UE context at the target SN.

Figure 10.5.1-4: Conditional SN Change – SN initiated

Editor’s Note: the procedure may be revisted if RAN3’s feedback is not supporting solution 2.

Figure 10.5.1-4 shows an example signalling flow for the Conditional Secondary Node Change initiated by the SN:

1.	The source SN initiates the conditional SN change procedure by sending SgNB Change Required message which contains a CPC initiation indication. The message also contains target SN ID information and may include the SCG configuration (to support delta configuration), and contains the measurements results related to the target candidate SN.

Editor’s Note: FFS whether the execution conditions for the candidate cells recommended by the source SN and the SCG measConfig for CPC are included in the SgNB Change Required message or in step 5.

2/3.	The MN requests the target candidate SN to allocate resources for the UE by means of the SgNB Addition procedure, including a CPC initiation indication, and the measurements results related to the target candidate SN and indicates the list of PSCells that received from the source SN. Within the list of PSCells, the SN decides the list of  n PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration****) contained in the SgNB Addition Request Acknowledge message. If forwarding is needed, the target SN provides forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The target SN can either accept or reject the candidate cells suggested by the Source-SN, i.e. it cannot come up with any alternative candidates. The MN suspends applying the lower layer parameters coordination.

4. The MN indicates the candidate PSCells accepted by the target SN to the source SN.

Editor’s Note: which message used for MN to indicate the candidate PSCells accepted by the target SN to the source SN is FFS; FFS whether Step 4 is mandatory or optional before the CPC configuration is sent to the UE.

5. The source SN may provide the measurement configurations for CPC to the MN or include the execution condition to the MN.

Editor’s Note: FFS which message used for the source SN to provide the SCG measurement configurations for CPC to the MN; step 5 is mandatory or optional; whether the execution condition is mandatory included within step 5.

6.  If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the target SN.

Editor’s Note:whether step 6 is needed and which message is used to inform source SN to not stop providing user data to the UE, and/or the data forwarding addresses as received from the target SN and if applicable, to start early data forwarding.

7.	The MN sends to the UE an RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) including the CPC configuration, (i.e. a list of RRCConnectionReconfiguration messages RRCConnectionReconfiguration***) and associated execution conditions, in which RRCConnectionReconfiguration*** contains RRCReconfiguration**** received from the candidate SN. Besides, the RRCConnectionReconfiguration message (RRCConnectionReconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements, as well as the NR RRC configuration message (RRCReconfigutation**) generated by the source-SN.

8.  The UE applies the RRC configuration (in RRCConnectionReconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRCConnectionReconfigurationComplete message (RCConnectionReconfigurationComplete*), which can include an NR RRC response message (RRCReconfigutationComplete**). In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration* message, it performs the reconfiguration failure procedure.

8a. If an NR RRC response message is included, the MN informs the source SN with the NR RRC response message (RRCReconfigutationComplete**) for the source SN.

Editor’s Note:: FFS which message to inform the source SN.

9.  The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRCConnectionReconfiguration message (RRCConnectionReconfiguration***) corresponding to the selected candidate PSCell, and sends an RRCConnectionReconfigurationComplete message (RRCConnectionReconfigurationComplete***), including an NR RRC message (RRCReconfigurationComplete****) for the selected candidate PSCell, and the selected PSCell information to the MN.

10: The MN informs source SN to stop providing user data to the UE, and provide the address of the selected target SN and if applicable, start late data forwarding.

Editor’s Note: whether SgNB Change Confirm message is used in step 10 to inform source SN to stop providing user data to the UE, and the address of the selected target SN is still FFS.

11.	If the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SgNB Reconfiguration Complete message, including the SN RRCReconfigurationComplete**** response message for the target SN. The MN cancels CPC in the other target candidate SNs, if configured.

12.	The UE synchronizes to the target SN indicated in RRCConnectionReconfiguration***.

13.	For SN terminated bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

14.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding message from the MN.

15.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the related E-RABs.

NOTE 7:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SgNB may send the report when the transmission of the related bearer is stopped.

16-20.	If applicable, a path update is triggered by the MN.

21.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

### 10.5.2	MR-DC with 5GC

## ----------Skip the unchanged parts--------------

MN initiated conditional SN Change

The Conditional Secondary Node Change procedure is initiated by the MN to configure the CPC configuration and CPC execution.

Figure 10.5.2-3: Conditional SN change procedure - MN initiated

Figure 10.5.2-3 shows an example signalling flow for the conditional SN Change initiated by the MN:

1/2.	The MN initiates the conditional SN change by requesting the target candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN may include measurement results related to the target SN, provides the upper limit for the number of PSCells and indicates the list of PSCells that may be prepared by the MN. Within the list of PSCells indicated by the MN, the SN decides the list of  n PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration***) contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration. The MN suspends applying the lower layer parameters coordination.

NOTE 4:	The MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1.

4.	The MN sends to the UE an RRC reconfiguration message (RRC reconfiguration*) including the CPC configuration, (i.e. a list of RRC reconfiguration messages RRC reconfiguration**) and associated execution conditions, in which RRC reconfiguration** contains RRCReconfiguration*** received from the candidate SN. Besides, the RRC reconfiguration message (RRC reconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements.

5.  The UE applies the RRC configuration (in RRC reconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRC reconfiguration complete message (RRC reconfiguration complete*) without any NR SN RRC response messag). In case the UE is unable to comply with (part of) the configuration included in the RRC reconfiguration* message, it performs the reconfiguration failure procedure.

Upon receiving the MN RRC reconfiguration complete message from the UE, the MN informs the SN that the CPC has been triggered via Xn-U Address Indication procedure, the source SN, if applicable, starts early data forwarding. The PDCP PDU and/or PDCP SDU forwarding may take place during early data forwarding.

5a. The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRC reconfiguration message (RRC reconfiguration**) corresponding to the selected candidate PSCell, and sends an MN RRC reconfiguration complete message (RRC reconfiguration complete**), including an NR RRC reconfiguration complete message (RRCReconfigurationComplete***) for the selected candidate PSCell, and the selected PSCell information to the MN.

Editor’s note: Whether a message is needed and which message is used to inform source SN to stop providing user data to the UE, and the address of the selected target SN and if applicable, to start late data forwarding are FFS.

6.  The MN sends the Xn-U Address Indication message to inform the source SN to stop providing user data to the UE, and the address of the selected target SN and if applicable, to start late data forwarding.

7.	If the RRC connection reconfiguration procedure was successful, the MN informs the selected target SN via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete*** response message for the target SN. The MN sends the SN Release Request messages to cancel CPC in the other target candidate SNs, if configured.

8.	If configured with bearers requiring SCG radio resources the UE synchronizes to the selected SN indicated in RRC reconfiguration**.

9.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target selected SN, if needed.

10.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the early data forwarding address in step 3.

11.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 5:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

12-16.	If applicable, a PDU Session path update procedure is triggered by the MN.

17.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

SN initiated conditional inter-SN Change

The SN initiated conditional inter-SN change procedure is used to configure CPC configuration.

The SN initiated conditional inter-SN change procedure may also be initiated by the source SN, to modify the existing CPC configuration, or to trigger the release of the target SN by cancellation of all the prepared PSCells at the target SN and releasing the CPC related UE context at the target SN.

Figure 10.5.2-4: Conditional SN change procedure - SN initiated

Editor’s Note: the procedure may be revisted if RAN3’s feedback is not supporting solution 2.

Figure 10.5.2-4 shows an example signalling flow for the conditional SN Change initiated by the SN:

- The source SN initiates the conditional SN change procedure by sending the SN Change Required message, which a CPC initiation indication.The message also contains a candidate target node ID and may include the SCG configuration (to support delta configuration) , and contains the measurements results which may include cells that are not CPC candidates.

Editor’s Note: whether the execution conditions for the candidate cells recommended by the source SN included in the SgNB Change Required message or in step 5.

2/3.	The MN requests the target SN to allocate resources for the UE by means of the SN Addition procedure, including a CPC initiation indication, and the measurements results which may include cells that are not CPC candidates received from the source SN to the target SN, and indicates the list of PSCells that received from the source SN. Within the list of PSCells, the SN decides the list of  n PSCell(s) to prepare and, for each prepared PSCell, the SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRC configuration message (RRCReconfiguration****) contained in the SgNB Addition Request Acknowledge message.. If data forwarding is needed, the target SN provides data forwarding addresses to the MN. The target SN includes the indication of the full or delta RRC configuration, and the list of prepared PSCell IDs to the MN. The target-SN can either accept or reject the candidate cells suggested by the Source-SN, i.e., it cannot come up with any alternative candidates. The MN suspends applying the lower layer parameters coordination.

4. The MN indicates the candidate PSCells accepted by the target SN to the source SN.

Editor’s Note: which message used for MN to indicate the candidate PSCells accepted by the target SN to the source SN; step 4 is mandatory or optional before the CPC configuration is sent to the UE.

5. The source SN may provide the updated measurement configurations to the MN or include the execution condition to the MN.

Editor’s Note: which message used for source SN to provide the updated measurement configurations to the MN; step 5 is mandatory or optional; whether the execution condition is mandatory included within step 5.

6.  The MN informs the source SN the data forwarding addresses as received from the target SN and if applicable, to start early data forwarding.

Editor’s Note:whether step 6 is needed and which message is used to inform the source SN to not stop providing user data to the UE, and/or the data forwarding addresses as received from the target SN and if applicable, to start early data forwarding.

7.	The MN sends to the UE an RRC reconfiguration message (RRC reconfiguration*) including the CPC configuration, (i.e. a list of RRC reconfiguration messages RRC reconfiguration***) and associated execution conditions, in which RRC reconfiguration*** contains RRCReconfiguration**** received from the candidate SN. Besides, the RRC reconfiguration message (RRC reconfiguration*) can also include the current MCG updated configuration, e.g., to configure the required conditional measurements, as well as the NR RRC configuration message (RRCReconfigutation**) generated by the source-SN.

8.  The UE applies the RRC configuration (in RRC reconfiguration*) excluded the CPC configuration, stores the CPC configuration and replies to the MN with an RRC reconfiguration complete message (RRC reconfiguration complete*), which can include an NR RRC response message (RRCReconfigutationComplete**). In case the UE is unable to comply with (part of) the configuration included in the RRC reconfiguration* message, it performs the reconfiguration failure procedure.

8a.	If an SN RRC response message is included, the MN informs the source SN via SN Reconfiguration Complete message with the SN RRC response message (RRCReconfigutationComplete**) for the source SN.

9.  The UE starts evaluating the execution conditions. If the execution condition of one candidate PSCell is satisfied, the UE applies RRC reconfiguration message (RRC reconfiguration***) corresponding to the selected candidate PSCell, and sends an RRC reconfiguration complete message (RRC reconfiguration complete***), including an NR RRC message (RRCReconfigurationComplete****) for the selected candidate PSCell, and the selected PSCell information to the MN.

10: The MN informs the source SN to stop providing user data to the UE, and provides the address of the selected target SN and if applicable, starts late data forwarding.

Editor’s Note:: whether SgNB Change Confirm message is used in step 10 to inform source SN to stop providing user data to the UE, and the address of the selected target SN is still FFS.

11.	If the RRC connection reconfiguration procedure was successful, the MN informs the target SN via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete**** response message for the target SN. The MN cancels CPC in the other target candidate SNs, if configured.

12.	The UE synchronizes to the target SN indicated in RRCConnectionReconfiguration***.

13.	If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer, which the MN sends then to the target SN, if needed.

14.	If applicable, data forwarding from the source SN takes place. It may be initiated as early as the source SN receives the data forwarding address related information from the MN.

15.	The source SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 6:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

16-20.	If applicable, a PDU Session path update procedure is triggered by the MN.

21.	Upon reception of the UE Context Release message, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

## ----------End of the Changes--------------