3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220624

# Online, 17 – 26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.423 | CR | 1662 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE X2AP ID in the ERROR INDICATION message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the ERROR INDICATION message, current semantics description for both the Old eNB UE X2AP ID and New eNB UE X2AP ID IEs could be misleading. For example, for the Old eNB UE X2AP ID it says “Allocated for handover at the source eNB and for dual connectivity at the SeNB or the eNB from which the E-RAB context is transferred”. The original intention of adding “from which the E-RAB context is transferred” was to clarify specially for the SN STATUS TRANSFER message, however such add-on causes confusion for the other messages. For instance, if Error Indication is triggered for the SeNB Addition procedure, then the Old eNB UE X2AP ID can be either filled up with SeNB for DC or MeNB where the E-RAB context is actually transferred from, which is incorrect. To solve this issue, there are two ways. Option 1 is to add clarification that the last part is only applicable for the SN Status Transfer procedure. Option 2 is to remove the last part which causes the confusion. Compared the two, Option 1 is a safer way without changing the legacy implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description that “the eNB from which the E-RAB context is transferred” for both Old eNB UE X2AP ID and New eNB UE X2AP ID within the ERROR INDICATION message is only applicable for SN Status Transfer procedure. Impact analysis: This CR has no ASN.1 impact. This CR only has an impact on the handling of Error Indication procedure. This CR is backward compatible. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability issue exists when Error Indication procedure is triggered in case of DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 0731 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------Start of the change--------------------------------------------------

#### 9.1.2.2	ERROR INDICATION

This message is used to indicate that some error has been detected in the eNB/en-gNB.

Direction: eNB1  eNB2 or eNB  en-gNB or en-gNB  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Old eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the source eNB and for dual connectivity at the SeNB or the eNB from which the E-RAB context is transferred for a SN Status Transfer procedure. | YES | ignore |
| New eNB UE X2AP ID | O |  | eNB UE X2AP ID 9.2.24 | Allocated for handover at the target eNB and for dual connectivity/EN-DC at the MeNB or the eNB to which the E-RAB context is transferred for a SN Status Transfer procedure. | YES | ignore |
| Cause | O |  | 9.2.6 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the source eNB and for dual connectivity at the SeNB or the eNB from which the E-RAB context is transferred for a SN Status Transfer procedure. | YES | ignore |
| New eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated for handover at the target eNB and for dual connectivity at the MeNB or the eNB to which the E-RAB context is transferred for a SN Status Transfer procedure. | YES | ignore |
| Old en-gNB UE X2AP ID | O |  | en-gNB UE X2AP ID 9.2.100 | Allocated for EN-DC at the en-gNB. | YES | ignore |
| Interface Instance Indication | O |  | 9.2.143 |  | YES | reject |

------------------------------------------------End of the change--------------------------------------------------