3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220707

E-meeting, 17-26 Jan 2022

Title: 	(TP to TS 38.300 BL CR) Broadcast service continuity

Source: 	Huawei, CBN, China Unicom, China Telecom

Agenda item:	22.4

Document Type:	other

# 1. Introduction

In last RAN3 meeting, the following agreements were made on broadcast service continuity:

NR MBS supports MBS frequency layer prioritization for broadcast MBS sessions

In this paper, we further discuss the potential impacts of the Broadcast Service Continuity.

# 2. Discussion

To support service continuity for broadcast service, there are several basic mechanisms to be considered:

Mechanism 1: Support of MBMS frequency layer prioritization

In LTE, a UE that is receiving/interested in MBS service(s) is allowed to prioritize the frequency where it is able to receive MBS service, e.g. based on the information provided via SIB15 or knowledge of the proper MBMS frequency via other means, i.e. USD. The mechanism is to ensure the UE to camp on the right frequency for the MBS servicing monitoring. Note that the LTE SIB 15 contains the MBMS Service Area Identities (SAI) of the current and/or neighbouring carrier frequencies.

As agreed in RAN2, for IDLE/INACTIVE, the UE is allowed to prioritize the MBS frequency of interest when the cell of the MBS frequency provides MBS SIB carrying the MCCH configuration, as LTE SC-PTM. Meanwhile, the UE is allowed to prioritize the MBS frequency of interest when the UE is only capable of receiving the MBS service by camping on the MBS frequency, as LTE SC-PTM. RAN2 also assumes the UE should be allowed to prioritize a frequency in case this frequency is signalled in SIBy for the UEs service/session of interest (e.g. identified by an additional ID such as SAI) regardless of whether this frequency is included in the USD for this service.

Mechanism 2: Neighbor cell broadcasting status for ongoing services

In LTE, the information about intra-frequency and inter-frequency neighbouring cells providing an ongoing service in serving cell can be provided in SC-MCCH. It should be noted that this information is not used for the UE to prioritize cells providing MBS service in cell reselection, it is used by the UE to decide to trigger unicast connection earlier if the MBS service that is receiving is not provided in the target cell after cell reselection or handover.

As agreed in RAN2 #116, the network may broadcast in MCCH a list of neighbor cells providing the same broadcast MBS service(s) as provided in the current cell, same as in LTE SC-PTM

Mechanism 3: Support of MII (MBMS interesting indication)

Similar to LTE eMBMS, for UEs in RRC_ CONNECTED mode, the UE may report interested MBS services to the RAN node, so that the RAN node could try to handover the UE to cells supporting MBS or even broadcasting this ongoing service.

The Mechanism 3 has been agreed in RAN2#113 e-meeting: for Broadcast service, MII is supported for UEs in RRC_CONNECTED mode but not in idle/inactive mode. Considering of the similarity for LTE MBMS and the broadcast in NR MBS, we see the benefits to reuse the above LTE mechanisms to maintain the service continuity for the service(s) delivered via broadcast session(s). Regarding Mechanism 2 and 3, some RAN3 impacts are expected as below:

To support Mechanism 2 and 3, the gNB needs to know neighbor cells that providing the ongoing service. In LTE, a cells list in which the MBS service is to be started is included in session start/update message and the cells list would include both cells in the current eNB and also the cells in neighbor eNBs. Therefore, the eNB could know the neighbor cells providing the same service on session start/update procedure. Consider to use such mechanism as baseline, upon the UEs in RRC_CONNECTED mode informs RAN node the MII, the RAN node could take MII and the neighbor cell broadcasting status for ongoing services into account during HO to maintain the service continuity.

SA2 has agreed to send MBS service area which could be represented by cell list or TAI list to RAN within MBS Session Resource Setup Request, the following statement can be found in TS23.247:

“The NEF may further convert the broadcast area, which can be defined in external format, to MBS service area. The MBS service area can be represented by cell list or TAI list.”

“The AMF sends an MBS Session Resource Setup Request (TMGI, LL MC and source host address, 5G Authorized QoS Profile) message to all NG-RANs in the MBS service area. The AMF may include the MBS service area.”

In order to reuse Mechanism 2 and 3, it is necessary to inform the source gNB about the cells that provides the broadcast service. There are two potential methods to transfer the information of the cells that provides the broadcast service:

- Method 1: exchange the broadcast services which are ongoing in neighbor cells via Xn interface;

- Method 2: transfer cell list containing all of the cells that provide the broadcast service over NG.

The neighbour broadcast information from Xn interface may be limited in the aspect of cell numbers, e.g., the source RAN node has no Xn interference with the target RAN node, even the target RAN node provides the MBS service indicated by the MII. Hence Method 1 only is not enough and Method 2 shall be considered. To support neighbor cell broadcasting status for ongoing services and NR MBS interesting indication report, the CN is needed to provide the MBS Service Area (i.e. cell list and/or TAI list) in which the NR MBS session to be broadcasted to the NG-RAN node during Broadcast Session Start/Update, and the MBS Service Area includes all the involved cells and or TAIs for that broadcast session.

- For broadcast service, the MBS Service Area to be transferred over the NG interface includes all the cells and/or TAIs for that broadcast session.

Moreover, to avoid repeated MII report after handover, the MII can be included in the HANDOVER REQUEST message from source node to target node. In LTE, the MII is included in the inter-node message HandoverPreparationInformation, which is contained by the RRC context in HANDOVER REQUEST message and with limited RAN3 specification impacts. The same approach can be applied in NR and MII is included in inter-node message HandoverPreparationInformation as LTE.

- MII is included in inter-node message HandoverPreparationInformation as LTE.

# 3. Proposals

In this contribution, we discuss the potential RAN3 impacts to support Service Continuity for broadcast sessions, and get the following proposals:

- For broadcast service, the MBS Service Area to be transferred over the NG interface includes all the cells and/or TAIs for that broadcast session.

- MII is included in inter-node message HandoverPreparationInformation as LTE.

The Text Proposal to TS 38.300 BL CR is provided in section 5.

# 4. Reference

[1] R3-215802, Reply LS on MBS broadcast service continuity and MBS session identification, SA2.

# 5. Text proposal to TS 38.300 BL CR

--------------------------------   Start of Change   -----------------------------

#### 16.x.5.4	Broadcast Mobility

The core network provides the MBS Service Area (i.e. cell list and/or TAI list) in which the NR MBS session to be broadcasted to the NG-RAN node during Broadcast Session Start/Update, and the MBS Service Area includes all the involved cells and or TAIs for that broadcast session.

During the mobility of connected mode UEs, the source NG-RAN nodes may provide the MII to the target NG-RAN node via the inter-node RRC HandoverPreparationInformation message.

--------------------------------   End of Change   -----------------------------