3GPP TSG-RAN WG3 Meeting #114bis-e		R3-220717

Online, Jan 17-26,   2022

Source:	CATT

Title:	Latency Improvement for PRS Measurement

Agenda Item:	19.3

Document for:	Discussion and Decision

1 Introduction

According to the LS from RAN1 [1], RAN1 agreed the following conclusions regarding the latency improvement for PRS measurement with MG:

Agreement Preconfiguration of MG(s) in RRC is supported from RAN1 perspective. - Each MG in the preconfiguration is associated with an ID - The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG - Send an LS to RAN2 and RAN3

Agreement: Support the following options (in the agreement made in RAN1#106-e) for a new mechanism of MG activation request for the purpose of positioning. - Option 2: by UE (via UCI or UL MAC CE) - Select only one of UCI and UL MAC CE in RAN1#106bis-e - Option 1: by LMF (via an NRPPa message) - Note: This is transparent to the UE

According to the LS from RAN1 [2], RAN1 agreed the following conclusions regarding the PRS processing window:

Agreement PRS processing window request to the gNB by the LMF is supported from RAN1 perspective. - It is up to RAN3 to design the necessary information to be transferred in the NRPPa message. - Note: It is up to gNB to determine the usage of measurement gap or PRS processing window - Include it in the LS to RAN2 and RAN3. Agreement For PRS processing window configuration and indication, at least the following mechanism is supported - RRC (pre-)configuration for PRS processing window configuration and DL MAC CE activation for PRS processing window, respectively. Include it in the LS to RAN2 and request RAN2 to decide whether DL MAC CE is feasible for this indication.

In this contribution, we provide analysis and proposals on impact on RAN3 specification of Latency Improvement. Accordingly two draft Reply LSs are provided in chapter 5.

2 Discussion

2.1 PRS measurement with MG

Pre-configuration of MG

According to RAN1 discussion, to reduce latency of NR positioning with MGs for DL PRS processing, the pre-configuration of MG is supported by majority of companies. Specifically, the corresponding signaling details need to be designed by RAN2/3, and two main issues need to be further considered: one is how to notify UE of the pre-configuration of MG and the other is how to obtain the pre-configuration of MG.

On the first issue, RAN1 has reached a consensus, as mentioned in LS [1]: "Pre-configuration of MG(s) in RRC is supported from RAN1 perspective and each MG in the Pre-configuration is associated With an ID. ". Therefore, the specific signaling details need to wait for RAN2 discussion.

In addition, in NG-RAN split architecture, each MG configuration and associated ID information need to be notified to gNB-DU via F1AP, so that the gNB-DU can properly schedule data transmission based on the MAC indication from the UE.

Proposal 1:  Signaling details of Pre-configuration of MG should wait for RAN2, and each MG configuration and associated ID information need to be notified to gNB-DU via F1AP.

However, on the second issue, RAN3 may discuss how gNB could determine the proper positioning MG pattern in RRC message, there may be two possible options:

- Option 1: Pre-configuration of MG can be directly provided by LMF.

- Option 2: LMF provides a full PRS configuration to gNB, and the gNB determines the pre-configuration of MG.

For Option 1, LMF only knows the PRS configuration information of an UE, but it cannot know the real-time transmission status of the UE and the UE’s radio capability information. Therefore, the MG pattern may be inaccurate or causing many redundant configurations. But for Option 2, LMF only needs to transmit positioning assistance information (e.g., a full PRS configuration) to both UE via LPP and gNB via NRPPa, so that gNB can determine the proper pre-configuration of MG based on real-time transmission status and radio capability information of the UE.

Compared with the two options, Option 2 has a slight advantage, but anyway, NRPPa needs to use a UE associated signaling procedure to transfer such assistance information, e.g., a new defined assistance information delivery procedure, or reusing positioning information request procedure.

Proposal 2: NRPPa introduces a new defined assistance information delivery procedure to transfer MG related assistance information.

MG activation request

On this issue, RAN1 discussed two ways, one is by UE and the other is by LMF.

The majority of companies preferred to use UE-initiated activation request procedure, that is, “The UL MAC CE for MG activation request by the UE can be one ID associated with the pre-configuration of the MG”.

According to such design, the signaling flow is shown as below:

Step 1: Pre-configuring MG

The gNB notifies UE of the pre-configured MG(s) and associated ID(s) via RRC messages;

Step 2: Activating MG

The UE selects appropriate positioning MG(s) based on service receiving status, RRM MG configuration, and positioning measurement requirement etc., and then notifies the gNB via UL MAC CE which contains one or more active positioning MG ID(s).

According to such signaling flow, it mainly needs the signaling detail of RAN2, without the impact of RAN3 specification.

Proposal 3:  Signaling details of MG activation request by UE should wait for RAN2, without the impact of RAN3 specification.

In RAN1, for MG activation request by the LMF, the views from proponents are quite diverse. Therefore, this approach is more suitable for RAN3 discussion.

The main disadvantage of the approach is that LMF can’t know other service receiving status of the UE and RRM MG configuration information. In addition, LMF does not know the PRS resources that UE actually chooses. Therefore, compared with the first method, it seems more blind.

On the other hand, there is already a way activating MG that works well, and it seems not necessary to introduce another way to do so. Therefore, activation of MG by LMF is not proposed.

Proposal 4:  Not support the activation of MG by LMF.

2.2 PRS processing window

PRS processing window configuration and indication

According to RAN1 discussion, to further reduce the delay of UE for downlink positioning measurement, RAN1 agreed to introduce the concept of PRS Processing window and reached the following conclusions at RAN1#106bis and RAN1#107 meeting: "for PRS processing window configuration and indication, at least the following mechanism is supported: RRC (pre-)configuration for PRS processing window configuration and DL MAC CE activation for PRS processing window respectively". Therefore, the specific signaling details need to wait for RAN2 discussion.

In addition, in NG-RAN split architecture, PRS processing window configuration needs to be notified to gNB-DU via F1AP, so that the gNB-DU can activate/deactivate the configuration via MAC CE and properly schedule data transmission based on the data transmission status.

Proposal 5: Signaling details of PRS processing window configuration should wait for RAN2, and the configuration needs to be notified to gNB-DU via F1AP.

According to RAN1 discussion, to support PRS measurement outside the MG, different UE capabilities need to be considered:

Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.)

When gNB configures and activates PRS processing window configuration, the above capability information helps gNB know which service data may be affected and schedule them accordingly. At present, this part of UE capability is most likely to be transmitted to LMF via LPP message. Therefore, NRPPa needs to support the corresponding signaling procedure to transfer this part of capability to the gNB.

In addition, in NG-RAN split architecture, this part of UE capability needs to be notified to gNB-DU via F1AP, like the UE-CapabilityRAT-ContainerList IE, so that the gNB-DU can properly schedule data transmission based on the data transmission status.

Proposal 6: To support the corresponding signaling procedure to transfer the UE’s positioning capability to the gNB via NRPPa, and the part of UE capability also need to be notified to gNB-DU via F1AP.

In addition, to allow gNB to be aware of symbol level scheduling availability of Capability 2 UE, a full PRS configuration, instead of the PRS span should be useful. The full configuration could also provide the PRS bandwidth and numerology information for the gNB to decide whether BWP switching is required.

In addition, in NG-RAN split architecture, the full PRS configuration needs to be notified to gNB-DU via F1AP, so that the gNB-DU can symbol level schedule data transmission based on the UE capability.

Proposal 7: To support the corresponding signaling procedure to transfer a full PRS configuration of the UE to the gNB via NRPPa, and the full PRS configuration also need to be notified to gNB-DU via F1AP.

PRS processing window request

According to RAN1 discussion: " PRS processing window request to the gNB by the LMF is supported from RAN1 perspective ". Then, how does the LMF decide to initiate this request? It is generally assumed that LMF can make correct decision according to the positioning capability of UE, all PRS configuration for the UE, positioning delay requirement, as well as how to configure positioning MG, etc.

Based on the previous discussion, if NRPPa can support the transmission of UE positioning capability and all PRS configuration information, gNB itself can well decide whether to configure the PRS processing window as long as LMF can provide enough assistance information. Similarly in case of discussing the MG activation request by LMF, we mentioned what gNB really needs is the assistance information from LMF, not a trigger.

Proposal 8: The LMF helps gNB to configure and indicate PRS processing window by providing assistance information, but not directly requesting.

Another important assistance information is the delay requirement of UE positioning, which may need to consider two options. One option is not to provide quantitative delay parameter, but the use of above assistance information implies a high delay requirement for the subsequent positioning procedure. The other is to standardize a quantified delay parameter, e.g., the response time IE so that gNB can decide the optimal positioning configuration.

Proposal 9: To provide quantitative delay parameter to gNB, e.g., the response time IE, as a kind of assistance information from LMF.

According to such design, the signaling flow is shown as below:

Step 1: PRS processing window request

By NRPPa message, LMF informs gNB of UE's positioning capability, a full PRS configuration and other assistance information;

Step 2: PRS processing window configure and activate

The gNB determines whether to configure the positioning MG according to UE positioning capability, the actual service transmission status, and the possible PRS measurement, etc. Meanwhile, it also decides whether to configure PRS processing window, and pre-configure PRS processing window to UE via RRC messages, then uses DL MAC CE to activate it.

According to such signaling flow, RAN3 can design the corresponding assistance signaling procedure, we propose to reuse the assistance information delivery procedure to transfer MG related assistance information.

Proposal 10: To reuse the assistance information delivery procedure for positioning MG to transfer the assistance information related to PRS processing window.

Finally, we prepared two reply LSs to response the LSs of RAN1 and a draft TP for TS 38.455 and TS 38.473.

3 Conclusion

In the previous sections we made the following proposals:

Proposal 1:  Signaling details of Pre-configuration of MG should wait for RAN2, and each MG configuration and associated ID information need to be notified to gNB-DU via F1AP.

Proposal 2: NRPPa introduces a new defined assistance information delivery procedure to transfer MG related assistance information.

Proposal 3: Signaling details of MG activation request by UE should wait for RAN2, without the impact of RAN3 specification.

Proposal 4: Not support the activation of MG by LMF.

Proposal 5: Signaling details of PRS processing window configuration should wait for RAN2, and the configuration needs to be notified to gNB-DU via F1AP.

Proposal 6: To support the corresponding signaling procedure to transfer the UE’s positioning capability to the gNB via NRPPa, and the part of UE capability also need to be notified to gNB-DU via F1AP.

Proposal 7: To support the corresponding signaling procedure to transfer a full PRS configuration of the UE to the gNB via NRPPa, and the full PRS configuration also need to be notified to gNB-DU via F1AP.

Proposal 8: The LMF helps gNB to configure and indicate PRS processing window by providing assistance information, but not directly requesting.

Proposal 9: To provide quantitative delay parameter to gNB, e.g., the response time IE, as a kind of assistance information from LMF.

Proposal 10: To reuse the assistance information delivery procedure for positioning MG to transfer the assistance information related to PRS processing window.

4 Reference

- R3-220090 ,  LS on latency improvement for PRS measurement with MG

- R3-220092,  LS on PRS processing window

5 Appendix

5.1 Reply LS (Draft)

5.1.1 Reply LS on latency improvement for PRS measurement with MG

Title:	[DRAFT] Reply LS on latency improvement for PRS measurement with MG

Reply to:	LS on latency improvement for PRS measurement with MG (R1-2112784)

Release:	Release 17

Work Item:	NR_pos_enh

Source:	CATT

To:	RAN1, RAN2

Cc:

Contact Person:

#### Name:	Jiancheng Sun

E-mail Address:	sunjiancheng@catt.cn

Attachments:	n/a

1. Overall Description:

RAN3 thanks RAN1 for the LS on latency improvement for PRS measurement with MG

RAN3 has discussed the signalling procedure of pre-configuration of MGs and MG activation request, and would like to inform RAN1/2 about our progress as follows:

- NRPPa introduces a new defined assistance information delivery procedure to transfer MG related assistance information. e.g, a full PRS configuration;

- RAN3 does not support the MG activation request by LMF.

Except, in NG-RAN split architecture, each MG configuration and associated ID information need to be transferred via F1AP, but the signalling details of Pre-configuration of MG should be discussed in RAN2, so RAN3 waits for further progress of RAN2.

2. Actions:

To RAN1 working group.

ACTION: RAN3 respectfully requests RAN1 to take the above information into consideration in their future work.

To RAN2 working group.

ACTION: RAN3 respectfully requests RAN2 to take the above information into consideration in their future work and further specified the signalling details of Pre-configuration of MG via RRC message.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#115	             21 Feb - 3 Mar 2022				Online

5.1.2 Reply LS on PRS processing window

Title:	[DRAFT] Reply LS on PRS processing window

Reply to:	LS on PRS processing window (R1-2112881)

Release:	Release 17

Work Item:	NR_pos_enh

Source:	CATT

To:	RAN1, RAN2

Cc:

Contact Person:

#### Name:	Jiancheng Sun

E-mail Address:	sunjiancheng@catt.cn

Attachments:	n/a

1. Overall Description:

RAN3 thanks RAN1 for the LS on PRS processing window.

RAN3 has discussed the signalling procedure of PRS processing window configuration and indication, and would like to inform RAN1/2 about our progress as follows:

- To use a new defined assistance information delivery procedure to transfer the assistance information related to PRS processing window including a full PRS configuration, UE positioning capability, etc;

- By providing assistance information, LMF helps gNB to configure and indicate PRS processing window;

Except, in NG-RAN split architecture, the PRS processing window configuration needs to be transferred via F1AP, but the signalling details of PRS processing window configuration should be discussed in RAN2, so RAN3 waits for further progress of RAN2.

2. Actions:

To RAN1 working group.

ACTION: RAN3 respectfully requests RAN1 to take the above information into consideration in their future work.

To RAN2 working group.

ACTION: RAN3 respectfully requests RAN2 to take the above information into consideration in their future work and further specified the signalling details of PRS processing window configuration via RRC message.

3. Date of Next TSG-RAN WG3 Meetings:

RAN3#115	             21 Feb - 3 Mar 2022				Online

5.2 TP for BL CR 38.455

8.4.x	Assistance Information Delivery

8.4.x.1	General

The purpose of the Assistance Information Delivery procedure is to allow the LMF to signal DL positioning assistance information for the UE to the NG-RAN Node. This procedure applies only if the NG-RAN node is a gNB.

8.4.x.2	Successful Operation

Figure 8.4.x.2-1: Assistance Information Delivery procedure

The LMF initiates the procedure by sending an ASSISTANCE INFORMATION DELIVERY message.

If the UE Positioning Capability IE is included in the ASSISTANCE INFORMATION DELIVERY message, the NG-RAN Node may take it into account when configuring PRS process window.

8.4.x.3	Abnormal Conditions

TBD.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.3.x	ASSISTANCE INFORMATION DELIVERY

This message is sent by the LMF to transfer assistance information for DL Positioning.

Direction: LMF  NG-RAN Node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| DL Positioning Assistance Information | M |  | 9.2.x |  | YES | reject |
| UE Positioning Capability | O |  | FFS |  |  |  |
| Response Time | O |  | FFS |  |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.x	DL Positioning Assistance Information

This IE contains the assistance information for DL PRS Configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL PRS Assistance Data List |  | 1..<maxNrOfFreqLayers> |  |  |
| >Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …) |  |
| >PRS bandwidth | M |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >Start PRB | M |  | INTEGER(0..2176) | Starting PRB to Point A |
| >Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >Comb Size | M |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >CP Type | M |  | ENUMERATED(normal, extended, …) |  |
| > TRP List |  | 1 .. <maxnoTRPsPerFreqLayer> |  |  |
| >>TRP ID |  |  | INTEGER (0..255) |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >>NR CGI | M |  | 9.2.9 |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  |
| >>PRS Configuration | M |  | 9.2.44 |  |

| Range bound | Explanation |
| --- | --- |
| maxNrOfFreqLayers | Maximum number of Frequency Layers. Value is FFS. |
| maxnoTRPsPerFreqLayer | Maximum number of TRPs per Frequency Layer. Value is 256. |

5.2 TP for BL CR 38.473

8.13.x	Positioning Assistance Information Delivery

8.13.x.1	General

The purpose of the Positioning Assistance Information Delivery procedure is to allow the gNB-CU to signal DL positioning assistance information for the UE to the gNB-DU. The procedure uses UE-associated signalling.

8.13.x.2	Successful Operation

Figure 8.13.x.2-1: Positioning Assistance Information Delivery procedure

The gNB-CU initiates the procedure by sending a POSITIONING ASSISTANCE INFORMATION DELIVERY message.

8.13.x.3	Abnormal Conditions

TBD

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.2.12.x	POSITIONING ASSISTANCE INFORMATION DELIVERY

This message is sent by the gNB-CU to transfer assistance information of DL positioning.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DL Positioning Assistance Information | M |  | 9.3.1.x |  | YES | reject |
| UE Positioning Capability | O |  | FFS |  |  |  |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.x	DL Positioning Assistance Information

This IE contains the assistance information for DL PRS Configuration.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL PRS Assistance Data List |  | 1..<maxNrOfFreqLayers> |  |  |
| >Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120, …) |  |
| >PRS bandwidth | M |  | INTEGER(1..63) | 24,28,…,272 PRBs |
| >Start PRB | M |  | INTEGER(0..2176) | Starting PRB to Point A |
| >Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >Comb Size | M |  | ENUMERATED(2, 4, 6, 12, …) |  |
| >CP Type | M |  | ENUMERATED(normal, extended, …) |  |
| > TRP List |  | 1 .. <maxnoTRPsPerFreqLayer> |  |  |
| >>TRP ID |  |  | INTEGER (0..255) |  |
| >>NR PCI | M |  | INTEGER (0..1007) | NR Physical Cell ID |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>NR ARFCN | M |  | INTEGER (0..3279165) |  |
| >>PRS Configuration | M |  | 9.3.1.177 |  |

| Range bound | Explanation |
| --- | --- |
| maxNrOfFreqLayers | Maximum number of Frequency Layers. Value is FFS. |
| maxnoTRPsPerFreqLayer | Maximum number of TRPs per Frequency Layer. Value is 256. |

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>