3GPP TSG-RAN WG3 #114bis-e	R3-220789

E-meeting, 17th – 26th January 2022

Source:	CATT

Title:	(TP for TR 37.817) Discussion on Standards Impact on Mobility

Agenda Item:	18.4.3

Document for:	Discussion and decision

- Introduction

Recently we tried to train some simple AI/ML models aiming to predict the UE’s location and found that the output is fairly accurate. In this document we briefly show our trial and propose that such prediction can be delivered toward the target RAN node during handover.

- Discussion

As a primary trial for AI/ML-based UE location prediction, we decided to train a model with historical geographical coordination as input and predicted geographical coordination as output. We downloaded the data set “GeoLife GPS Trajectories” provided by Microsoft Research Asia [1], and selected the longest roughly continuous section within it as the source data.

Figure 1: Source data: change of geographical coordinates against time.

The models we trained were based on LSTM. As a common practice, we used the former ⅘ part (the real value is slightly larger) for training and the latter ⅕ part for testing. For convenience we trained separate models to use the “historical” geographical coordinates prior to a given time to predict the geographical coordinates 1s, 2s, 5s, 10s, 20s, 50s, 100s, 200s, 500s, and 1000s respectively later to the given time.

The average training and testing error, conversed to metre for convenience, is shown in Figure 2:

Figure 2: Case after change on flow.

Although these models were quite primary (and not “intelligent” enough, frankly speaking), we can see that the accuracy was fairly well, although the device usually moved at a speed of ~30m/s (we guess that the device was in a car driving on expressway):

- When predicting the device’s position at ~1s later, the errors were generally at the level of metres.

- When predicting the device’s position at ~10s later, the errors were generally at the level of tens of metres.

We also tried similar method on a few other sections within “GeoLife GPS Trajectories” and the results were similar. Therefore we can conclude that the accuracy of UE location prediction can be like what is shown in Figure 2, if the UE provides its geographical location periodically.

Considering the ordinary RAN cell size (~500m for macro cells), we believe that UE location prediction can be precise enough, so that the (candidate) target gNB during handover preparation can use such prediction to optimise the RRC configuration. Therefore we prefer capturing this into the TR.

Proposal: To capture in TR 37.817 that UE location prediction can be used in (candidate) target gNBs.

- Conclusion

Proposal: To capture in TR 37.817 that UE location prediction can be used in (candidate) target gNBs.

- Reference

[1] https://www.microsoft.com/en-us/download/details.aspx?id=52367.

- TP for TR 37.817 (on the basis of v0.4.0)

//////////////////////////////////////////////////////////////////////skip unrelated text//////////////////////////////////////////////////////////////////////

          - 5.3.2.5			Output data

- UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time)

- Note: The UE trajectory prediction can be used either in the node hosting the Model Inference function or in the target node of UE context delivery, e.g. handover.

- Estimated arrival probability in CHO and relevant confidence interval

- Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication

//////////////////////////////////////////////////////////////////////end//////////////////////////////////////////////////////////////////////