3GPP TSG-RAN WG3 Meeting #114bis-e	R3-220824

17-26 Jan 2022

Agenda item:	13.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for BL CR for TS 38.401) Inter donor-DU re-routing

Document for:	Discussion and Decision

# Introduction

The source IP address filtering in IP routers introduces a problem in inter donor DU re-routing. As a result of inter donor DU rerouting the new donor DU (and possible also transport routers above the donor DU) receive IP packets with a source IP address that does not belong to its subnet and therefore the source IP filtering, if set on, will discard packets.

The RAN3#114-e meeting agreed the following for the IP filtering issue:

Agreements:

For inter-Donor-DU re-routing, the re-routed packet is only allowed to be transmitted between IAB-donor-DUs.

The static tunnel is selected to be established between IAB-donor-DUs for inter-donor-DU re-routing.

The re-routed packet between IAB-donor-DUs can be an UL IP packet without BAP header.

Target donor-DU determines the UL packet to be re-routed, by comparing IP prefixes and/or a list of IP address(es) configured by donor-CU, and the source address field of the UL IP packet.

It is up to donor-CU implementation about when to send to target donor-DU the information used for distinguishing the re-routed UL packets.

Some open issues were still identified, for example:

How target donor-DU knows which tunnel a re-routed UL packet should be delivered to.

This contribution discusses further about the remaining issue(s).

# Discussion

How the target donor-DU selects a tunnel for a rerouted packet

According to RAN3#114-e agreement:

”Target Donor DU determines the UL packet to be re-routed, by comparing IP prefix and/or a list of IP address(es) configured by donor-CU, and the source address field of the IP packet.”,

donor-DU inspects the source address of the IP header of the F1 packet to determine which packets are subject to inter donor-DU tunnelling. Simplest solution would be if the same source IP address of the F1 packet is used for selecting the tunnel pointing to the intended donor-DU. The donor CU knows the IP address(es)/prefixes used under each Donor DU and is therefore able to configure the source IP address to tunnel mapping.

In case of multiple, for example QoS based, tunnels between donor-DU pair are established, the mapping to tunnels can take DSCP field of the F1 packet’s IP header and/or ingress RLC channel into account, in addition to the source IP address. At a starting point, we can agree to use source IP address. The DSCP/ingress BH RLC channel can be discussed if there is a need.

Proposal 1: The donor DU selects the inter donor-DU tunnel based on the source IP address. FFS on DSCP and/or ingress RLC channel.

If the selection of packets for inter donor DU forwarding and destination donor DU and the exact tunnel selection base on the IP layer operations, it’s natural to tunnel IP packets instead of BAP packets.

Proposal 2. IP packets (BAP SDUs) are tunnelled between Donor DUs.

The configuration of the donor DU for inter donor DU tunnelling

The RAN3#114-e already agreed:

“It’s up to donor-CU implementation about when to send to target donor-DU the information used for distinguishing the re-routed UL packets.”

In addition to “The distinguishing re-routed UL packets”, donor DU must also map packets to the right tunnel, for which the mapping filters must be configured to the donor DU. As explained above, mapping filters can base on:

- the source IP addresses of the re-routed UL packets i.e. IP addresses of descendant IAB nodes of neighbour donor-DU

and in case of multiple tunnels between donor DUs

- DSCP of the IP header of the possible re-routed UL packet

- ingress RLC channel

The information from CU to Donor-DU needs to include:

- ID to identify the tunnel. The ID may be the tunnel ID, e.g. F-TEID

- Information to identify the applicable UL packets, e.g. a list of IP address/prefix that are used as the the source IP address of the UL packets.

In case of multiple tunnels, the information may be a list of {ID of tunnel, information to identify applicable UL packets}.

Proposal 3: The F1-AP procedure is used to configure the necessary information to donor DU for inter donor DU tunnelling. The information includes

- ID to identify the tunnel, e.g. F-TEID

- Information to identify the applicable UL packets, e.g. a list of IP address/prefix.

# Conclusion

In this contribution, we analyses the impact to support the inter-DU tunnel. Our proposals are:

Proposal 1: The donor DU selects the inter donor-DU tunnel based on the source IP address. FFS on DSCP and/or ingress RLC channel.

Proposal 2. IP packets (BAP SDUs) are tunnelled between Donor DUs.

Proposal 3: The F1-AP procedure is used to configure the necessary information to donor DU for inter donor DU tunnelling. The information includes

- ID to identify the tunnel, e.g. F-TEID

- Information to identify the applicable UL packets, e.g. a list of IP address/prefix.

Stage-2 TP can be found in the annex section.

# References

- R3-216197,Summary of Offline Discussion on CB: # 1306_IAB_Multi-hop

Proposed TP for BL CR for TS 38.401:

#### 8.2.3.1	Intra-CU topology adaptation procedure in SA

During the intra-CU topology adaptation in SA, both the source and the target parent node are served by the same IAB-donor-CU. The target parent node may use a different IAB-donor-DU than the source parent node. The source path may have common nodes with the target path. Figure 8.2.3.1-1 shows an example of the topology adaptation procedure, where the target parent node uses a different IAB-donor-DU than the one used by the source parent node.

**  Unchanged part is skipped  **

11.	The IAB-donor-CU configures BH RLC channels and BAP-sublayer routing entries on the target path between the target parent IAB-node and target IAB-donor-DU as well as DL mappings on the target IAB-donor-DU for the migrating IAB-node’s target path. These configurations may be performed at an earlier stage, e.g. immediately after step 3, or before step 3. The IAB-donor-CU may establish additional BH RLC channels to the migrating IAB-MT via RRC message.

12.	The F1-C connections are switched to use the migrating IAB-node’s new TNL address(es), IAB-donor-CU updates the UL BH information associated to each GTP-tunnel to migrating IAB-node. This step may also update UL FTEID and DL FTEID associated to each GTP-tunnel. All F1-U tunnels are switched to use the migrating IAB-node’s new TNL address(es). This step may use non-UE associated signaling in E1 and/or F1 interface to provide updated UP configuration for F1-U tunnels of multiple connected UEs or child IAB-MTs. The IAB-donor-CU may also update the UL BH information associated with non-UP traffic. Implementation must ensure the avoidance of potential race conditions, i.e. no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures.

In case IPsec tunnel mode is used for TNL protection, the IAB-node may use MOBIKE to migrate the IPsec tunnel to the new IP addresses. After the completion of the MOBIKE procedure, the existing SCTP association and the DL FTEID can be reused.

When the target IAB-donor-DU is configured with the information to support inter-DU routing, the target IAB-donor-DU may identify the applicable packets based on the source IP address field of the UL packet, and forward the received UL IP packet to the source IAB-donor-DU via a tunnel.

13.	The IAB-donor-CU sends a UE CONTEXT RELEASE COMMAND message to the source parent node IAB-DU.