3GPP TSG-RAN WG3 Meeting #114b-e	R3-220851

# E-meeting, 17-26 Jan 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0624 | rev | 2 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correcion of QoS monitoring disabled |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, ZTE, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At last RAN3-111-e meeting, the set of CRs R3-211276/R3-211224/R3-211225/R3-211226 were agreed, to stop the QoS monitoring according to TS 29.512. For XnAP and E1AP, due to the non-extendable QosMonitoringRequest in ASN.1, the new QoS Monitoring Disabled IE was introduced. However the procedure text for the optional QoS Monitoring Disabled IE is missing. The QoS Monitoring Disabled IE is included in the QoS Flow Level QoS Parameters IE, which may be included in many procedures e.g. over Xn - Handover Request - Retrieve UE Context Reponse - S-Node Additon Request - S-node Modification Request It should be clarified which of above messages can carry the QoS Monitoring Disabled IE. A typical example is that there is no need to carry this IE in the DRB setup case. Instead, only for DRB modify case, the QoS Monitoring Disabled IE could be included.. |
| Summary of change: |  | - Add procedure texts for QoS Monitoring Disabled IE in Bearer Context Modification (gNB-CU-CP initiated). Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only affects the QoS monitoring function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is amguious when the optional QoS Monitoring Disabled IE is signaled.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.423 CR0649 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-213755 Rev1: R3-215162 Resubmit to RAN3-114-e meeting Rev2: R3-220851 Update against the latest specification. Remove the change on QoS monitoring request. |  |  |  |  |  |  |  |  |

Change Begins

8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

<Unchanged Text Omitted>

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE is included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20]. If the QoS Monitoring Reporting Frequency IE is included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, use it for RAN part delay reporting. If the QoS Monitoring Disabled IE is included in the QoS Flow Level QoS Parameters IE in DRB To Modify List IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, stop the delay measurement and QoS monitoring for the concerned QoS flow.

<Unchanged Text Omitted>

Change Ends