3GPP TSG-RAN WG3 #114bis-e                                                                         R3-220876

Online, 16th – 27th January 2022

Title: 	(TP to TS 38.413) UE history information in MR-DC

Source: 	CMCC, CATT

Agenda item:	10.2.1.4

Document Type:	other

# 1. Introduction

This contribution is the TP for TS 38.413 BLCR with addition of UE history information for SN.

# 3. Annex – TP to TS 38.413 BLCR

Start of the first change

9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |  |  |
| Cell Type | M |  | 9.3.1.98 |  |  |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |  |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |  |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. |  |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.2.3.X | The PSCell related information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is FFS. |

#### 9.2.3.X	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited PSCell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN PSCell Information | M |  | 9.2.3.X1 |  |
| >E-UTRAN Cell |  |  |  |  |
| >>Last Visited E-UTRAN PSCell Information | M |  | OCTET STRING | Defined in TS 36.413 [16]. |
| >No PSCell |  |  |  |  |
| >>No Secondary Cell Information | M |  | 9.2.3.X2 | FFS |

9.2.3.X1	Last Visited NG-RAN PSCell Information

The Last Visited NG-RAN PSCell Information contains information about a cell that is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell Global Cell ID | O |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Time UE stayed in PSCell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the PSCell in seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in PSCell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the PSCell in 1/10 seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 40950. | YES | ignore |

9.2.1.X2	No PSCell Information

No PSCell Information contains the duration of time when SN is released.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Time UE stayed in PSCell | M |  | INTEGER (0..4095) | The duration of the time the UE stayed in the PSCell in seconds. If the UE stays in a cell more than 4095s, this IE is set to 4095. | - |  |
| Time UE stayed in PSCell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of the time the UE stayed in the PSCell in 1/10 seconds. If the UE stays in a PSCell more than 4095s, this IE is set to 40950. | YES | ignore |

End of the last change