3GPP TSG-RAN WG3 #114-bis-e	R3-220976

17-26 Jan 2022

Online

Title: 	(TP for SON BL CR for TS 38.413) Introduce UE History Information in MR-DC

Source: 	ZTE, Lenovo, Motorola Mobility, China Unicom

Agenda item:	10.2.1.4

Document for:	Approval

1	Introduction and discussion

The discussion part can be seen in R3-220972 UE History Information in MR-DC.

Proposal 1: Instead of Time spent without SCG, RAN3 shall introduce Time stamp IE for SN UHI.

Proposal 4: Reuse the UE History Information from UE IE to transfer the SN UHI from UE.

Proposal 5: Set the maximum number of last visited PSCell to 16.

This paper provides the corresponding TP for TS 38.413.

2	TP to TS 38.413

*** Start change ***

#### 9.3.1.97	Last Visited NG-RAN Cell Information

This IE contains information about a cell. In case of NR cell, this IE contains information about a set of NR cells with the same NR ARFCN for reference point A, and the Global Cell ID IE identifies one of the NR cells in the set. The information is to be used for RRM purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |  |  |
| Cell Type | M |  | 9.3.1.98 |  |  |  |
| Time UE Stayed in Cell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. |  |  |
| Time UE Stayed in Cell Enhanced Granularity | O |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. |  |  |
| HO Cause Value | O |  | Cause 9.3.1.2 | The cause for the handover. |  |  |
| Last Visited PSCell List |  | 0..<maxnoofPSCellsPerPrimaryCellinUEHistoryInfo> |  | List of cells configured as PSCells. Most recent PSCell related information is added to the top of the list. | YES | ignore |
| >Last Visited PSCell Information | M |  | 9.2.3.X | The PSCell related information. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPSCellsPerPrimaryCellinUEHistoryInfo | Maximum number of last visited PSCell information records that can be reported in the IE. Value is 16. |

*** Next change ***

#### 9.2.3.X	Last Visited Secondary PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Last Visited PSCell Information | M |  |  |  |
| >NG-RAN Cell |  |  |  |  |
| >>Last Visited NG-RAN PSCell Information | M |  | 9.2.3.Y |  |
| >E-UTRAN Cell |  |  |  |  |
| >>Last Visited E-UTRAN PSCell Information | M |  | OCTET STRING | Defined in TS 36.413 [16]. |

#### 9.2.3.Y	Last Visited NG-RAN PSCell Information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Global Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  | - |  |
| Time UE Stayed in PSCell | M |  | INTEGER (0..4095) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in seconds. If the duration is more than 4095s, this IE is set to 4095. | - |  |
| Time UE Stayed in PSCell Enhanced Granularity | M |  | INTEGER (0..40950) | The duration of time the UE stayed in the cell, or set of NR cells with the same NR ARFCN for reference point A, in 1/10 seconds. If the duration is more than 4095s, this IE is set to 40950. | - |  |
| Time Stamp | O |  | 9.3.1.75 | Indicates the UTC time when the PSCell was added. | YES | ignore |

*** Next change ***

- 9.3.1.166	UE History Information from UE

This IE contains information about mobility history report for a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE History Information from UE | M |  |  |  |
| >NR |  |  |  |  |
| >>NR Mobility History Report | M |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 38.331 [18]). VisitedPSCellInfoList also contained in the UEInformationResponse message (TS 38.331 [18]). |

*** Start of ASN.1 change ***

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

-------- skip unchanged part ----------

id-UserLocationInformationW-AGF,

id-LastVisitedPSCellList,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

-------- skip unchanged part ----------

maxnoofXnGTP-TLAs,

maxnoofXnTLAs

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo

-------- skip unchanged part ----------

#### -- L

LastVisitedNGRANCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

cellType									CellType,

timeUEStayedInCell							TimeUEStayedInCell,

timeUEStayedInCellEnhancedGranularity		TimeUEStayedInCellEnhancedGranularity 			OPTIONAL,

hOCauseValue								Cause		 									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

ID id-lastVisitedPSCellList	CRITICALITY ignore	EXTENSION	LastVisitedPSCellList		PRESENCE	OPTIONAL,

...

}

LastVisitedPSCellList ::= SEQUENCE (SIZE(1.. maxnoofPSCellsPerPrimaryCellinUEHistoryInfo)) OF LastVisitedPSCellInformation

LastVisitedPSCellInformation ::= CHOICE {

nGRANCell		LastVisitedNGRANPSCellInformation,

eUTRANCell		LastVisitedEUTRANPSCellInformation,

choice-Extensions		ProtocolIE-SingleContainer { {LastVisitedCellInformation-ExtIEs} }

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LastVisitedEUTRANPSCellInformation ::= OCTET STRING

LastVisitedNGRANPSCellInformation::= SEQUENCE {

globalCellID								NGRAN-CGI,

timeUEStayedInPSCell						TimeUEStayedInPSCell,

timeUEStayedInPSCellEnhancedGranularity		TimeUEStayedInPSCellEnhancedGranularity 		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {LastVisitedNGRANCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedNGRANPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-Time-Stamp							CRITICALITY ignore	EXTENSION	TimeStamp		PRESENCE	OPTIONAL},

...

}

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-------- skip unchanged part ----------

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofAllowedAreas					INTEGER ::= 16

maxnoofAllowedCAGsperPLMN			INTEGER ::= 256

-------- skip unchanged part ----------

maxnoofCandidateCells				INTEGER ::= 32

maxNRARFCN							INTEGER ::= 3279165

maxnoofPSCellsPerPrimaryCellinUEHistoryInfo		INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AllowedNSSAI											ProtocolIE-ID ::= 0

id-AMFName												ProtocolIE-ID ::= 1

-------- skip unchanged part ----------

id-PduSessionExpectedUEActivityBehaviour					ProtocolIE-ID ::= 281

id-MicoAllPLMN											ProtocolIE-ID ::= 282

id-QosFlowFailedToSetupList								ProtocolIE-ID ::= 283

id-lastVisitedPSCellList								ProtocolIE-ID ::= xxx

END

-- ASN1STOP

*** End of change ***